A biobjective stochastic simulation-based optimization with a risk constraint for urban transportation problems

A stochastic constrained biobjective simulation-based optimization framework for urban transportation problems

This study proposes a simulation-based optimization (SO) method to solve stochastic constrained biobjective urban transportation problems, whose biobjectives and risk (probability) constraint can only be evaluated by computationally expensive stochastic traffic simulators. It presents Gaussian process regression to estimate the means and variances at unsampled solutions with those at sampled solutions and applies knowledge gradient sampling policy to search for the final non-dominated solutions. Under very tight computational budgets, the proposed SO framework is computationally efficient and suitable for complex transportation problems with large-scale discrete or continuous solution spaces. During numerical experiments, we evaluate the performance of this SO method considering a network-wide traffic signal timing problem for the Changsha city of China, various uncertainty scenarios, and tight computational budgets. The SO method leads to well-performing nondominated signal plans, which can not only balance biobjectives of expected normalized traffic efficiency and equity, but also guarantee a probabilistic constraint for the normalized vehicular emissions, under some bounded perturbation of traffic volumes on the road network.

Keywords: simulation-based optimization; biobjectives; risk constraint; traffic signal timing.

Thus, a scientific and reasonable signal control strategy can not only reduce network-wide vehicular delays and emissions, but also give some priorities to vehicle flow in the arterial road.

1 Introduction

Traffic and transportation systems are dynamic, stochastic, non-linear and complex. It is extremely challenging to understand such complexity with the classic analytical and sometimes simplified mathematical models. A powerful alternative is simulation-enabled approach. Recently, a variety of traffic simulation tools have been developed both commercially and open source, and thus provide us a more flexible while comprehensive way to capture the intricate interactions between the supply and the demand components of the mobility network. In current practices, there are widely applied traffic simulators such as microscopic ones (e.g., VISSIM, PARAMICS, SUMO) and macroscopic ones (e.g., VISUM, DTALITE, DYNUST). Specifically, the former… The latter… These microscopic and macroscopic traffic simulators can provide estimates of traffic network performance in various levels of resolution. They are therefore often used to evaluate a set of predetermined strategies (e.g., traffic facility location or network design problems) in the context of what-if analysis or sensitivity analysis. While, once the number of candidate strategies is huge or innumerable, i.e., the decision space is discrete with numerous solutions or continuous, the aforementioned enumeration method for the simulation evaluations of a limited number of candidate strategies is not applicable. In this condition, a research framework of simulation-based optimization (SO) emerges, which mainly involves the construction of SO platform and the design of SO algorithm. The former can be realized by the inter-process communication and data exchange between the traffic simulator and other compatible software (e.g., Matlab, Python), which allows us to flexibly design or modify the interactions among various traffic elements. The latter has been studied for a few decades and is still being a hot topic in the area of operations research and computer science. The relevant reviews about the SO algorithms can refer to …

However, for traffic and transportation SO studies, the use of traffic simulation as an evaluation tool of traffic planning, design, management or control schemes usually has the following challenges. First of all, there is a need for novel calibration methods that exploit new sources of field data, are suitable for metropolitan-scale traffic simulation models and have appropriate compute-time requirements for offline or real-time application. Traffic simulation model calibration is essential to enable micro- and macroscopic traffic phenomena to be reproduced and reliable network performance estimates to be derived. Moreover, it would result in unaffordable computation costs if the calibrated traffic simulation model is computationally expensive and used together with optimization algorithms that rely on extensive exploration of the entire domain (e.g., Genetic Algorithm (GA), Tabu search) or even with the enumeration method to pick out the best solution from a limited number of predetermined strategies. After that, the “black-box” property of traffic simulation makes it difficult to derive the explicit expressions of the objective or constraint functions that fit traditional optimization methods that usually rely on gradient information, and deviation in the objective or constraint function values caused by stochastic noises in traffic simulation can lead to the failure of traditional optimization methods designed for noise-free optimization problems. That is, the properties, such as costly to compute, non-analytical form of the (simulation) function and randomness, of traffic simulation preclude the use of traditional optimization methods that rely on deterministic gradient information or on extensive and noise-free exploration of the feasible region. Besides, other more difficult challenges come along with the commonly existing traffic SO problems with high dimensions, multi-criteria (e.g., traffic efficiency, safety, environmental impact and equity) and various models for uncertainties (e.g., a probability distribution, a range or an ambiguity set of probability distributions about uncertain parameters). So, there is the urgent need for novel optimization algorithms (e.g., multiobjective stochastic or (distributionally) robust SO algorithms) that enable the systematic use of calibrated traffic simulators to tackle these challenging problems.

Suppose x is the decision vector from a known feasible set X, and ξ is the system parameter vector in the traffic simulator, which has a known distribution p(ξ) with bounded support Ξ. We focus on traffic and transportation problems of the following form:

minx∈X Eξ[f1x,ξ],Eξ[f2x,ξ]          s.t.   Prξ f3x,ξ-τ≤0≥1-E                                 (1)

where the evaluation functions f1x,ξ to f3x,ξ are three stochastic performance measures output from traffic simulator and depend on the decision vector x and the exogenous parameter vector ξ in the traffic simulator.  Specifically, ξ is composed of the fixed parameters (e.g., road network topology and signal phase setting) and stochastic parameters (e.g., uncertainties on driving behaviors, routing behaviors and traffic demands). Eξ and Prξ denotes the expectation and the probability with respective to the random perturbation in ξ, respectively. τ is the maximum acceptable threshold with respective to f3x,ξ. E is a small scalar predetermined by the decision maker (e.g., E=0.05). So, this typical problem is a stochastic constrained biobjective SO problem whose objective is to minimize the expected values of f1 and f2 subject to a probabilistic constraint for f3, under some bounded perturbations of stochastic components in ξ.

Usually, there are three ways to address the common SO problems, that is, direct-search methods, stochastic gradient methods and surrogate methods (Osorio and Bierlaire, 2013). While, the three evaluation functions f1x,ξ to f3x,ξ in the problem (1) are black-box (i.e., no available closed and analytical form) with stochasticity and computationally expensive if the traffic simulation is computationally intensive. The stochasticity inherent in traffic simulation outputs along with their high computational costs make the accurate estimation of search directions or gradients an expensive task. In this condition, surrogate based SO techniques are appropriate alternatives to tackle the problem (1). With very tight computational budgets, we first use the observed objective functions at a small number of design inputs to construct the surrogate model of traffic simulation, which serves as an approximation of the true response surface at the unexplored regions. It provides information about the entire space and helps decide new points by some criteria to locate the optimum efficiently. In other words, such SO techniques attempts to search the local optimal solution or the second-Pareto front optimal solutions in a relative small promising region, which is guided and updated by a small historical data regime and surrogate models built with it. For example, based on the built surrogate models with limited evaluated samples in the whole decision space, the expected improvement (EI) criterion (***), scoring criterion (***), trust-region search (***), Bayesian optimization (***), knowledge gradient policy (***) and Gaussian Markov random fields (***) are used to sample new solutions and then update current Pareto front.

Moreover, different from deterministic biobjective SO problems with explicit constraints, the model (1) considers the expectations of two stochastic simulation measures as biobjectives and a probabilistic constraint about another stochastic simulation measure, which results in some specific tough challenges that have to address as follows. That is, 1) the stochastic noises along with the simulation evaluation outputs have to be well smoothed or mitigated, which is very necessary due to that the better approximated expectations of biobjective stochastic measures for sampled solutions can help to construct more accurate surrogate models for biobjective functions. This benefits the sample infill strategies aforementioned to search more promising solutions. Although some previous studies have attempted to deal with this challenge, such as using average approximation method (AAM) (***), simulation allocation rule (SAR) (***), shrinking ball method (***) and various types of kriging models (***), there is still room for improvement. 2) We must consider the sensitivities of the searched solutions to the stochastic parameters in ξ, in addition to their dominance relationships considering a constraint under the nominal (or average) parameter values. A nondominated solution under the nominal (or average) parameter setting may become dominated when a certain stochastic parameter is perturbed. Thus, it is necessary to introduce the dominance relationship with the consideration of the expectations and the probabilistic constraint. 3) We consider a chance constraint in the problem (1) because in many cases the key quantity to be constrained (e.g., traffic emissions, total travel time) is random in nature depending on the realization of underlying input or parameters of the traffic system. The distribution of the quantity to be constrained is usually unknown a priori. Thus, a chance constrained problem can be challenging to solve even though the distribution of the uncertain parameters or input is given or can be well estimated.

To address aforementioned challenges, we propose Gaussian process regression with knowledge gradient sampling policy (GPR-KG) to solve the model (1), and this methodology is applicable to solve both continuous and discrete stochastic SO problems similar as the model (1).

The remainder of this paper is structured as follows. We present a literature review of the related SO methods in Section 2. A network-wide signal timing problem under uncertainties (NSTPU) similar as the model (1) is formulated and described in Section 3. We then propose Gaussian process regression based solution approach in Section 4. Later, we show the applicability of the proposed method in addressing a test problem and NSTPU in Section 5. Finally, some important findings are drawn in Section 6.

2 Literature review

This section mainly reviews (multiobjective) stochastic SO methods that can address computationally expensive simulation-based problems, whose objectives are formulated as expectations with respect to some stochastic parameters involved in the simulation. A brief review of the knowledge gradient (KG) policy, the sampling policy we adopt in our traffic SO framework, is also given.

2.1 Stochastic SO methods

For stochastic simulations, the expectations of simulated performance measures are often considered to be optimized. The stochastic noises in response need to be carefully considered by surrogate model and optimization algorithm. Based on random search, Hong and Nelson (2006) proposed a convergent optimization via most-promising-area stochastic search (COMPASS) algorithm to solve constrained or unconstrained discrete optimization via simulation (DOvS) problems. It employs SAR to determine the simulation repetitions for each candidate solution and AAM to estimate its performance measures. For responses with homoscedastic noises, Huang et al. (2006) proposed sequential kriging optimization (SKO) with the nugget effect Gaussian process (GP) (also termed kriging) as the surrogate model and an augmented EI as the infill strategy. For responses with heterogeneous noises, Picheny et al. (2013) put forward an extension of EI named expected quantile improvement to consider the known noise levels at both the already observed design points and the future candidates. To handle the heteroscedastic simulation noises, Ankenman et al. (2010) proposed stochastic kriging, and Deng et al. (2012) and Ng and Yin (2012) developed Bayesian kriging, which also account for the extrinsic uncertainty about the unknown response surface. The difference between them lies in the fact that the model parameter uncertainty is considered in the latter but not in the former. Recently, Chen et al. (2018) enhanced the stochastic kriging and Bayesian kriging by incorporating the parameter uncertainty into the model architecture, which results in the Bayesian stochastic kriging. This approach explicitly accounts for the parameter uncertainty in the regression and prediction, and relaxes specific assumptions on the prior probability distributions of model parameters that exist in Bayesian kriging. Additionally, Chen et al. (2017) extended the cokriging model, which was originally developed by Kennedy and O’Hagan (2000) to model deterministic multifidelity problems, to stochastic simulation metamodeling for expectations, i.e., stochastic cokriging model. Quan et al. (2013) and Pedrielli et al. (2020) proposed the two-stage sequential optimization (TSSO) and the extended TSSO (eTSSO), respectively, in which the stochastic kriging is applied as the surrogate model, the EI criterion as the infill strategy and the optimal computing budget allocation (OCBA) (Chen et al., 2000) technique to decide the number of replications at design points. Besides, Salemi et al. (2019) used a Gaussian Markov random field (GMRF) to learn and exploit spatial relationships among the decision variables and objective function values, which is particularly well suited to the discrete stochastic SO problem with a large number of feasible solutions and a high computational cost for each simulation. Later, Semelhago et al. (2020) proposed a rapid Gaussian Markov Improvement Algorithm (rGMIA), in which the underlying Gaussian process is a discrete GMRF, and this algorithm delivers both a global convergence guarantee and finite-sample optimality-gap inference for significantly larger problems. Fan and Hu (2018) presented an adaptive search algorithm called surrogate-based promising area search (SPAS) for solving stochastic SO problems with Lipschitz continuous objective functions. The SPAS algorithm employs a shrinking ball method to estimate the performance of sampled solutions, a surrogate model fitted iteratively with the performance estimates to approximate the response surface of the objective function, and a promising area adaptively constructed for searching improved solutions.

As for multiobjective stochastic SO methods, Forrester and Keane (2009) introduced regressing kriging to deal with homoscedastic noisy data and multiobjective EI as the infill strategy, which can help to tackle multiobjective stochastic SO problems. Li et al. (2015) developed multiobjective COMPASS (MO-COMPASS), as an extension of the single-objective COMPASS, to solve DOvS problems with two or more objectives by considering the Pareto optimality and the probability of correct selection. Notably, MO-COMPASS also applies SAR to determine the simulation budget for each candidate solution and AAM to estimate its performance measures. Cooper et al. (2020) proposed the retrospective partitioned epsilon-constraint with relaxed local enumeration (R-PERLE) algorithm to solve the biobjective stochastic SO problem on integer lattices, and numerical experiments showed its favorable performance relative to R-MinRLE and MO-COMPASS. Based on a biobjective derivative-free trust region (BOTR) algorithm proposed by Ryu and Kim (2014), Zheng et al. (2019) put forward a biobjective stochastic simulation-based optimization (BOSSO) method, which introduces regressing kriging models to smooth the simulation noises, an adaptive selector to determine samples in the local trust-region to be evaluated by simulation, and a decision maker’s non-interactive role to impact the search direction and trust-region. The proposed BOSSO method has been successfully applied in the network-wide traffic signal timing problem and validated to outperform other counterpart algorithms.

Previous single- or multi-objective stochastic SO algorithms involving surrogate models have mostly dealt with bound constrained (also called box-constrained) SO problems with only expensive objective functions, and so the surrogate models are used to approximate the objective function only. As for addressing (single- or multi-objective) SO problems with expensive constraints, Regis (2011) presented a new derivative-free algorithm called ConstrLMSRBF to optimize expensive black-box objective functions subject to expensive black-box inequality constraints. This algorithm uses radial basis function (RBF) surrogate models and extends the Local Metric Stochastic RBF (LMSRBF) algorithm by Regis and Shoemaker (2007) that can handle black-box inequality constraints. Müller et al. (2013) introduced a surrogate model based algorithm named SO-MI to address expensive mixed-integer black-box global optimization problems that may have expensive constraints, in which the RBF surrogate model is used to select candidates at which the expensive objective and constraint functions are to be evaluated. Later, Müller et al. (2014) presented a variant of SO-MI, which is called surrogate optimization-integer (SO-I) algorithm, to solve purely integer optimization problems that have expensive black-box objective functions and constraints. Müller and Woodbury (2017) put forward a global optimization with surrogate approximation of constraints (GOSAC) algorithm with two phases to address problems with expensive black-box constraints, cheap objective functions and continuous, integer or mixed-integer variables. For a given budget of expensive constraint evaluations allowed, GOSAC finds better feasible solutions more efficiently than other algorithms. Feliot et al. (2016) proposed Bayesian Multi-Objective Optimization (BMOO), which handles objectives and constraints in a unified way by an extended domination rule and calculate and optimize an expected hyper-volume improvement sampling criterion using Sequential Monte Carlo techniques.

As is known, the mean of the response is a widely used performance measure in a stochastic system, while it is not able to evaluate the possible variability or describe the entire distribution adequately. In this condition, the quantile has become increasingly popular and of great interest in many fields to provide more thorough profiles of the response distribution (Longerstaey and Martin, 1996; Wipplinger, 2007; Cope et al., 2009). In risk management, quantile, also termed value at risk, is one of the primary risk measures to quantify and interpret the risk that one stochastic system may face. So, the surrogate models for quantiles have been developed extensively, e.g., quantile regression (QR) model, stochastic kriging model, and stochastic cokriging model (Koenker and Hallock, 2001; Chen, 2009; Dabo-Niang and Thiam, 2010; Chen and Kim 2016; Xie et al. 2018; Wang et al., 2021), based on which some optimization algorithms can be integrated for quantile optimization. Furthermore, the black-box constraints in aforementioned constrained SO algorithms are usually evaluated by expensive deterministic simulations, and do not consider the stochastic performance about the constraints. However, as mentioned previously, the stochasticity inherent in the simulation evaluation tool that commonly exists in various engineering fields would result in stochastic evaluation values of the constraint functions. Therefore, it is necessary and more challenging to account for the stochastic performance about the constraints. In this study, as formulated by the model (1), besides of using the expectations on two stochastic performance measures as biobjectives, we set a probabilistic (risk) constraint about another performance measure evaluated by stochastic simulation, which can be treated as the inverse solution of the corresponding quantile function. That is, the probabilistic (risk) constraint is a risk measure way to constrain the risk that the stochastic traffic simulation system may face in terms of this performance measure.

2.2 Bayesian optimization and the knowledge gradient policy

One notable stream of the SO literature takes the Bayesian perspective which assumes a prior probability distribution (or belief) on the performance (or reward) of each solution (or alternative) and keeps updating the posterior distribution based on new observations. Both the sampling decision and the decision for implementation are based on the posterior distributions, this is commonly known as the Bayesian optimization (BO). Many studies adopt the KG policy for the sampling decisions in BO. The KG policy chooses the next solution (or alternative) to be sampled which has the largest KG-factor, i.e., the expected improvement in the best posterior mean conditional on all the previous samples. Such a methodology was shown to be effective and empirically efficient for many SO applications compared to many other approaches. The KG policy was firstly introduced in Gupta and Miescke (1996) for the independent belief case, where the unknown mean of the performance of each solution has an independent prior distribution. Frazier (2009a) proved serval key properties of the KG policy such as asymptotic optimality and derived a more general KG policy for correlated beliefs, where the prior mean vector follows a multi-variate Gaussian distributions (Frazier 2009b). Parallel-KG policy that samples alternatives in batches was also developed for the independent prior case (Wu and Frazier 2016), which was shown to outperform other batch BO algorithms.

KG policies that incorporate different belief models were further developed, which either learn the parameters of a structured functional form assumed for the underlying mean (e.g., Negoescu et al. 2011), or learn the probabilities that each of the functionals (with known parameters) is the true one (Chen et al. 2015). For problems with large number of alternatives, KG policies with parameterized beliefs enable affordable computation by maintaining beliefs only on those hyper-parameters or features of the decision vector rather than on each alternative. For example, linear-belief based KG policies were effectively applied to the drug discovery problem (Negoescu 2011) and the traffic network tolling problem (Tan and Gao, 2016). Approximation schemes of the KG policy have also been proposed to accelerate the algorithm by computing the KG-factor only for a subset of alternatives at each iteration. One intuitive and effective approach in choosing the subset is by Monte-Carlo (MC) sampling based on the posterior distributions of the mean performance (Ryzhov and Powell 2009) or of the hyper-parameters in the belief model (Tan and Gao, 2016). As most KG policies are designed for discrete alternatives, they have also been extended to continuous alternatives by designing Guassian process representations and maintaining posterior beliefs at sampled solutions and those promising ones based on local derivate estimates (Scott et al. 2011).

However, those methods found in most KG literature are not readily applicable to the type of problems this study focuses on. Firstly, probabilistic constraints for simulation-based quantities have not been addressed in previous studies on KG policy, although some studies considered explicit simple deterministic (e.g., linear) constraints that defines the feasible set of the solutions space (Wang, 2021). Secondly, most of these studies only focused on single performance measure. Although some literature considered multi-objective KG problem, each performance measure was assumed independent across different alternatives (Yahyaa 2014). Thirdly, previous studies on the KG policy either assumed a known variance of the performance measure (e.g., Gupta and Miescke 1996, Frazier 2009a, Frazier2009b), or proposed local updating equation for the variance estimate for each sampled alternative (e.g., Tan and Gao 2016, Wang et al., 2021), or designed approximate formulae for updating the posterior variance vector of all the alternatives under the multi-variate Gaussian framework (e.g., Qu 2015, Zhang 2016). While the local updating approach neglects the possible similarities in variances of two close solutions, the approximate global posterior updating formula cannot be easily adapted to the setting with parameterized beliefs (thus is not applicable to problems with large solution space). Note that this unknown variance would make it more difficult to deal with the chance constraint as mentioned in the first point.

In this study, we will design a KG-based SO approach with parameterized belief and unknown variances that is applicable to problems with more than one performance measures as well as a chance constraint on another stochastic performance measure. As explained earlier, such a problem setting is commonly seen in urban transportation optimization problems and makes those problems challenging to solve, thus a reliable and efficient SO-based solution approach is highly needed. One contribution that may be of separate interest is that we proposed an updating scheme of variance estimates which not only accounts for possible similarities between the different solutions and can be easily implemented for the cases with parameterized beliefs as well. Such an updating scheme is shown effective especially when a chance constraint is considered, in which case one wants to estimate the distribution of the constrained quantity as accurate as possible under limited simulation budget.

3 Network-wide Signal Timing Problem under Uncertainties (NSTPU)

Urban signalized intersections are not only the bottleneck of traffic efficiency and dense area of vehicular emissions, but also influence the equity of right-of-way in the urban road network. In this condition, traffic efficiency is set as an evaluation indicator rather than the only indicator, when designing the traffic control system. Specifically, the traffic equity has been considered in various traffic control problems, such as temporal and spatial inequities of ramp metering control (Levinson and Zhang, 2006), aggregated and non-aggregated equity indices of corridor control (Ma and Zhang, 2008), spatial equity index of dynamic ramp control (Meng and Khoo, 2010), travel time-based Atkinson index of network-wide traffic signal timing plan (Zheng et al., 2019a). Meanwhile, driving modes of vehicles are substantially influenced by traffic signal control plan and various driving modes would result in various vehicular emissions. In details, vehicular emissions tend to decrease (in descending order) for acceleration, cruise, deceleration and idle (Unal et al., 2003), and increase considerably if vehicle stops are introduced (Rakha and Ding, 2003). Thus, apart from traffic efficiency, the fuel consumption and vehicular emissions on roads with signalized intersections are greatly influenced by the traffic signal control plan, due to its significant impact on various driving modes and their durations. In this condition, traffic signal control optimization provides an alternative way to mitigate urban vehicular emissions. For example, network-wide traffic signal control SO is applied to reduce total delay, fuel consumption and CO2 emissions (Stevanovic et al., 2009). A weighted index of average travel time and high-resolution vehicular emissions is reduced by the regional traffic signal timing SO, which applies a combined surrogate model of functional component and physical component (Osorio and Nanduri, 2015a, 2015b). Total delay and mean excess exposure are simultaneously minimized by using a simulation-based biobjective GA to solve a signal timing optimization model (Zhang et al., 2013). A biobjective stochastic simulation-based optimization (BOSSO) method is used to address a network-wide signal timing optimization problem with balance consideration of a linear combination of four pollutants’ emissions and total delay (Zheng et al., 2019b).

Due to the conflicting nature between traffic efficiency and equity (Kotsialos and Papageorgiou, 2004; Zhang and Levinson, 2004; Kesten et al., 2013; Zheng et al., 2019a), we take traffic efficiency and equity as biobjectives in this study. Meanwhile, the issue of urban vehicular emissions attract more and more attentions worldwide and particularly the international policies about “peak carbon dioxide emissions” and “carbon neutral” push us to adopt scientific and stringent regulations to reduce the urban vehicular emissions. So, vehicular emissions is set as another evaluation measure for the probabilistic constraint, that is, vehicular emissions has to satisfy a certain predefined improvement degree under uncertainties. Notably, the uncertainties we consider here are the perturbations on the calibrated traffic volume inputs on the network, and they are modeled by a random relative perturbation ξ>0, which represents the ratio of the traffic volume to its nominal value. ξ follows a distribution with bounded support Ξ, e.g., Ξ = [0.8, 1.2], representing a maximum of ±20% perturbation of the nominal traffic volume. Then, based on the three evaluation indices formulated as Eqs. (a1, a3, a6) in Appendix A, we use Dx,ξ, Ax,ξ and Ex,ξ to represent stochastic measures of traffic efficiency, equity and vehicular emissions, respectively, which are all output from the microscopic traffic simulator with inputs of a specific network-wide signal timing plan x and stochastic parameters (i.e., traffic demands here) in ξ. In this condition, the network-wide signal timing problem under uncertainties (NSTPU) is modelled as

minx∈X Eξ[Dx,ξ],Eξ[Ax,ξ]                         s.t.    Prξ⁡[ Ex,ξ-τE≤0]≥1-E                              (2)

where the signal timing plan x∈xl,xu is an integer valued column vector, xl∈Zn and xu∈Zn are its lower and upper bounds, respectively, and n is the dimension of x that depends on the scale of the urban road network. X= x:x∈xl,xu, x∈Zn  is the solution space. Eξ[Dx,ξ] and Eξ[Ax,ξ] denote the expected values of stochastic measures of traffic efficiency and equity, respectively, with respective to the random perturbations on traffic demands in ξ. τE is the maximum acceptable vehicular emissions intensity relative to the nominal value. E is a small scalar predetermined by the decision maker (e.g., E=0.05). Therefore, the model (2) is a stochastic constrained signal timing biobjective SO problem, which aims to minimize the expected values of the normalized traffic efficiency and equity indices subject to a probabilistic constraint for the normalized vehicular emissions, under some bounded perturbations on traffic volume inputs for the road network. Notably, x is the set of green signal timings of all signal phases in the urban road network, and thus the model (2) is with the high-dimensional and discrete solution and a huge discrete solution space, if the network is large-scale. Here, we suppose K is the total number of such signal time plans in X.

The difficulties in solving problem (2) lie in the fact that functions Dx,ξ, Ax,ξ, and Ex,ξ are unknown and in general so complicated that can only be estimated through network traffic simulations. Thus the traditional solution approaches based on mathematical programming techniques are not capable of solving problem (2). In the next section we propose a simulation-based optimization approach for solving problem (2) using Gaussian process regression with knowledge gradient sampling policy.

4 Gaussian process regression based solution approach for NSTPU

4.1 Gaussian process approximation for NSTPU

Under random traffic volume perturbation, the evaluation indices are also random, whose exact distributions are in general unknown a priori due to the complicated underlying traffic network system and limited historical data. We denote the random variable Dx as the normalized total delay output for decision vector x under uncertain traffic volume perturbation, i.e., Dx is variable Dx,ξ with ξ follows a probability density function pξ. In the same way, we define Ax and Ex for normalized system output of traffic equity and vehicular emissions.

For each x∈X, we assume that Dx, Ax and Ex follow Guassian distributions:

Dx ~ NμxD,λxD,                                                                      (8a)

Ax ~ NμxA,λxA,                                (8b)

Ex ~ NμxE,λxE                                (8c)

where μxi and λxi, i∈{D, A, E} are the unknown mean and variance of the corresponding evaluation index i for solution x. The key purpose of the simulation is to learn these quantities with the goal of solving optimization problem (7).

Suppose we are allowed to conduct a total of N simulations, each simulation chooses a solution x∈X for evaluation, the evaluation is noisy with an independent random realization of the uncertain parameter ξ drawn from pξx∈X. We denote the chosen solution for the n-th simulation by xn and the corresponding evaluation indices computed based on the traffic flow output from the simulation by yn=(yD,n,yA,n,yE,n). In other words, each simulation gives us a noisy sample of each of the three evaluation indices corresponding to the chosen solution, based on which we update our estimates for the underlying mean and variances of each evaluation index. In sequel we will use the notation pn, Pn, En, and Varn, n=1, 2,…,N, respectively, to represent posterior probability distribution function, probabilities, expectations, and covariances conditional on all the sampled solutions Xn={x1,…, xn} and observations Yn={y1,…, yn} up to stage n.

For convenience, we define the mean vector μi=({μxi, ∀ x∈X)} for each evaluation index i∈{D, A, E}. We take a Bayesian viewpoint for the estimation process of the mean vector μi for each i∈{D, A, E}. We start with some prior distribution p0(μi), then we update the posterior distribution pn(μi) based on Xn and Yn for n=1, 2,…,N one after another. For tractability in computation, for each i∈{D, A, E} we choose p0(μi) to be a K-dimensional normal distribution with mean θi,0 and covariance matrix Σi,0. Then based on the property of normal distribution, we know that pn(μi) is also a multivariate normal distribution for n=1, 2,…,N. Specifically, given θi,n-1,Σi,n-1 and the chosen sample xn as well as observation yn at stage n (n=1, 2,…,N), the posterior distribution pn(μi) is also a K-dimensional normal distribution with mean θi,n and covariance Σi,n as

θi,n=θi,n-1+yi,n-θxni,n-1Σxnxni,n+λxni,nΣi,n-1exn,                        (9a)

Σi,n=Σi,n-1+Σi,n-1exnexnTΣi,n-1Σxnxni,n+λxni,n,                          (9b)

where exn is a unit K-dimensional column vector with the entry corresponds to solution xn being one; θxni,n-1is the entry of vector θi,n-1 that corresponds to solution xn; Σxnxni,n denotes the diagonal entry of matrix Σi,n-1 that corresponds to solution xn; λxni,n is our best estimate of the sample variance of evaluation index i of solution xn at stage n, as the true underlying  λxni is assumed unknown a priori in our problem. In section 3.2, we will propose a parametric sequential estimation procedure for these sample variances.

Since the true underlying distributions of the evaluation indices Dx, Ax and Ex are not known a priori, problem (7) is itself unknown exactly. Thus we replace the true expectations E[Dx] and E[Ax] in (7) by their stage-N posterior means ENμxD=θxD,N and EN[μxA]=θxA,N, respectively, when the simulation budget is exhausted. We also replace the true probability P(Ex≤τE) by the posterior probability PN(Ex≤τE). Note that under the Guanssian assumption, we have

PNEx≤τE=PEx-θxE,NλxE,N1/2≤τE-θxE,NλxE,N1/2=ΦτE-θxE,NλxE,N1/2,

thus PNEx≤τE≥1-E is satisfied by restricting

θxE,N≤ τE-Φ-11-EλxE,N1/2                      (10)

where Φ is the cumulative density function of the standard normal distribution, so Φ-1 is the corresponding quantile function. Note that similar as in (9), we use the stage-N estimate of the sample variance, λxE,N, in inequality (10).

Therefore, we can rewrite problem (7) based on Guassian process approximation as

minx∈X θxD,NθxD,N,θxA,NθxA,N                                                      s.t.  θxE,NθxE,N≤ τE-Φ-11-EλxE,N1/2                       (11)

4.2 Parametric belief on model parameters

4.2.1 Parametric belief on the mean vector

It is generally not convenient to work with a high dimensional Guanssian distribution, when such distribution information are used in the learning algorithm for sampling decisions (e.g., the sampling policy that we will discuss in Section 3.3). For NSTPU, we note that the number of signal timing plan (i.e., K) can be very large. For example, consider a traffic network with only three intersections, each has only two phases whose green time ranges from 21 to 40 seconds, then we have K=(202)3=206, which is 64 million, a number that is already prohibitive for normal computation.

To this end, we seek proper parametric belief model to reduce the number of covariates in the learning problem. Specifically, we assume that the underlying mean of any solution x is a linear combination of a set of basis functions (or feature functions) of x plus a deviation term, i.e., we have

μxi=ζxi+j=1Miβjifjix,  i∈{=D, A, E},                       (12a)

where fji is the j-th deterministic basis function for evaluation index i, and Mi is the number of basis functions used for evaluation index i. The scalar βji represents the marginal contribution of the basis function value fjix to the means of the evaluation indices for the same solution, μμxi. ζxi is the deviation term. Thus (12a) assumes that the underlying mean of each evaluation index for a certain signal plan can be written as a linear combination of a series of basis functions with respect to the very signal plan plus a specific deviation term. Let column vectors βi=(β1i,…,βMii)T and ζi=(ζxi,x∈X), and let Fi be a K by Mi matrix with fjix being the j-th entry of the row corresponds to solution x. We also denote fxi as the row of matrix Fi that corresponds to solution x. Then we can express (12a) in matrix form as

μi=Fiβi+ζi,  i=D, A, E.                        (12b)

Now for i=D, A, E, we use a Mi-dimensional normal distribution for the prior of the coefficient vector βi, p0βi, with mean bi,0 and covariance matrix Bi,0, and an independent K-dimensional normal distribution for the prior of vector ζi, p0ζi, with zero mean and covariance matrix zi,0I, where I is a K-by-K identity matrix. Then the mean and the covariance matrix of the prior distribution of μi can be written as

θi,0=Fibi,0,                                     (13a)

Σi,0=FiBi,0(Fi)T+zi,0I,                          (13b)

Now we define for each i=D, A, E a stage-n coefficient vector βi,n, which contains vector βi as well as the deviation terms ζxi for all the distinct solutions x sampled up to stage n. Following this definition, we have clearly βi,0=βi as no sample has been taken at stage 0, so the prior of βi,0 is also Nbi,0,Bi,0. Then due to the property of multivariate normal distribution, it can be verified that for n=1, 2,…,N, the posterior distribution (conditional on Xn, Yn and λxni,n-1), pnβi,n, is also a multivariate normal distribution, whose mean bi,n and covariance Bi,n can be determined by (i=D, A, E)

bi,n=bi,n-1+yi,n-fxni,nbi,n-1fxni,nBi,n-1(fxni,n)T+λxni,n-1Bi,n-1(fxni,n)T,                (14a)

Bi,n=Bi,n-1+Bi,n-1(fxni,n)Tfxni,nBi,n-1fxni,nBi,n-1(fxni,n)T+λxni,n-1,                           (14b)

where the column vector bi,n-1 and the matrix Bi,n-1 are defined in the following way: if xn has been sampled before stage n (i.e., the deviation term ζxni has already been included in vector bi,n-1),   then bi,n-1=bi,n-1 and Bi,n-1=Bi,n-1; otherwise, bi,n-1 is formed by appending a zero to bi,n-1, and Bi,n-1 is formed by appending a row and a column to Bi,n-1, and among these new entries added, the new diagonal entry is zi,0 and all the others are zero. Namely, the following revisions are made if xn is first sampled at stage n:

bi,n-1=bi,n-10,     Bi,n-1=   Bi,n-10T0zi,0.

fxni,n is a row vector that has a commensurate dimension with βi,n and consists of fjixn at the j-th entry for j=1,…,Mi, and one at the entry whose index is the same with that of the entry in βi,n corresponding to the deviation term ζxni, and zeros elsewhere. For instance, suppose the last entry of βi,n corresponds to ζxni, then we have

fxni,n = f1ixn⋯fMiixn     0T1.

The updating equation (14) is similar with (9), but now instead of maintaining beliefs on the original mean vector μi, we only have to update the posterior distribution for parameter βi,n, which is typically much shorter than μi (although the size of βi,n generally grows with sampling stage n as new solutions are being sampled).

4.2.2 Parametric update for the variance estimate

Note that the estimate of the variance at stage n-1, λxni,n-1, is needed in equation (14) for generating the posterior mean and variance for pnβi,n. However, it is very likely that the solution to be sampled at stage n, xn, has not yet been sampled or has only been sampled a few times (e.g., less than three) before stage n, leading to a poor estimate λxni,n-1 and thus pnβi,n. In problems such as NSTPU, the sampling budget N is typically much smaller than the total number of solutions K, which makes it challenging for estimating the variance of unsampled solutions. Therefore, a parametric model which captures possible similarities in the variability of two close solutions can also be designed. In our application, we adopt a straightforward parametric model for estimating the variances based on set discretization. Specifically, we denote the domain of fjix on x∈X by Fji, for each j=1,…,Mi, and partition Fji into a set of mji disjoint subsets, fj,1i,…,fj,mjii, with l=1mjifj,li=Fji. Then there are a total of j=1Mimji different combinations of such sets over all j=1,…,Mi, and for a given x∈xl,xu we let q(x) be the combination index of one of these combinations such that fjix is in the j-th set of this combination for all j=1,…,Mi. Starting with some initial estimate of the variance of each evaluation index i=D, A, E, Λwi,0, for each combination index w=1,…,j=1Mimji, we consider the following three cases in updating the variance estimates iteratively for stage n=1, 2,…,N.

Case 1: If x is sampled at stage n, i.e., x=xn, then its variance is estimated by the weighted average of the initial estimate and the squared deviation of each sample from the corresponding posterior means:

λxi,n=s0Λqxi,0+l=1sxnyi,nxl-θxi,nxl2s0+ sxn=λxi,n-1s0+ sxn-1+yi,n-θxi,n2s0+ sxn,         (15a)

where s0 is the weight for the initial variance estimate, sxn is the number of samples collected for solution x up to stage n, and nxl is the stage at which the l-th sample of x is collected. The second equality is due to the definition of λxi,n-1 by the first equality and l=1sxnyi,nxl-θxi,nxl2=l=1sxn-1yi,nxl-θxi,nxl2+yi,n-θxi,n2, this iterative expression is computationally more efficient than the original definition in the first equality.

Case 2: If x is unsampled up to stage n but has the same combination index as the solution that is sampled at stage n, i.e., x≠xn, sxn=0 and qx=qxn, then its variance estimate is set to be equal to the average of the variance estimate of those sampled solutions that have the same combination index as x, including xn:

λxi,n=Λqxi,n=x:qx=qx,sxn>0λxi,nrqxn=Λqxi,n-1+λxni,n-λxni,n-1rqxn=λxi,n-1+λxni,n-λxni,n-1rqxn,      (15b)

where rqxn is the number of solutions that have been sampled by stage n and have combination index qx, and we denote Λwi,n as the stage-n variance estimate of the evaluation index i for all the unsampled solutions with the same combination index w (w=1,…,j=1Mimji). The first and the fourth equalities hold by our definition of Λqxi,n, and the third equality is true by considering the two possible cases: 1) If sxnn>1 (i.e., xn has been sampled before stage n), then rqxn=rqxn-1,noting that so we have Λqxi,n-1-Λqxi,n-1=(λxni,n-λxni,n-1)/rqxn ; and if sxnn=1 (i.e., xn is firstly sampled at stage n), we have rqxn= rqxn-1+1 and thus

λxi,n=Λqxi,n-1rqxn-1+λxni,nrqxn=Λqxi,n-1rqxn-1+λxni,n-1+λxni,n-λxni,n-1rqxn

=Λqxi,n-1rqxn-1+1+λxni,n-λxni,n-1rqxn-1+1=Λqxi,n-1+λxni,n-λxni,n-1rqxn,

where the second last equality is due to λxi,n-1=Λqxi,n-1 by construction.

Case 3: If x is sampled before stage n but not at stage n or x is unsampled up to stage n and does not have the same combination index as xn, then its variance estimate is unchanged, i.e., λxi,n=λxi,n-1=Λqxi,n-1.

In summary, the variance estimates evolve iteratively for stage n=1, 2,…,N as following:

λxi,n=λxi,n-1s0+ sxn-1+yi,n-θxi,n2s0+ sxn,                                    if x=xnλxi,n-1+λxni,n-λxni,n-1rqxn,      if x≠xn, sxn=0,qx=qxnλxi,n-1,                                                                       otherwise       (15c)

So after xn each stage n, we just need to update the variance estimates corresponding to xn λxniΛqxniby the process described above. The variance estimate of any other xnsolution is either set equal to Λqxni,n or remain unchanged. We will see in Section 3.3 that this variance updating approach also help make our sampling algorithm efficient.

4.2.3 The reformulated decision model for NSTPU based on parametric beliefs

Finally, using the proposed parametric belief models, problem (11) can be reformulated as

minx∈X fxD,NbD,N,fxA,NbA,N                             s.t.  fxE,NbE,N≤ τE-α1-EλxE,N1/2                      (16)

where we define α1-E=Φ-1(1-E). Problem (16) seems nontrivial as there is a solution specific righthand side value in the constraint. However, we note that (15) ensures that the posterior variance estimate λxE,N has a common value for those x’s that have not been sampled by stage N and have the same combination index. Therefore, we design the following strategy in solving (16). Firstly, we solve a problem by using only information from the posterior mean of the coefficients βji, j=1,…,Mi, for i∈{D, A, E} and applying a common posterior variance estimate ΛwE,N to all solutions x that have the same combination index w, as following:

minx∈X j=1MDbjD,NfjDx,j=1MAbjA,NfjAx                                                                 s.t.  j=1MEbjE,NfjEx≤τE-α1-EΛj,wE,N12                                                                fjix∈fj,lwi,∀ w∈1,…,j=1Mimji,j∈1,…,Mi,i∈D, A, E, (17)

where bji,N is the j-th entry of bi,N, i∈{D, A, E}. By our definition of the partition fj,li, l=1,…,mji for the domain Fji, j=1,…,Mi, one natural way to solve (17) is by solving a set of j=1Mimji subproblems and comparing their results to derive the Pareto optimal solutions of (17). Each of these subproblems corresponds to a unique combination index w, and can be solved by standard multi-objective solver such as NSGAII. The next step is that for each sampled solution x, we evaluate the objective values fxD,NbD,N and  fxA,NbA,N, as well as check its feasibility fxE,NbE,N≤ τE-α1-EλxE,N1/2. This step can be done conveniently as the simulation budget N is generally much smaller than the total number of solutions in X. Finally, we can simply compare the objective values of the sampled feasible solutions to those of the Pareto optimal solutions of (17) (with those sampled ones deleted if any) to form the true Pareto optimal solutions for the original problem (16).

4.3 The Knowledge Gradient (KG) based sampling policy

We adopt the knowledge gradient based sampling policy for solving problem (16). Specifically, at each sampling stage n=1,…,N, we go through the following steps:

1) Generate a set of candidate solutions, Sn, which is the union of two separately generated sets, i.e., Sn= S1n∪S2n. The first set S1n is simply a random selection of K1 solutions x∈X. While the second set S2n contains the solutions that are close to the optimizers (i.e., nondominant solutions) of the deterministic optimization problem (16) (with subscript N replaced by n). Formally, we define Pn as the Pareto optimal front of problem (16) (with subscript N replaced by n):

Pn=x∈X:∄ x∈X s.t. fxE,NbE,N≤ τE-α1-EλxE,N12, fxibi,n><fxibi,n ∀i∈D, A,   (18)

if Pn-1 contains more than K2 solutions, we simply choose S2n= Pn-1; otherwise we generate K2-|Pn-1| new solutions each by firstly choosing a x uniformly at random from Pn-1 and then sampling a solution from Nx,Cn and rounding it to the nearest integer solution in X. This approach of sampling set generation is given by Algorithm 2 in the Appendix.

2) Compute the Knowledge Gradient (KG) factor associated with evaluation indices i∈{D, A} for each candidate solution in Sn, the KG factor for evaluation index i of solution x before sampling at stage n, denoted by vKG,i, n(x), is defined as the expected improvement in the best posterior mean of evaluation index i among all the candidate solutions if x∈Sn is sampled given the historical sampling decisions Xn-1 and observations Yn-1 (i∈{D, A}):

vKG,i,nx=minx∈Snθxi,n-1-En-1minx∈Snθxi,n|xn=x

=minx∈Snfxibi,n-1-En-1minx∈Snfxibi,n|xn=x                   (19)

Algorithm 3 and Algorithm 4 in the Appendix detail the computation of the KG factor vKG,i, n(x), the key of which is to utilize the conditional predictive distribution pn-1θi,n|xn=x, which has mean and covariance as (these quantities can be derived from (14)):

En-1θi,n|xn=x=θi,n-1=fxi,n-1bi,n-1,   Varn-1θi,n|xn=x=σi,n-1(x)(σi,n-1(x))T,  (20a)

where the column vector σi,n-1(x), as a function of x, can be expressed as

σi,n-1x=Fi,n-1xBi,n-1x(fxi,n-1)T,                     (20b)

where Fi,n-1x=Fi,n-1 and Bi,n-1x=Bi,n-1 if x has been sampled before stage n; otherwise, Fi,n-1x is formed by appending a column to the right of Fi,n-1 with the entry corresponds to solution x being one and others zero, and Bi,n-1 is formed by appending a row and a column to Bi,n-1, with the new diagonal entry zi,0 and all the other new entries zero, as shown below:

Fi,n-1  FFi,n-1   ex ,   Bi,n-1  BBi,n-1   00        zi,0 ,

where x is the index of solution x in X, and ex is a unit column vector with x-th entry one.

3) Pick the sampling decision xn as any solution in Sn with nondominant KG factors for two evaluation indices i=D, A, i.e., xn∈PKG,n, where PKG,n denotes the Pareto optimal front of (vKG,D,n(x), vKG,A,n(x)) over x∈Sn:

PKG,n=x∈Sn:∄ x∈Sn s.t. vKG,i,nx>vKG,i,nx ∀i∈{D, A},         (21)

certain rule can also be enforced as to which solution in the Pareto front to be sampled, e.g., we can pick the one with the largest crowding distance (i.e., the normalized sum of its distances to its adjacent solutions in the Pareto front with respect to each objective function).

4) Simulate the signal plan xn and observe the traffic flow output, then compute the evaluation indices yn=(yD,n,yA,n,yE,n). Update posterior beliefs on model parameters βi,n by (14), and update the variance estimates by (15).

5) If n=N (i.e., the simulation budget is exhausted), we solve problem (16) and obtain an optimizer for final implementation. Similar to the case for selecting the sampling decision, certain rule can be applied for selecting the final implementation plan if there is more than one nondominant solutions to (16), e.g., choose the solution with the largest crowding distance.

The above solution procedure is summarized in Algorithm 1 in the appendix.

4.4 The choice of the basis functions for NSTPU

We adopt a polynomial basis function with at most order of three. Clearly, the higher the order of the polynomial, the more likely the model can better fit the true underlying function, but more terms including the cross-multiplication terms have to be included in the belief model. More specifically, the size of the coefficient vector βi grows exponentially in the dimension n of the solution, a higher order polynomial model can result in significantly more computational cost. As noted by (MIT2013), a quadratic polynomial can already provide decent approximation for traffic network problems.

5 Numerical experiments

This section aims to validate the performance of GPR-KG in addressing the biobjective stochastic discreate optimization problem with a probabilistic constraint. First, the GPR-KG algorithm is tested by solving three synthetic constrained biobjective stochastic discrete optimization problems, whose theoretical Pareto optimal solutions are known. Then, it is applied to address a real-field NSTPU problem, whose objectives and constraint are all evaluated by costly stochastic simulation.

5.1 Test problems

By introducing stochastic noises and probabilistic constraint expressions and discretizing decision variables, the revised versions of ZDT1, ZDT2 and ZDT5 (Zitzler et al. 2000) come into being and are treated as the benchmark problems. Specifically, the white noises are added to each objective function and constraint function, respectively, and then biobjectives and constraint are expressed by their expectation and probabilistic counterparts. Obviously, such noises incorporated veil the relationship between the decisions and biobjective expectations/the probability of violating the constraint, and the dominance relationship between two decisions. This in fact makes the revised ZDT1, ZDT2 and ZDT5 be challenging to be solved and their formulations are given as follows.

1) Revised ZDT1 (RZDT1):

minLx,ξ=Eξl1x,ξ,Eξl2x,ξs.t.   Prξcx,ξ≤0≥1-E                                       (22)

where

l1x,ξ=x1100+ξ                                                                       l2x,ξ=1+9d-1j=2dxi100∙1-l1x,ξ1+9d-1j=2dxi100+ξcx,ξ=-x1100-0.52+0.04+ξ

x=(x1,x2,⋯,xd)∈Zdxi∈0,100, i=2,3,⋯,dξ~N(0,0.05)ε=0.05x1∈0,1,⋯,15∪{85,86,⋯,100}xi=0,i=2,3,⋯,d

2) Revised ZDT2 (RZDT2):

minLx,ξ=Eξl1x,ξ,Eξl2x,ξs.t.   Prξcx,ξ≤0≥1-E                                        (23)

where

l1x,ξ=x1100+ξ                                                                       l2x,ξ=1+9d-1j=2dxi100∙1-l1x,ξ1+9d-1j=2dxi1002+ξcx,ξ=-x1100-0.52+0.04+ξ

x=(x1,x2,⋯,xd)∈Zdxi∈0,100,i=1,2,⋯,d .ξ~N(0,0.05)ε=0.05x1∈0,1,⋯,15∪{85,86,⋯,100}xj=0,,j=2,3,⋯,d

3) Revised ZDT5 (RZDT5):

minLx,ξ=Eξl1x,ξ,Eξl2x,ξs.t.   Prξcx,ξ≤0≥1-E                                     (24)

where

l1x,ξ=x130+ξ                                     l2x,ξ=j=2dxi5+1∙1x1+1+ξcx,ξ=x130-0.5+ξ

x=(x1,x2,⋯,xd)∈Zdx1∈[0,30]xi∈0,5,i=2,3,⋯,dξ~N(0,0.052)ε=0.05x1∈{0,1,⋯,13}xj=0,j=2,3,⋯,d

For RZDT1, RZDT2 and RZDT5, the decision vector x=(x1,x2,⋯,xd)∈Zd, and xi∈0,100, i=1,2,3,⋯,d. The white noise is ξ~N(0,0.05), and ε=0.05. Then, the theoretical Pareto optimal solutions of RZDT1 and RZDT2 are x1∈0,1,⋯,15∪{85,86,⋯,100}, and xi=0,i=2,3,⋯,d, and those of RZDT5 are x1∈{0,1,⋯,13}, and xj=0,j=2,3,⋯,d. Here, the dimension d is set as 5.

To solve RZDT1, RZDT2 and RZDT5, the critical parameters 	of the GPR-KG algorithm are set as: the maximum number of iterations N=100, the number of pre-samples N0=11, the weight for the initial variance estimate s0=0.5, the number of random candidate samples K1=10, and the number of candidate samples generated by solving the current posterior parametric belief model K2=90. Moreover, three priori parametric belief models are built to estimate the underlying mean values of biobjectives and the constraint of any decision x and each one is a linear combination of 2×d basis functions: f1ix=x1,⋯, fdix=xdn, fdn+1ix=x12,⋯, f2dnix=x1xd, i∈{l1, l2, c}. For each basis function, we divide its domain into two equal partitions, and so 4d combinations of the partitions exist with the combination index qx∈{1,2,⋯,4d}. The initial estimate of the variance of each combination is set as Λwi,0=0.01, i∈{l1, l2, c}, w=1,2,…,4d.

Numerical results in Fig. 1 illustrate that the biobjective points of learned solutions include two parts, that is, most points lying on the theoretical Pareto front (c.f., blue triangles), and only some ones not lying on the theoretical Pareto front and usually on its left-side (c.f., gray triangles). This indicates the good performance of GPR-KG in addressing the biobjective stochastic discrete optimization problem with a probabilistic constraint at the cost of limited computational resources. Specifically, in the iterative process, the biobjective points of iterative solutions usually scatter in the regions close to the theoretical Pareto front, and some of them fortunately locate on it, which helps the parametric belief models to learn the information about theoretical pareto front. However, there are also some biobjective points of iterative solutions that are distant from the theoretical pareto front or even infeasible (c.f., gray asterisks). This generally results from the following two causes. That is, the candidate solutions selected by the KG factor at each iterate might be from S1n, which is sampled randomly, and so may contain the dominated or infeasible solutions; the early posterior parametric belief models fitted with limited observations might be less accurate, which causes a high probability of not finding the biobjective points closed to the theoretical Pareto front and not complying with the probabilistic constraint.

(a)                       (b)                       (c)

Fig. 1 Biobjective points of iterative solutions and finally searched Pareto optimal solutions by GPR-KG for (a) RZDT1, (b) RZDT2 and (c) RZDT5. Note: the iterative solutions represent the solutions selected from Sn by the corresponding biobjective KG factors. The learned solutions represent the finally optimized solutions when GPR-KG terminates. The biobjective points of all solutions in this figure are evaluated by real (or nominal) objective and constraint functions without stochastic noises.

Meanwhile, a reliable performance indicator named inverted generational distance (IGD) (Zitzler et al. 2003) is introduced to quantify the closeness of the generated non-dominated front to a set of reference points and its diversity. It is formulated as IGD=pr∈Pminpo∈Opr-po2/|P|, where P is the set of reference points in the theoretical Pareto front, O is the set of biobjective points of optimized (or iterative) solutions generated by GPR-KG and |∙| denotes the cardinality of a set. Obviously, the smaller IGD means the better convergence and diversity of the optimized (or iterative) solutions. During the iteration process of GPR-KG, we obtain the iterative non-dominated solutions by the posterior parametric belief models and their posterior variance estimates at each iteration and then compute the corresponding IGD value. Moreover, to mitigate the impact of randomness, we carry out 10 independent runs of GPR-KG for RZDT1, RZDT2 and RZDT5, respectively, so as to calculate the corresponding average IGD values.

Numerical results in Fig. 2 show that the average IGD values decrease rapidly in the early few iterations, then the decline decelerates and finally reaches a very small level. This indicates the good convergence performance of the GPR-KG algorithm. Meanwhile, detailed results in Table 1 tell us that the learned Pareto optimal solutions by GPR-KG at the cost of 111 function evaluations cover most or almost all theoretical Pareto optimal solutions, and their final IGD values are 0.055, 0.022 and 0.005 for RZDT1, RZDT2 and RZDT5, respectively. These results further validate the good convergence, diversity and efficiency of GPR-KG in dealing with the biobjective stochastic discrete optimization problems with probabilistic constraints.

(a)                        (b)                        (c)

Fig. 2 Average IGD values over 10 independent runs of GPR-KG for solving (a) RZDT1, (b) RZDT2 and (c) RZDT5.

Table 1 Detailed average results over 10 independent runs of GPR-KG.

| Test problem | No. of FEs. | No. of TPOS. | A. No. of LPOS. |  | A. IGD | A. Comp. time |
| --- | --- | --- | --- | --- | --- | --- |
| RZDT1 | 111 | 32 | 18.5 |  | 0.055 | About 50 min |
| RZDT2 | 111 | 32 | 16.3 |  | 0.103 | About 50 min |
| RZDT5 | 111 | 12 | 11.1 |  | 0.005 | About 50 min |

Note: No. of FEs. denotes the number of function evaluations for each run of GPR-KG, and No. of TPOS. is the number of theoretical Pareto optimal solutions. A. No. of LPOS., A. IGD and A. Comp. time represent the average number of learned Pareto optimal solutions, the average IGD value and the average running time over 10 independent runs of GPR-KG, respectively.

Moreover, we also use the iterative non-dominated solutions as the testing set to evaluate the estimation (prediction) errors of the current posterior parametric belief models with respect to biobjectives and probabilistic constraint during the iteration process. These errors are measured by Root Mean Square Errors (RMSEs) based on the estimated (predicted) and real (nominal) biobjective and constraint function values. Fig. 3 apparently shows that average RMSEs are relatively high in the early few iterations, and then decrease rapidly to reach a small and stable level. This implies the more and more accurate posterior parametric belief models and posterior variance estimates as the iteration goes on (i.e., the number of accumulative evaluated samples increases). Specifically, after 50 iterations, average RMSE of Eξ(l1) reaches as small as about 0.021, 0.022 and 0.018 for RZDT1, RZDT2 and RZDT5, respectively; average RMSE of Eξ(l2) reaches as small as about 0.227, 0.123 and 0.159 for RZDT1, RZDT2 and RZDT5, respectively; and average RMSE of Eξc+Φ(1-ε)∙σ reaches as small as about 0.009, 0.006 and 0.025 for RZDT1, RZDT2 and RZDT5, respectively. So, the fitted posterior parametric belief models after 50 iterations in fact are accurate enough and can well approximate the response surfaces of the two objectives and the probability constraint.

(a)                         (b)                         (c)

Fig. 3 Average estimation (prediction) errors of the current posterior parametric belief models with respect to (a) Eξ(l1), (b) Eξ(l2) and (c) Eξc+Φ(1-ε)∙σ, over 10 independent runs of the GPR-KG algorithm for solving RZDT1, RZDT2 and RZDT5. Note: ε=0.05, σ=0.05.

In addition, we evaluate each of the iterative non-dominated solutions by 100 times of stochastic constraint function (i.e., cx,ξ) and determine it as an infeasible solution if it violates the constraint (i.e., cx,ξ>0) more than five times. cx,ξ>0Numerical results in Fig. 4 illustrate that average number of infeasible solutions in the iterative non-dominated solutions is relatively high in the early few iterations, and then decreases rapidly to stabilize at zero. This validates the good ability of the GPR-KG algorithm in dealing with the probability constraint.

(e)                       (f)                     (g)

Fig. 4 Average number of infeasible solutions in the iterative non-dominated solutions over 10 independent runs of the GPR-KG algorithm for solving (a) RZDT1, (b) RZDT2 and (c) RZDT5.

Last but not least, we further carry out the sensitivity analyses of the performance of the GPR-KG algorithm with respect to three critical parameters, i.e., N, Λ and N0. ΛObviously, Fig. 5(a) Λwi,0shows the decreasing trend of average IGD values as N increases. That is, the convergence and diversity performance of the GPR-KG algorithm would be improved by consuming more computation burden (i.e., more function evaluations). This is because the more accurate posterior parametric belief models fitted with more observations are conducive to searching more non-dominated solutions close to the theoretical Pareto optimal ones. Fig. 5(b) indicates the increasing trend of the average IGD values as Λwi,0 increases, which results from that the higher Λwi,0 would produce more conservative non-dominated solutions against the probabilistic constraint. Fig. 5(c) illustrates the average IGD values are rather insensitive to N0, which means the fitting accuracies of the priori parametric belief models do not influence the convergence and diversity performance of the GPR-KG algorithm. Moreover, the sensitivity analyses with respect to N and Λwi,0 also explain why we set N and Λwi,0 as high as 100 and 0.01 respectively in aforementioned numerical experiments. That is, the performance of the GPR-KG algorithm under N=100 or Λwi,0=0.01 is satisfactory, and further increasing N or decreasing Λwi,0 would not help significantly improve the performance. Λwi,0Λwi,0

(a)                         (b)                         (c)

Fig. 5 Average IGD values over 10 independent runs of the GPR-KG algorithm for solving RZDT1, RZDT2 and RZDT5 under various values of (a) N, (b) Λwi,0 and (c) N0.

5.2 Network-wide signal timing problem

Signalized intersection plays an important role in the network-wide traffic efficiency, traffic equity and vehicular emissions, and it is also a hot and difficult topic to study the network-wide signal timing optimization considering multi-criteria. This application employs the microscopic traffic simulator VISSIM as an evaluation tool of network-wide signal timing plan, due to its flexibility of designing any type of traffic element, and capability of capturing the complicated relationship between the signal timing plan and multi evaluation indices. Meanwhile, the mature COM interface in VISSIM also allows the inter-process communication and data exchange between VISSIM and other compatible computational software (e.g., MATLAB). So, we first construct the SO platform by integrating VISSIM with MATLAB through the COM interface. Then, an urban road network in Changsha, China is modelled by VISSIM as the experimental scenario, which consists of 15 signalized intersections, 5 non-signalized intersections, 29 entries, 24 exits and 31 road links (c.f., Fig. 6). The detailed calibration process and results about the traffic simulation model can refer to Zheng et al. (2019a, 2019b). In the SO platform, given a specific network-wide signal timing plan, the calibrated traffic simulator will output individual vehicular information after completing the traffic simulation run, which can be used to calculate three typical traffic evaluation indices (i.e., D, A and E in Appendix A).

- (b)

Fig. 6 Study site, (a) a regional urban road network in Changsha, China, and (b) its VISSIM model.

For this NSTPU problem, the decision vector x consists of the signal timings of 47 signal phases in this regional urban road network. The perturbed network inflow vector is expressed by VI1 ×29∙ξ, where VI1 ×29 is the calibrated network inflow vector of 29 entries in this network (Zheng et al., 2019a, 2019b) and can be treated as the nominal network inflow vector. τE=0.9 and E=0.05 are set for the probabilistic constraint. Several key parameters of the GPR-KG algorithm are set as: N=300, N0=100, s0=0.5, K1=10, and K2=90. Meanwhile, the same type of combination of the basis functions in section 5.1 is applied here. To reduce the computation burden, only the domains of some particular basis functions instead of all ones are divided into two equal partitions. These particular basis functions are the linear item ones which correspond to green timings of signal phases for five intersections numbered 2, 6, 8, 12 and 14 along two main roads (i.e., Shaoshan Rd and Furong Middle Rd), c.f., Fig. 6(b)). So, there are 220 combinations of these partitions with the combination index qx∈{1,2,⋯,220}. The initial estimate of the variance of each partition is set as Λwi,0=0.03, i∈{D, A, E}, w∈=1,…,2201,…,220, which is determined by a small number of simulation evaluations.

During the numerical experiment, 100 simulation evaluations for pre-samples are used for initially fitting the parametric belief models with respect to biobjectives and probabilistic constraint, and another 300 simulations are allocated to evaluate the infilled signal control plans in the latter iterations. It takes about 30 hours on an Intel Xeon 2.0 GHz to terminate the GPR-KG algorithm, and then the non-dominated solutions (that correspond to learned PF) result from solving the final posterior parametric belief models. Meanwhile, all iterative signal control plans also generate their own non-dominated solutions (that correspond to PF for iterative solutions). Then, above two types of non-dominated solutions result in the final ones that correspond to the final PF. Aforementioned numerical results in Fig. 7 show that the final non-dominated solutions significantly perform better than the iterative ones from three aspects of D, A and E. In particular, the E values of all non-dominated solutions are smaller than 0.9, which implies the satisfaction of the probalistic constraint, while those of the early iterative solutions cannot guarantee this aspect. In addition, the dispersive values of D, A and E of iterative signal control plans also indicates the extensive exploration capability of the GPR-KG algorithm in the domain space of three evaluation indices.

E≥0.9

Fig. 7 D, E and A values of iterative and non-dominated signal control plans by GPR-KG. Note: the tri-objective points of all iterative and non-dominated solutions in this figure are (re)evaluated by the final posterior parametric belief models.

Furthermore, we compare nine signal control plans by performing 100 independent simulation evaluations for each comparison solution so as to mitigate the influence of stochastic network inflows, that is, five non-dominated solutions xo1 to xo5 (that correspond to learned PF), three non-dominated solutions xc1 to xc3 (that correspond to PF for iterative solutions) and the field-implemented signal control plan xFSP. Numerical results in Fig. 8 show that various percentile values and fluctuation ranges of D, A and E values by 100 independent simulation evaluations for each non-dominated signal control plan are smaller than those for the field-implemented one. This in fact validates the effectiveness of GPR-KG in solving the NSTPU problem. Moreover, xo1 to xo5 do not perform worse than or outperform xc1 to xc3 in terms of D, A and E, which means the final posterior parametric belief models are accurate enough to generate the well-performed solutions. From Fig. 9, similar conclusions can also be found and only xo1 to xo5 are able to comply with the probabilistic constraint.

xo1xo2xo3xo4xo5xc1xc3xc2

In addition, more detailed comparison results of eight non-dominated signal control plans are given in Table 3. Obviously, xo1 to xo5 improve D, A and E by at most 16.2%, 14.2% and 22.6%, respectively, and xc1 to xc3 improve D, A and E by at most 11.4%, 14.7% and 15.7%, respectively. At the same time, the D, A and E variances for xo1 to xo5 are smaller or not larger than those for xc1 to xc3. Particularly, the 95% quantities of the E values for xo1 to xo5 are all smaller than 0.9, while those for xc1 to xc3 are all higher than 0.9. These results in fact accord with those in Fig. 9(c). This means that the non-dominated solutions resulting from iterative solutions do not obey the probability constraint, although being able to achieve good or satisfactory performance in terms of D, A and E. Fortunately, the non-dominated solutions resulting from the final posterior parametric belief models can overcome the issue of violating the probability constraint. This is because the final posterior parametric belief models and posterior variance estimates have more accurate estimation (or prediction) of the mean values of biobjectives and the probabilistic constraint than the intermediate parametric belief models and variance estimates used to determine the iterative solution. In conclusion, all above numerical results validate the good ability of GPR-KG in addressing the NSTPU problem.

(a)                         (b)                          (c)

Fig. 8 Boxplots of (a) relative total delay, (b) relative traffic equity and (c) relative vehicular emission for nine comparison solutions. xFSPxc1xc2xc3xo1xo2xo3xo4xo5

(a)                         (b)                       (c)

Fig. 9 Empirical CDFs of (a) relative total delay, (b) relative traffic equity and (c) relative vehicular emission for nine comparison solutions.

Table 3 Detailed comparison of eight non-dominated signal control plans.

| Signal control plan | D | A | E | (PD, PA, PE) | (VarD, VarA, VarE) | E1-ε |
| --- | --- | --- | --- | --- | --- | --- |
| xc1 | 0.890 | 0.902 | 0.843 | (11.0%↓, 9.8%↓, 15.7%↓) | (0.004,0.002, 0.005) | 0.960 |
| xc2 | 0.886 | 0.853 | 0.859 | (11.4%↓, 14.7%↓, 14.1%↓) | (0.004,0.001, 0.005) | 0.915 |
| xc3 | 0.966 | 0.910 | 0.915 | (3.4%↓, 9.0%↓, 8.5%↓) | (0.003,0.001, 0.007) | 1.022 |
| xo1 | 0.876 | 0.885 | 0.792 | (12.4%↓, 11.5%↓, 20.8%↓) | (0.001, 0.002, 0.004) | 0.893 |
| xo2 | 0.896 | 0.858 | 0.806 | (10.4%↓, 14.2%↓, 19.4%↓) | (0.003, 0.001, 0.004) | 0.899 |
| xo3 | 0.860 | 0.889 | 0.796 | (14.0%↓, 11.1%↓, 20.4%↓) | (0.002,0.001, 0.003) | 0.871 |
| xo4 | 0.844 | 0.903 | 0.780 | (15.6%↓, 9.7%↓,22.0%↓) | (0.002, 0.001, 0.004) | 0.878 |
| xo5 | 0.838 | 0.917 | 0.774 | (16.2%↓, 8.3%↓, 22.6%↓) | (0.001, 0.001, 0.003) | 0.861 |

Note: D, A and E denote the average D, A, and E values over 100 independent simulation evaluations, respectively, and VarD, VarA and VarE represent the corresponding D, A, and E variances, respectively. Compared with the benchmark values (1,1,1) of the field-implemented signal control plan, PD, PA and PE represent the percentage changes of D, A and E, Erespectively, for a non-dominated signal control plan. E1-ε denotes 1-ε quantile of the E values over 100 independent simulation evaluationsε=0.05. ↓ is the improvement indicator for D, A and E.

7 Conclusion

Appendix A: Three evaluation indices of the network signal timing

Given a network-wide signal timing plan, the calibrated traffic simulation model will output the individual vehicle information, which helps to calculate the evaluation indices about traffic efficiency, traffic equity and vehicular emissions. As for the traffic efficiency, the relative total delay (c.f., Eq.(a1)) is utilized as the evaluation index.

D=h=1Nvdh/C                                (a1)

where dh is the delay experienced by vehicle h during the simulation, Nv is the total number of vehicles during the simulation, C is the normalization constant regarding the total delay from the calibrated traffic simulator with the field implemented signal plan. Obviously, the lower D value means the higher traffic efficiency.

As for the traffic equity, the Atkinson index proposed to measure the inequity in social welfare distribution (Atkinson, 1970) is adopted and formulated as

At(tr,1,tr,2,⋯,tr,h,⋯,tr,Nv)=1-1π1Nvh=1Nvtr,h1-η11-η,for 0≤η≠11-1πh=1Nvtr,h1Nv,for η=1                                  (a2)

where tr,h=th/tJh denotes the travel time ratio of vehicle h that can compare the benefits achieved by travelers from the same and various routes, th is the travel time of vehicle h, Jh denotes the route that vehicle h traverses, tJh=h=1|Jh|th/|Jh| is the average travel time of route Jh, |Jh| indicates the number of vehicles from route Jh. That is, tJh are measured by the average travel time of all vehicles from route Jh. π is the mean travel time ratio. η is often called the “inequality aversion parameter”. When η=0 (no aversion to inequity), At=0. When η=∞ (infinite aversion to inequity), At=1. So, At varies between 0 and 1, and the lower At for a given η value means the more equal distribution of travel time ratios.

Moreover, to describe the relative change of traffic equity under various signal plans, the relative Atkinson index is given as

A=At/At0                                (a3)

where At0 is the Atkinson index evaluated by the calibrated traffic simulator with the field implemented signal plan, and treated as the normalization constant regarding the traffic equity.

As for the vehicular emissions, a linear combination of four pollutants’ emissions (Mayeres et al., 1996; Osorio and Nanduri, 2015b) is employed and formulated as

EM=wCO2nCO2fCO2+wNOxnNOxfNOx+wVOCnVOCfVOC+wPMnPMfPM             (a4)

where wCO2, wNOx, wVOC and wPM are the economic weighting parameters of CO2, NOx, VOC, and PM pollutants, respectively, and nCO2, nNOx, nVOC and nPM are normalization constants for these pollutants. The total emission (g) of pollutant p∈{CO2, NOx, VOC, PM} during the simulation is calculated by

fp=h=1Nvt=thThERhpt                            (a5)

where ERhpt=maxE0hp;C1hp+C2hpvh(t)+C3hpvh(t)2+C4hpah(t)+C5hpah(t)2+C6hpvh(t)ah(t) is the instantaneous emission rate of pollutant p (g/s) for vehicle h at time t (Panis et al., 2006), th and Th are the beginning and ending time-instants of vehicle h appearing in the simulation network, respectively. vh(t) and ah(t) denote the instantaneous speed (m/s) and the instantaneous acceleration (m/s2) of vehicle h at time t, respectively. E0hp is the minimal emission rate of pollutant p (g/s) for vehicle h, and C1hp to C6hp are emission rate coefficients for pollutant p and vehicle h.

Similarly, to describe the relative change of vehicular emissions under various signal timing plans, the relative vehicular emission is used and expressed as

E=EM/EM0                                (a6)

where EM0 is the vehicular emissions computed by the calibrated traffic simulator with the field implemented signal plan, and treated as the normalization constant regarding vehicular emissions.

Appendix B: Pseudo-codes of algorithms.

Algorithm 1: GPR based simulation optimization algorithm for solving NSTPU

Inputs: bi,0, Bi,0, zi,0, Fji, fj,li,l=1,…,mji, Λwi,0,w=1,…,j=1Mimji, for  i∈D, A, E, j=1,…,Mi, Cn for n=1,…,N, K1, K2, xL, xU;

Output: PN (the Pareto optimal front)

1:  s0  an empty matrix % stores sampled solutions and their times sampled

2:  for n = 1 to N do

3:    generate Sn by Algorithm 2

4:    for k =1 to |Sn| do

5:        x  k-th element in Sn

6:      if n ==1  or  xn ≠ any entry of 1st column of sn-1 and qx==qxn-1

7:         λx i,n  Λq(x)i,n-1

8:      else

9:         λx i,n  λx i,n-1

10:     end if

11:   end for

10:   compute xn by Algorithm 3

12:   simulate solution xn and observe output yn=(yD,n,yA,n,yE,n)

13:   compute bi,nand Bi,n (i∈{D, A, E}) by formula (14)

14:   compute λxn i,n by (15a) and Λq(xn)i,n by (15b)

15：  if xn ≠ any entry of 1st column of sn-1

sn sn-1 x    1

16:    else

snsn-1 with the last entry of the row corresponds to x added by 1

17:  end if

18: end for

19: solve problem (16) and obtain its Perato front PN defined in (18)

Algorithm 2: Parametric belief based candidate sample set generation for NSTPU

Inputs: Cn, K1, K2, xL, xU;  Output: Sn

1:   Sn  Ø

2:   for k =1 to K1 do

3:    pick an integer vector x randomly from [xL,xU]

4:    if x∉Sn then

5:      Sn  Sn∪{x}

6:    end if

7:   end for

8:   solve for the Perato front Pn-1 defined in (18) of problem (16) (with N replaced by n-1)

9:   if |Pn-1|≥ K2

10:    Sn  Sn∪Pn-1

11:  else

12:    for k =1 to K2-Pn-1 do

13:      select a solution x randomly from Pn-1

14:      sample x from Nx,Cn and round to integer values.

15:      x  minmaxx,xL,xU

16:      if x∉Sn then

17:        Sn  Sn∪{x}

18:      end if

19:    end for

20:  end if

Algorithm 3: KG-based sampling decision for NSTPU

Inputs: Sn, sn-1, x, bi,n-1, Bi,n-1, zi,0, λxi,n-1,x∈Sn; Output: xn

1:   VKG,n  a |Sn|-by-3 matrix

2:   PKG,n  Ø

3:   for k =1 to |Sn| do

4:    x  k-th element in Sn

5:    for i = 1 to 2 do % 1 and 2 represent “D” and “A”, respectively

6:       compute vKG,i,nx using Algorithm 4

7:    end for

8:   k-th row of VKG,n k, vKG,1,nx,vKG,2,nx

9:   end for

10: sort the rows of VKG,n in nondecreasing order of its 2nd column

11: for k =1 to |Sn| do

12:  if k =|Sn| or  maxk'=k to |Sn| VKG,n(k',3) ≤VKG,n(k,2)

13:    x  k-th element in Sn

14:    PKG,n  PKG,n∪{x}

15:  end if

16: end for

17: return xn  an element in PKG,n %pick arbitrarily or by certain rule

Algorithm 4: Computation of the KG factor

Inputs: Sn, sn-1, x, bi,n-1, Bi,n-1, zi,0, λxi,n-1,x∈Sn; Output: vKG,i,n

1:  index each element in  Sn by 1 to |Sn|

2:  x  the index of solution x

3:  Fi,n-1 a |Sn|-by-dim(βi,n-1) matrix

4:  for k = 1 to |Sn|

5:    x' k-th element in Sn

6:    k-th row of Fi,n-1  fx'i,n

7:  end for

8:  Bi,n-1  Bi,n-1

9:  if x ≠ any entry of 1st column of sn-1 % x has not been sampled

10:       Fi,n-1  Fi,n-1   ex  % add a column vector ex to the right

11:       Bi,n-1  Bi,n-100        zi,0

12: end if

13: p  Fi,n-1bi,n-1

14: q  Fi,n-1Bi,n-1(fxi,n-1)T/λxi,n-1+fxi,n-1Bi,n-1(fxi,n-1)T1/2

# 15: sort the pairs (pj,qj)j=1|Sn| so that qj are in non-decreasing order, and ties in q are broken so that pj≤pj+1 if qj=qj+1

16:  remove each pair j in (pj,qj)j=1|Sn| if (pj,qj)j=1|Sn| studyate intger p alternatives in each iteration

17:  J  the number of remaining pairs (pj,qj)

18:  a0-∞, aJ+∞

19:  for j =1 to J-1 do

20:     aj(pj-pj+1)/(qj+1-qj)

21:  end for

22: remove each triplet j in (pj,qj,aj)j=1J if aj≥aj+1

23:  J'  the number of remaining triplets (pj,qj,aj)

24: return vKG,i,nlogj=1J'-1φ(-|aj|)-|aj|ϕ(-|aj|)

References

Levinson, D., Zhang, L., 2006. Ramp meters on trial: evidence from the Twin Cities metering holiday. Transport. Res. Part A 40 (10), 810–828.

Ma, J.T., Zhang, M.H., 2008. An Efficiency-Equity Solution to the Integrated Corridor Control Problem. Institute of Transportation Studies. University of California Davis, Research Report: UCD-ITS-RR-08-51.

Meng, Q., Khoo, H.L., 2010. A Pareto-optimization approach for a fair ramp metering. Transport. Res. Part C 18 (4), 489–506.

Zheng, L., Xue, X., Xu, C., Ran, B., 2019a. A stochastic simulation-based optimization method for equitable and efficient network-wide signal timing under uncertainties. Transportation Research Part B 122, 287-308.

Zheng, L., Xu, C., Jin, P.J., Ran, B., 2019b. Network-wide signal timing stochastic simulation optimization with environmental concerns. Applied Soft Computing Journal 77, 678-687.

Unal, A., Rouphail, N.M., Frey, H.C., 2003. Effect of arterial signalization and level of service on measured vehicle emissions. Transp. Res. Rec. 1842, 47–56.

Rakha, H., Ding, Y., 2003. Impacts of stops on vehicle fuel consumption and emissions. ASCE J. Transp. Eng. 129 (1), 23–32.

Stevanovic, A., Stevanovic, J., Zhang, K., Batterman, S., 2009. Optimizing traffic control to reduce fuel consumption and vehicular emissions. Integrated approach with VISSIM, CMEM, and VISGAOST. Transp. Res. Rec. 2128, 105–113.

Osorio, C., Nanduri, K., 2015a. Energy-efficient urban traffic management: a microscopic simulation-based approach. Transport. Sci. 49 (3), 637–651.

Osorio, C., Nanduri, K., 2015b. Urban transportation emissions mitigation: coupling high-resolution vehicular emissions and traffic models for traffic signal optimization. Transport. Res. Part B 81, 520-538.

Zhang, L., Yin, Y., Chen, S., 2013. Robust signal timing optimization with environmental concerns. Transp. Res. C 29, 55–71.

Kotsialos, A., Papageorgiou, M., 2004. Efficiency and equity properties of network-wide ramp metering with AMOC. Transport. Res. Part C 12 (6), 401–420.

Zhang, L., Levinson, D., 2004. Optimal freeway ramp control without origin-destination information. Transport. Res. Part B 38 (10), 869–887.

Kesten, A.S., Ergün, M., Yai, T., 2013. An analysis on efficiency and equity of fixed-time ramp metering. J. Transport. Technol. 3 (2A), 48–56.

Cooper K., Hunter S.R., Nagaraj K., 2020. Biobjective simulation optimization on integer lattices using the epsilon-constraint method in a retrospective approximation framework, INFORMS Journal on Computing, Articles in Advance, pp.1-21.

Hong L.J., Nelson B.L., 2006. Discrete optimization via simulation using COMPASS, Operations Research, 54(1), pp.115-129.

Li H.B., Lee L.H., Chew E.P., Lendermann P., 2015. MO-COMPASS: a fast convergent search algorithm for multi-objective discrete optimization via simulation, IIE Transactions, 47, 1153-1169.

Semelhago M., Nelson B.L., Song E., Wachter A., 2020. Rapid discrete optimization via simulation with Gaussian Markov random fields, INFORMS Journal on Computing, Articles in Advance, pp.1-16.

Fan Q., Hu J., 2018. Surrogate-based promising area search for Lipschitz continuous simulation optimization, INFORMS Journal on Computing, 30(4), pp.677-693.

Zheng L., Xue X., Xu C., Ran B., 2019. A stochastic simulation-based optimization method for equitable and efficient network-wide signal timing under uncertainties, Transportation Research Part B, 122, 287-308.

Ryu J., Kim S., 2014. A derivative-free trust-region method for biobjective optimization, SIAM Journal on Optimization, 24(1), 334-362.

Feliot P., Bect J., Vazquez E., 2016. A Bayesian approach to constrained single- and multi-objective optimization, 4th World Congress on Global Optimization (WCGO), Gainesville, FL, FEB 22-25, 2015.

Müller J., Woodbury, J.D., 2017. GOSAC: global optimization with surrogate approximation of constraints, Journal of Global Optimization, 69, 117-136.

Müller J., Shoemaker C.A., Piché R., 2014. SO-I: a surrogate model algorithm for expensive nonlinear integer programming problems including global optimization applications, Journal of Global Optimization, 59, 865-889.

Müller J., Shoemaker C.A., Piché R., 2013. SO-MI: a surrogate model algorithm for computationally expensive nonlinear mixed-integer black-box global optimization problems, Computers & Operations Research, 40, 1383-1400.

Regis R.G., 2011. Stochastic radial basis function algorithms for large-scale optimization involving expensive black-box objective and constraint functions, Computers & Operations Research, 38, 837-853.

Regis R.G., Shoemaker C.A., 2007. A stochastic radial basis function method for the global optimization of expensive functions, INFORMS Journal on Computing, 19(4), 497-509.

Mayeres, I., Ochelen, S., Proost, S., 1996. The marginal external costs of urban transport. Transp. Res. D 1 (2), 111–130.

Panis, L., Broekx, S., Liu, R., 2006. Modeling instantaneous traffic emission and the influence of traffic speed limits. Sci. Total Environ. 371, 270–285.

Osorio, C., Bierlaire, M., 2013. A simulation-based optimization framework for urban transportation problems. Operations Research 61 (6), 1333-1345.

Huang D, Allen TT, Notz WI, Zeng N (2006) Global optimization of stochastic black-box systems via sequential kriging meta-models. J. Global Optim. 34(3):441–466.

Forrester AIJ, Keane AJ (2009) Recent advances in surrogate-based optimization. Progress in Aerospace Sciences. 45(1), 50–79.

Picheny V, Ginsbourger D, Richet Y, Caplin G (2013) Quantile-based optimization of noisy computer experiments with tunable precision. Technometrics 55(1):2–13.

Ankenman B, Nelson BL, Staum J (2010) Stochastic kriging for simulation metamodeling. Oper. Res. 58(2):371–382.

Deng H, ShaoW, Ma Y,Wei Z (2012) Bayesian metamodeling for computer experiments using the Gaussian Kriging models. Quality Reliability Engrg. Internat. 28(4):455–466.

Ng SH, Yin J (2012) Bayesian kriging analysis and design for stochastic simulations. ACM Trans. Modeling Comput. Simulation (TOMACS) 22(3):1–26.

Chen, X., He, X., Xiong, C., Zhu, Z., Zhang, L., 2018. A Bayesian stochastic kriging optimization model dealing with heteroscedastic simulation noise for freeway traffic management. Transportation science, Articles in advance, 1-21.

Quan N, Yin J, Ng SH, Lee LH (2013) Simulation optimization via kriging: A sequential search using expected improvement with computing budget constraints. IIE Trans. 45(7):763–780.

Pedrielli G, Wang S, Ng SH (2020) An extended two-stage sequential optimization approach: Properties and performance. Eur. J. Oper. Res. 287(3):929–945.

Chen CH, Lin J, Yücesan E, Chick SE (2000) Simulation budget allocation for further enhancing the efficiency of ordinal optimization. Discrete Event Dynamic Systems 10(3):251–270.

Chen X, Hemmati S, Yang F (2017) Stochastic co-kriging for steadystate simulation metamodeling. Page EH, Wainer G, Chan V, D' Ambroglio A, Zacharewicz, Mustafee N, eds. Proc. 2017 Winter Simulation Conf. (IEEE, Piscataway, NJ), 1750–1761.

Kennedy MC, O’Hagan A (2000) Predicting the output from a complex computer code when fast approximations are available. Biometrika 87(1):1–13.

Longerstaey J, Martin S (1996) Riskmetrics technical document (Morgan Guaranty Trust Company of New York, New York).

Wipplinger E (2007) Philippe Jorion: Value at risk: The new benchmark for managing financial risk. Financial Markets Portfolio Management 21(3):397–398.

Cope EW, Mignola G, Antonini G, Ugoccioni R (2009) Challenges in measuring operational risk from loss data. J. Oper. Risk. 4(4):3–27.

Koenker R, Hallock KF (2001) Quantile regression. J. Econom. Perspect. 15(4):143–156.

Chen EJ (2009) Metamodels for estimating quantiles of systems with one controllable parameter. Simulation 85(5):307–317.

Dabo-Niang S, Thiam B (2010) Robust quantile estimation and prediction for spatial processes. Statist. Probab. Lett. 80(17):1447–1458.

Chen X, Kim KK (2016) Efficient VaR and CVaR measurement via stochastic kriging. INFORMS J. Comput. 28(4):629–644.

Xie W, Wang B, Zhang Q (2018) Metamodel-assisted risk analysis for stochastic simulation with input uncertainty. Johnson B, Jain S, Rose O, Rabe R, Skoogh A, Mustafee N, Juan AA, eds. Proc. 2018 Winter Simulation Conf. (IEEE Press, Piscataway, NJ), 1766–1777.

Wang S., Ng S.H., Haskell W.B., 2021. A multilevel simulation optimization approach for quantile functions. Informs Journal of Computing, Articles in advance, 1-17.

Salemi P.L., Song E., Nelson B.L., Staum J., 2019. Gaussian Markov random fields for discrete optimization via simulation: Framework and algorithms. Operations Research, 67(1):250–266.

Zitzler, E., Deb, K., & Thiele, L. (2000). Comparison of multiobjective evolutionary algorithms: Empirical results. Evolutionary computation, 8(2), 173-195.