A probabilistically constrained bi-objective simulation-based stochastic optimization framework for urban transportation problems

Abstract

This study proposes a simulation-based optimization (SO) method to solve stochastic constrained bi-objective urban transportation problems, whose bi-objectives and constraint can only be evaluated based on computationally expensive stochastic traffic simulators. It presents a parametric belief based Gaussian process regression to estimate the expected bi-objective values as well as the quantile of the constraint function value. A computationally efficient approximate knowledge gradient sampling policy is designed for searching the feasible non-dominated solutions. We propose a convenient variance estimation scheme based on a parametric model (VEPM) defined on the feature space, which is particularly helpful in learning the quantile of the constraint for judging feasibility. The proposed SO method achieves satisfactory results for a set of synthetic numerical test problems with the effectiveness of the VEMP scheme validated. We also evaluate the performance of the algorithms for a real-word network-wide traffic signal timing problem which takes the expected relative traffic efficiency and relative equity as two objectives and has a probabilistic constraint on the relative vehicular emissions, considering random perturbation of traffic volumes on the road network. By comparing the performance of various choices of the parametric belief models, we conclude that including only linear and squared terms of the decision variables as the basis functions lead to the best performance with little compromise in computational cost. Compared to the field implemented signal plan, the signal plans found by our algorithm with this belief model achieve an average reduction of 15% and 9% for the expected delay and equity measures, respectively. Besides, almost all these solutions meet the probabilistic constraint for the relative traffic emissions, which is, in contrast, notably violated by the field implemented signal plan. The proposed SO method is promising for solving complex multi-criteria transportation problems with large solution spaces and can be applied to other similar bi-objective probabilistically constrained black-box problems.

Keywords: simulation-based optimization; bi-objectives; probabilistic constraint; traffic network signal timing; knowledge gradient sampling policy.

1 Introduction

1.1 Background and motivation

Decision problems in urban transportation systems can have huge socio-economic impact, which need careful analysis in various levels of resolution and an efficient optimization framework. In general, these systems are complex (e.g., dynamic, stochastic, non-linear) and difficult to model analytically, which precludes the use of traditional optimization methods. A powerful alternative is simulation-enabled approach. Recently, a variety of commercial or open-source traffic simulation tools have been developed, providing us with a more flexible and comprehensive way to capture the intricate interactions between the demand and the supply components of the mobility networks. In current practices, there are widely applied traffic simulators such as microscopic ones (e.g., VISSIM, PARAMICS, SUMO) and macroscopic ones (e.g., VISUM, DTALITE, DYNUST). The former focuses on simulating traffic flow dynamics by detailed modeling of driver behavior, while the latter focuses on flow distributions on larger networks with only aggregate traffic flow model on each link. These microscopic and macroscopic traffic simulators can provide estimates of traffic network performance under various system uncertainties (e.g., random traffic volume inputs, random driving and routing behaviors, etc.). These tools are therefore often used to evaluate and compare a set of predetermined solutions (e.g., traffic facility locations or network design plans) in the context of what-if analysis or sensitivity analysis for better decision-making.

However, when the number of feasible or candidate solutions becomes large, it is impractical or prohibitive to simulate all of them. This is common in urban transportation problems which involve computationally expensive calibrated traffic simulation model and high dimensional decision variables. For example, the network signal timing problem needs to optimize the green timings of all the phases at all the intersections in the network (Osorio and Bierlaire, 2013, Zheng et al. 2019a, 2019b, 2022), leading to a huge number of feasible solutions, which generally grows exponentially in problem dimension (Li et al., 2015). Simple enumeration is even less applicable when simulations are stochastic, as each simulation run only gives a noisy observation of the underlying quality of a solution. For such problems with large number of alternatives and measurement noises, research framework of simulation-based optimization (SO) was proposed, which design algorithms that seek good solutions quickly. SO has been an increasingly hot research topic, see relevant reviews (Tekin and Sabuncuoglu 2004, Fu et al., 2005, Amaran et al. 2016, Rojas-Gonzalez and Van Nieuwenhuyse 2020).

In addition to the abovementioned challenges, urban transportation problems often involve multiple performance measures or criteria (Zheng et al., 2019a, 2019b, 2021, 2022), such as traffic efficiency, safety, environmental impact and equity. Some of these performance measures are the primary objectives to be optimized, such as the total travel time on the network, whereas others have to satisfy some control targets such as the total traffic emissions on the network. Moreover, these performance measures are typically stochastic with very limited prior information about their distributions, making the problem harder. So, there is an urgent need for novel algorithms tackling stochastic constrained multi-objective problems arise in urban transportation systems. To this end, this study will focus on urban transportation problems in the following form:

minx∈X Eξ[g1x,ξ],Eξ[g2x,ξ]          s.t.   Prξ g3x,ξ-τ≤0≥1-ϵ                                (1)

Functions g1x,ξ to g3x,ξ are three scalar-valued stochastic performance measures output from traffic simulator, which depend on the decision vector x and the random parameter vector ξ in the traffic simulator. Suppose x lies in a known feasible set X, and ξ has a known distribution p(ξ) with bounded support Ξ. Depending on what underlying system stochasticity needs to be captured for the problem, ξ can contain uncertain parameters in traffic demand, driving behavior, routing behavior, etc. Eξ and Prξ denote the expectation and the probability with respect to ξ, respectively. The constraint means the probability that g3x,ξ is over the limit τ is at most ϵ∈(0,1), where τ and E are predetermined scalars. Problem (1) is therefore to find the nondominated solutions in terms of their expected g1 and g2 values with their g3 values satisfy the probabilistic constraint.

The main purpose of this study is to propose a novel SO methodology which can tackle problem (1) effectively. We will test the proposed method using both synthetic problems and a real-world traffic network signal timing problem which considers uncertainty and involves large discrete decision space.

1.2 Method overview and our contribution

Generally, there are three ways to address the SO problems: direct-search methods, stochastic gradient methods and metamodel methods (Osorio and Bierlaire 2013). Note functions fg1 to fg3 in problem (1) are all stochastic and costly to evaluate and may require different procedures or components in simulations. Thus, an ideal SO algorithm should make full use of the information given by the simulation observations, i.e., infer useful information globally with a local observation. In this regard, direct-search and stochastic gradient methods are not the best choice, since accurate estimation of search directions or gradients is sample-intensive and only provide local information. Thus, in our algorithm design, we focus on metamodels to approximate the expectations in the objectives and the probability in the constraint of problem (1). The metamodels are iteratively fitted using the simulated observations and provide improved prediction of the underlying performance measures globally to guide the search of the feasible Pareto optimal solutions. In fact, metamodel-guided sampling policies have been proposed extensively in SO literature. For instance, kriging model is used with expected improvement (EI) sampling (Forrester and Keane 2009), radial basis function (RBF) model with scoring criterion search (Müller et al. 2013, 2014), polynomials with trust-region search (Osorio and Bierlaire 2013), Gaussian process regression (GPR) with Bayesian optimization (BO) (e.g., Scott et al. 2011), etc. Naturally, the applicability and the performance of the algorithm rely on what combination of metamodel and sampling policy is used. We propose parametric-GPR-based approximate- knowledge gradient (KG) sampling policy for tackling challenging problem (1), which is the main contribution of this study.

BO is a powerful SO framework which starts with a prior belief (estimates of the function values and the uncertainties) on the solutions, selects the new sample which maximizes the value of the measurement, and then uses the new observation to update the posterior belief. The value of measurement is called the acquisition function defined by the sampling policy. In particular, the KG policy defines acquisition function as the expected improvement in the best posterior mean (known as the KG-factor). KG policy has theoretical convergence guarantee and performs well in practice (e.g., Negoescu et al. 2011, Tan and Gao 2016, Wu and Frazier 2016). While, GPR is commonly used in BO due to neat posterior update of the Gaussian process. Many popular metamodels have similar essence as GPR, such as various kriging models that estimate spatial correlations (Forrester and Keane 2009), and the Gaussian Markov random fields which restrict GPR to have certain dependence structure among the solutions (Semelhago et al. 2021). Advantages of the GPR-based KG policy include: 1) convenient KG-factor computation with explicit modeling of possible correlations among solutions (Frazier 2009); 2) flexibility in parameterizing the prior as linear combinations of any basis functions (or features) of the solution (Powell and Ryzhov 2009); and 3) available techniques for accelerating the computation of the sampling decision when solution space is large (Ryzhov and Powell 2009, Tan and Gao 2016). We design parametric GPR models for fg1 to fg3 with problem-specific feature vector of the solution, which makes computation affordable by maintaining posteriors only on model coefficients and the sampled solutions rather than on all the solutions. In addition, we extend the GPR based KG policy to multiobjective setting by combining the posterior distribution and the deterministic multiobjective solvers in generating candidate samples for the next iteration.

Our second contribution is designing a novel updating scheme for learning unknown and heterogeneous variances of the solutions. In urban transportation problems, the variance of the same performance measure can vary notably for different solutions, as observed, for example, in Osorio and Bierlaire (2013), Tan and Gao (2016) and Chen et al. (2019). This necessitates effective learning of the variances especially considering the fact that most solutions will not be sampled under limited budget. In our approach, we divide the feature space of the solutions into grids, then we estimate the variance of any unsampled solution x by the average of those of the sampled solutions whose features lies in the same grid as x. This grid-based learning approach is similar to that proposed by Salemi et al. (2019), but they defined grids directly on the solution space, which can be costly to manage when problem dimension grows. Our variance estimation approach enables cheap approximation of the KG-factor, which is one research focus of the KG policy (Qu 2015, Zhang 2017) since exact Bayesian inferences is impossible with single observation under a multivariate Gaussian prior. Also note that our approach for learning under measurement noises is quite different with those proposed for cases with relatively small number of alternatives, e.g., sample average approximation (Kim et al. 2015), simulation allocation rule such as OCBA (Li et al. 2015), etc., where each alternative is sampled for multiple times and variances are typically known.

Finally, this study enriches the BO literature by addressing both bi-objectives and constraint which are all noisy. Some studies looked at multi-objective BO problems with simple constraints (Galuzio et al., 2020) or constraints that can be expensive to evaluate (Feliot et al. 2017). Although these studies proposed interesting sampling policies that were shown effective, they only considered deterministic objectives and constraints. On the other hand, other studies have investigated BO problems with noisy constraint and single objective (Gelbart et al. 2014, Gramacy 2016, Letham et al., 2019). They applied a GP prior directly on all the solutions and implicitly assumed known noise distributions, which are not applicable to problems with large solution space and unknown variance. In contrast, our parametric GPR model and the variance updating scheme enable learning under noises for high-dimensional problems. Furthermore, these studies adopted the EI policy and multiplied the acquisition function for the unconstrained case by the probability of feasibility conditional on the posterior, leading to cumbersome computation. However, we do not add this complexity since unlike the EI policy, the KG policy can choose an unsampled decision for final implementation. So, our approach is more tractable and can have faster fitting of the proposed parametric GPR model by allowing broader exploration, thus identify promising solutions faster. Note that the probabilistic constraint in problem (1) is equivalent to restricting the (1-ϵ)-quantile of fg3 to be at least τ. Some related study also used Gaussian process models in optimizing quantiles (Wang et al. 2022). Based on similar Gaussian assumption, we can conveniently predict feasibility using the mean and variance estimates of fg3. We demonstrate the effectiveness of our approach using both test problems and a real-world urban transportation problem.

The remainder of the paper is organized as follows. We present a literature review of the related SO methods in Section 2 including the KG sampling policy. A network-wide signal timing problem under uncertainties (NSTPU) is formulated in Section 3 as an application of model (1). In Section 4, we design parametric GPR and KG sampling policy for NSTPU. We apply the proposed method to some synthetic numerical test problems and a real-world signal timing problem and discuss the results in Section 5. Finally, some important findings are drawn in Section 6.

2 Literature review

In this section, we will first review the metamodel-based stochastic SO methods that can address computationally expensive simulation-based problems, whose objectives are formulated as expectations with respect to some stochastic parameters involved in the simulation. We will review those SO methods for problems that include single/multiple objectives and are with/without nontrivial constraint (i.e., simulation-based constraint or probabilistic constraint). One notable stream of the SO literature adopts the BO approach which maintains some probabilistic beliefs of the underlying objective values over the solution space, based on which different sampling policies (a.k.a. infill strategy) are designed. The BO approach has been successfully applied to problems in various fields. In particular, we will adapt the GP-based KG sampling policy to our stochastic bi-objective problem with probabilistic constraint. To this end, we will also briefly review relevant studies on KG policy.

2.1 Stochastic SO methods

For stochastic SO, the expectations of simulated performance measures are often considered to be optimized and the stochastic noises in response need to be carefully considered in surrogate model and sampling policy. Ankenman et al. (2010) proposed stochastic kriging, and Deng et al. (2012) and Ng and Yin (2012) developed Bayesian kriging, to account for the heteroscedastic simulation noises and the extrinsic uncertainty about the unknown response surface. The difference between them lies in the fact that the model parameter uncertainty is considered in the latter but not in the former. Chen et al. (2019) proposed Bayesian stochastic kriging, which explicitly accounts for the parameter uncertainty in the regression and prediction and relaxes specific assumptions on the prior probability distributions of model parameters that exist in Bayesian kriging. Chen et al. (2017) extended the cokriging model, which was originally developed by Kennedy and O’Hagan (2000) to model deterministic multifidelity problems, to stochastic simulation metamodeling for expectations, resulting in stochastic cokriging model. Moreover, Huang et al. (2006) proposed sequential kriging optimization (SKO) with the nugget effect GP as the surrogate model and an augmented EI as the infill strategy for problems homoscedastic noises. For responses with heterogeneous noises, Picheny et al. (2013) put forward an extension of EI based on the ordinary kriging model (named expected quantile improvement (EQI)) to consider the known noise levels at both the already observed design points and the future candidates. Quan et al. (2013) and Pedrielli et al. (2020) proposed the two-stage sequential optimization (TSSO) and the extended TSSO (eTSSO), respectively, in which the stochastic kriging is applied as the surrogate model, the EI criterion as the infill strategy and the optimal computing budget allocation (OCBA) (Chen et al., 2000) technique to decide the number of replications at design points. Besides, Salemi et al. (2019) used a Gaussian Markov random field (GMRF) to learn the response curve of an objective function and designed a complete expected improvement (CEI) criterion considering the uncertainty in stochastic simulation, which are particularly well suited to solve large scale discrete SO problems. Later, Semelhago et al. (2020) proposed a rapid Gaussian Markov Improvement Algorithm (rGMIA), which combines infrequent evaluations of a full conditional distribution for global inference, with rapid learning on a smaller subset of promising solutions to dramatically reduce the computational costs for significantly larger problems. Fan and Hu (2018) presented an adaptive search algorithm called surrogate-based promising area search (SPAS), which employs a shrinking ball method to estimate the solution performance and a surrogate model fitted iteratively with the performance estimates to guide the establishment of a promising area for sampling in the next iteration.

Regarding stochastic SO with multiple objectives, Forrester and Keane (2009) introduced regressing kriging to deal with homoscedastic noisy data and multiobjective EI as the infill strategy, which were also applied to solve the optimal time-varying pricing for toll roads under multiple objectives (He et al., 2016). Based on a bi-objective derivative-free trust region (BOTR) algorithm proposed by Ryu and Kim (2014), Zheng et al. (2019a) put forward a bi-objective stochastic simulation-based optimization (BOSSO) method, which introduces regressing kriging models to smooth the simulation noises, an adaptive selector to determine samples in the local trust-region to be evaluated by simulation, and a decision maker’s non-interactive role to impact the search direction and trust-region. The BOSSO method has been successfully applied in the network-wide traffic signal timing problem and validated to outperform other counterpart algorithms. Later, Zheng and Li (2022) extended the SPAS algorithm to bi-objective setting and generated a biobjective surrogate-based promising area search (BOSPAS) algorithm, which was also successfully applied to an arterial signal control optimization problem.

Previous single- or multi-objective stochastic SO algorithms are usually applicable to stochastic simulation-based problems with bound constraints (also called box constraints) and so the surrogate models are only used to approximate the expensive objective functions. For addressing (single- or multi-objective) SO problems with expensive constraints, some relevant works are introduced as follows. Regis (2011) presented Constrained Local Metric Stochastic RBF (ConstrLMSRBF) algorithm to handle problems with expensive black-box objectives and inequality constraints, which is an extended version of Local Metric Stochastic Radial Basis Function (LMSRBF) algorithm (Regis and Shoemaker, 2007) that is limited to address the analytical inequality constraints. In ConstrLMSRBF, RBF is applied as the surrogate models for the objective and all constraints, and the scoring criterion according to the predicted objective value and minimum distance from previously evaluated points is used to select the promising sample. Müller et al. (2013) introduced a surrogate optimization-mixed integer (SO-MI) algorithm to address expensive mixed-integer black-box global optimization problems that may have expensive constraints. In SO-MI, RBF is used as the surrogate model for the penalty augmented objective function (i.e., the objective function with a penalty for violating the constraints) and the scoring criterion is also employed to select the promising samples. Later, Müller et al. (2014) proposed a surrogate optimization-integer (SO-I) algorithm (a variant of SO-MI) to solve purely integer optimization problems that have expensive black-box objective functions and constraints. Müller and Woodbury (2017) put forward a global optimization with surrogate approximation of constraints (GOSAC) algorithm to address problems with expensive black-box constraints, cheap objective functions and continuous, integer or mixed-integer variables. For a given budget of expensive constraint evaluations, GOSAC finds better feasible solutions more efficiently compared with other algorithms. Feliot et al. (2017) proposed a Bayesian Multi-Objective Optimization (BMOO) method, which uses the Gaussian process as a prior model for the expensive-to-evaluate functions and an expected hyper-volume improvement sampling criterion that handles objectives and constraints in a unified way. The BMOO method is applicable to address the problem of derivative-free (single- or multi-objective) optimization subject to computationally expensive objectives and constraints under limited number of evaluations. However, the black-box constraints of above SO problems are usually evaluated by expensive deterministic simulations, and do not consider their stochastic performance.

As is known, the mean of the response is a widely used performance measure in a stochastic system, while it is not able to evaluate the possible variability or describe the entire distribution adequately. In this condition, the quantile has become increasingly popular and of great interest in many fields to provide more thorough profiles of the response distribution (Longerstaey and Martin, 1996; Wipplinger, 2007; Cope et al., 2009). In risk management, quantile, also termed value at risk, is one of the primary risk measures to quantify and interpret the risk that one stochastic system may face. So, the surrogate models for quantiles have been developed extensively, e.g., quantile regression (QR) model, stochastic kriging model, and stochastic cokriging model (Koenker and Hallock, 2001; Chen, 2009; Dabo-Niang and Thiam, 2010; Chen and Kim 2016; Xie et al. 2018; Wang et al., 2022), based on which some optimization algorithms can be integrated for quantile optimization. Moreover, as aforementioned, the stochasticity inherent in the simulation evaluation tool that commonly exists in various engineering fields would result in stochastic evaluation values of the constraint functions. Therefore, it is necessary and more challenging to account for the stochastic performance about the constraints. In this study, as formulated by the model (1), besides of using the expectations on two stochastic performance measures as bi-objectives, we set a probabilistic constraint about another performance measure evaluated by stochastic simulation, which can be treated as the inverse solution of the corresponding quantile function. That is, the probabilistic constraint is a way to control the risk that the stochastic traffic system may face in terms of this performance measure. Some recent studies have proposed BO methods in dealing with single-objective probabilistically constrained problems (Gelbart et al. 2014, Gramacy 2016, Letham et al., 2019). These studies attached GP priors directly on the objective and constraint of each solution and proposed EI-based sampling policies, which are not applicable to problems that involve large number of feasible decisions and multiple objectives. In contrast, our study will build a new BO approach which aims at efficient learning of the mean performances of the two objective functions as well as the quantile of the constraint function, and the key challenge is that these three functions are all defined on a high-dimensional discrete decision space.

2.2 The knowledge gradient (KG) sampling policy

Note that some of the aforementioned SO methods (mainly for the single-objective problems) are good examples of the BO approach (e.g., Salemi et al. 2019, Letham et al., 2019). As noted earlier, the BO approach has two fundamental components: the belief model describing subjective probability distribution over the solutions as well as the sampling policy based on the acquisition function defined for the solutions. While the GP is the most-commonly adopted belief model in BO, the options for sampling policies are more diversified. In particular, many studies adopt the KG policy for the sampling decisions in BO, whose main advantages are mentioned in Section 1. The KG policy chooses the next solution (or alternative) to be sampled which has the largest KG-factor, i.e., the expected improvement in the best posterior mean conditional on all the previous samples. Such a methodology was shown to be effective and empirically efficient for many SO applications compared to many other approaches. The KG policy was firstly introduced in Gupta and Miescke (1996) for the independent belief case, where the unknown mean of the performance of each solution has an independent prior distribution. Frazier (2008) proved serval key properties of the KG policy such as asymptotic optimality and Frazier (2009) derived a more general KG policy for correlated beliefs, where the prior mean vector follows a multi-variate Gaussian distribution. Parallel-KG policy that samples alternatives in batches was also developed for the independent prior case (Wu and Frazier 2016), which was shown to outperform other batch BO algorithms.

KG policies that incorporate different belief models were further developed, which either learn the parameters of a structured functional form assumed for the underlying mean (e.g., Negoescu et al. 2011), or learn the discrete distributions over a finite set of candidate functionals with known parameters (Chen et al. 2015). For problems with large number of alternatives, KG policies with parameterized beliefs enable affordable computation by maintaining beliefs only on those hyper-parameters or features of the decision vector rather than on each alternative. For example, linear-belief based KG policies were effectively applied to the drug discovery problem (Negoescu 2011) and the traffic network tolling problem (Tan and Gao, 2016). Approximation schemes of the KG policy have also been proposed to accelerate the algorithm by computing the KG-factor only for a subset of altevrnatives at each iteration. One intuitive and effective approach in choosing the subset is by Monte-Carlo (MC) sampling based on the posterior distributions of the mean performance (Ryzhov and Powell 2009) or of the hyper-parameters in the belief model (Tan and Gao, 2016). KG policies have also been extended to continuous solution space by various approximation strategies (e.g., Scott et al. 2011, Wu and Frazier 2017, Balandat,2020, Pearce et al, 2020) for scalability and efficiency considerations, interested reader can refer to a recent brief review by Ungredda, et al. (2022).

However, those methods found in most KG literature are not readily applicable to the type of problems this study focuses on. Firstly, probabilistic constraints for simulation-based quantities have not been addressed in previous studies on KG policy, only some studies considered explicit simple deterministic (e.g., linear) constraints upon the solutions (Wang et al. 2023). Secondly, most of these studies only focused on single performance measure. Although some literature considered multi-objective KG problem for the online multi-armed bandit setting, each performance measure was assumed independent across different alternatives (Yahyaa 2014). Thirdly, previous studies on the KG policy either assumed a known variance of the performance measure (e.g., Gupta and Miescke 1996, Frazier 2008, Frazier 2009), or proposed local updating equation for the variance estimate for each sampled alternative (e.g., Tan and Gao 2016, Wang et al. 2022), or designed approximate formulae for updating the posterior variance vector of all the alternatives under the multi-variate Gaussian framework (e.g., Qu 2015, Zhang 2016). While the local updating approach neglects the possible similarities in variances of two close solutions, the approximate global posterior updating formula cannot be easily adapted to the setting with parameterized beliefs (thus is not applicable to problems with large solution space). Note that this unknown variance would make it more difficult to deal with the probability constraint as mentioned in the first point.

We will design a KG-based SO approach with parameterized GP belief and unknown variances that is applicable to problems with more than one performance measures as well as a probability constraint on another stochastic performance measure. As explained earlier, such a problem setting is commonly seen in urban transportation optimization problems and makes those problems challenging to solve, thus a reliable and efficient SO-based solution approach is highly needed. One contribution that may be of separate interest is that we proposed an updating scheme of variance estimates which not only accounts for possible similarities between the different solutions and can be easily implemented for the cases with parameterized beliefs as well. Such an updating scheme is shown effective especially when a probabilistic constraint is considered, in which case one wants to estimate the distribution of the constrained quantity as accurate as possible under limited simulation budget. Although the KG-based SO method proposed in this study is designed for discrete decision problems due to the nature of the signal timing problem, it can also be generalized to continuous or mixed-integer decision problems by extending the techniques used in the literature mentioned earlier.

3 Network-wide Signal Timing Problem under Uncertainties (NSTPU)

Urban signalized intersections are not only the bottleneck of traffic efficiency and dense area of vehicular emissions, but also influence the equity of right-of-way in the urban road network. In this condition, traffic efficiency is set as an evaluation indicator rather than the only indicator, when designing the traffic control system. Specifically, the traffic equity has been considered in various traffic control problems, such as temporal and spatial inequities of ramp metering control (Levinson and Zhang 2006), aggregated and non-aggregated equity indices of corridor control (Ma and Zhang 2008), spatial equity index of dynamic ramp control (Meng and Khoo 2010), travel time-based Atkinson index of network-wide traffic signal timing plan (Zheng et al. 2019a). Meanwhile, traffic signal control plan substantially affects the driving modes of vehicles which would result in various intensities of vehicular emissions. More specifically, vehicular emissions tend to decrease when the vehicle is accelerating, cruising, decelerating and idling (Unal et al. 2003), and increase considerably if vehicle stops are introduced (Rakha and Ding 2003). Thus, apart from traffic efficiency, vehicular emissions on signalized traffic networks are greatly influenced by the traffic signal control plan. In this condition, traffic signal control optimization provides an alternative way to mitigate urban vehicular emissions. For example, Stevanovic et al. (2009) applied network-wide traffic signal control SO to reduce total delay, fuel consumption and CO2 emissions. Osorio and Nanduri (2015a, 2015b) applied a combined surrogate model of functional component and physical component in the regional traffic signal timing SO to reduce a weighted index of average travel time and high-resolution vehicular emissions. Zhang et al. (2013) minimized total delay and mean excess exposure simultaneously by using a bi-objective GA to solve a signal timing simulation-based optimization model. Zheng et al. (2019b) used the BOSSO method to solve a network-wide signal timing optimization problem whose objectives are total delay and a linear combination of four pollutants’ emissions.

Due to the conflicting nature between traffic efficiency and equity (Kotsialos and Papageorgiou, 2004; Zhang and Levinson, 2004; Kesten et al., 2013; Zheng et al., 2019a), we take traffic efficiency and equity as bi-objectives in this study. Meanwhile, we set vehicular emissions as another evaluation measure subject to probabilistic constraint, that is, vehicular emissions must be no higher than a certain threshold value with a prespecified high probability. The uncertainty we consider in this study is the relative perturbation on the calibrated nominal traffic volume inputs to the network, and we denote this random perturbation level by ξ, representing the ratio of the traffic volume to its nominal value. We assume ξ follows a probability density function pξ with positive support. Let x denote an integer valued decision vector that specifies a network-wide signal timing plan. Then, the network-wide signal timing problem under uncertainties (NSTPU) is formulated as an example of model (1) by letting g1=D, g2=A, g3=E and τ=τE and the calculations of D, A and E refer to Appendix A:

minx∈X Eξ[Dx,ξ],Eξ[Ax,ξ]                         s.t.    Prξ⁡[ Ex,ξ-τE≤0]≥1-ϵ                            (2)

where X:= x∈Zn:xl≤x≤xu  is the feasible solution space, xl∈Zn and xu∈Zn are its lower and upper bounds, respectively, n is the dimension of x that depends on the operation needs of the intersections and the scale of the urban road network. Eξ[Dx,ξ] and Eξ[Ax,ξ] denote the expected values of the traffic efficiency measure and equity measure, respectively, with respect to the random perturbation level ξ. τE is the maximum acceptable level of vehicular emissions which cannot be violated with probability higher than ϵ (e.g., ϵ=0.05).

Since x is the set of green signal timings (usually in tens of seconds) of all signal phases (typically 2 to 6 phases for a typical signalized intersection) in the urban road network, the model (2) has to deal with large discrete solution space. Let K=|X| be the total number of feasible signal timing plans, then K is a very large number. Another difficulty of solving problem (2) lies in the fact that functions Dx,ξ, Ax,ξ, and Ex,ξ are unknown and in general so complicated that can only be estimated through network traffic simulations. In the next section we propose a simulation-based optimization approach for solving problem (2) using Gaussian process regression with knowledge gradient sampling policy.

4 Gaussian process regression-based solution approach for NSTPU

4.1 Gaussian process approximation for NSTPU

Under random traffic volume perturbation, the evaluation indices are also random, whose exact distributions are in general unknown a priori due to the complicated underlying traffic network system and limited historical data. We denote the random variable Dx as the relative total delay output for decision vector x under uncertain traffic volume perturbation, i.e., Dx is variable Dx,ξ with ξ~pξ. Similarly, we define Ax and Ex as the relative traffic equity and relative vehicular emissions, respectively.

For each x∈X, we assume that Dx, Ax and Ex follow Gaussian distributions:

Dx ~ NμxD,λxD,                                (3a)

Ax ~ NμxA,λxA,                                (3b)

Ex ~ NμxE,λxE                                (3c)

where μxi and λxi, i∈{D, A, E} are the unknown mean and variance of the corresponding evaluation index i for solution x. The key purpose of the simulation is to learn these quantities with the goal of solving optimization problem (2).

Suppose we are allowed to conduct a total of N simulations, each simulation chooses a solution x∈X for evaluation, the evaluation is noisy with an independent random realization of the uncertain parameter ξ drawn from pξ. We denote xn the solution selected for the n-th simulation, and yn=(yD,n,yA,n,yE,n) the corresponding evaluation indices computed based on the simulation output. In other words, each simulation gives us a noisy sample of each of the three evaluation indices of the chosen solution, based on which we update our estimates for the underlying mean and variance of each evaluation index. Note that since there is noise in measuring each sample, a specific solution may be sampled for multiple times, i.e., xn=xn' for some 1≤n≠n'≤N. In the rest of the paper, we will use the notation pn, Pn, En, and Varn, respectively, to represent posterior probability distribution function, probabilities, expectations, and covariances conditional on all the sampled solutions up to stage n, Xn={x1,…, xn}, and observations up to stage n, Yn={y1,…, yn}, for n=1, 2,…,N.

For convenience, we define the mean vector μi={μxi,  x∈X} for each evaluation index i∈{D, A, E}. We take a Bayesian viewpoint for the estimation process of the mean vector μi for each i∈{D, A, E}. We start with some prior distribution of μi, p0(μi), then we update the posterior distribution pn(μi) based on Xn and Yn for n=1, 2,…,N one after another. For computational tractability, for each i∈{D, A, E} we choose p0(μi) to be a K-dimensional normal distribution with mean θi,0 and covariance matrix Σi,0. Then based on the property of normal distribution, we know that pn(μi) is also a multivariate normal distribution for n=1, 2,…,N. Specifically, for n=1, 2,…,N, given θi,n-1,Σi,n-1, xn and yn, the posterior distribution pn(μi) is also a K-dimensional normal distribution whose mean θi,n and covariance Σi,n can be computed as

θi,n=θi,n-1+yi,n-θxni,n-1Σxnxni,n+λxni,nΣi,n-1exn,                        (4a)

Σi,n=Σi,n-1-Σi,n-1exnexnTΣi,n-1Σxnxni,n+λxni,n,                          (4b)

where exn is a unit K-dimensional column vector with the entry corresponds to solution xn being one; θxni,n-1is the entry of vector θi,n-1 that corresponds to solution xn; Σxnxni,n denotes the diagonal entry of matrix Σi,n-1 that corresponds to solution xn; λxni,n denotes our best estimate of the sample variance of evaluation index i of solution xn at stage n. In section 3.2, we will propose a parametric sequential estimation procedure for these sample variances.

Since the true underlying distributions of the evaluation indices Dx, Ax and Ex are not known a priori, problem (2) is itself unknown exactly. Thus we replace the true expectations E[Dx] and E[Ax] in (2) by their stage-N posterior means θxD,N and θxA,N, respectively, when the simulation budget is exhausted. We also replace the true probability P(Ex≤τE) by the posterior probability PN(Ex≤τE). Note that under the Guanssian assumption, we have

PNEx≤τE=PEx-θxE,NλxE,N1/2≤τE-θxE,NλxE,N1/2=ΦτE-θxE,NλxE,N1/2,

thus PNEx≤τE≥1-ϵ is satisfied by restricting

θxE,N≤ τE-Φ-11-ϵλxE,N1/2                      (5)

where Φ is the cumulative density function of the standard normal distribution, so Φ-1 is the quantile function of the standard normal distribution. Note that similar as in (4), we use the stage-N estimate of the sample variance, λxE,N, in inequality (5).

Therefore, we can rewrite problem (2) based on Gaussian process approximation as

minx∈X θxD,N,θxA,N                            s.t.  θxE,N≤ τE-Φ-11-ϵλxE,N1/2                       (6)

4.2 Parametric belief on model parameters

4.2.1 Parametric belief on the mean vector

It is generally not convenient to work with a high dimensional Gaussian distribution in the learning algorithm for sampling decisions (e.g., the one that we will discuss in Section 4.3). For NSTPU, we note that the number of signal timing plan (i.e., K) can be very large. For example, consider a traffic network with only three intersections, each has only two phases whose green time ranges from 21 to 40 seconds, then we have K=(202)3=206, a number that is already prohibitive for normal computation.

To this end, we seek proper parametric belief model to make the learning problem feasible. Specifically, we assume that the underlying mean of any solution x is a linear combination of a set of basis functions (or features) of x plus a deviation term, i.e., we have

μxi=ζxi+j=1Miβjifjix,  i=D, A, E,                     (7a)

where fji is the j-th deterministic basis function for evaluation index i, and Mi is the number of basis functions used for evaluation index i. The scalar βji represents the marginal contribution of the basis function value fjix to the mean of solution x’s i-th evaluation indices, μxi. ζxi is the deviation term. Thus, (7a) assumes that the underlying mean of each evaluation index for a certain solution can be written as a linear combination of the basis functions of the solution plus a specific deviation term. Define column vectors βi=(β1i,…,βMii)T and ζi=(ζxi,x∈X), and let Fi be a K-by-Mi matrix such that the j-th entry of its row corresponding to solution x is fjix. We also denote fxi as the row of matrix Fi that corresponds to solution x, i.e., fxi=(f1ix,…,fMiix). Then (7a) can be expressed in matrix form

μi=Fiβi+ζi,  i=D, A, E.                        (7b)

Now for i=D, A, E, we use a Mi-dimensional Gaussian distribution for the prior of the coefficient vector βi, p0βi, with mean bi,0 and covariance matrix Bi,0, and we use a K-dimensional Gaussian distribution for the prior of vector ζi, p0ζi, with zero mean and covariance matrix zi,0I, where zi,0>0 is a scalar and I is a K-by-K identity matrix, i.e., our prior belief is that the deviation terms of different solutions are independent and they have uniform prior uncertainty. Then the mean and the covariance matrix of the prior distribution of μi can be written as

θi,0=Fibi,0,                                     (8a)

Σi,0=FiBi,0(Fi)T+zi,0I,                          (8b)

Now we define for each i=D, A, E a stage-n coefficient vector βi,n, which contains vector βi as well as the deviation terms ζxi for all the distinct solutions x sampled up to stage n. Following this definition, we have βi,0=βi as no sample has been taken at stage 0, so the prior distribution of βi,0 is also Nbi,0,Bi,0. Then due to the property of multivariate Gaussian distribution, it can be verified that for n=1, 2,…,N, the posterior distribution (conditional on Xn, Yn and λxni,n-1), pnβi,n, is also a multivariate Gaussian distribution, whose mean bi,n and covariance Bi,n can be determined by

bi,n=bi,n-1+yi,n-fxni,nbi,n-1fxni,nBi,n-1(fxni,n)T+λxni,n-1Bi,n-1(fxni,n)T,                  (9a)

Bi,n=Bi,n-1-Bi,n-1(fxni,n)Tfxni,nBi,n-1fxni,nBi,n-1(fxni,n)T+λxni,n-1,                             (9b)

where the column vector bi,n-1 and the matrix Bi,n-1 are defined in the following way: if xn has been sampled before stage n (i.e., the deviation term ζxni has already been included in vector bi,n-1),   then bi,n-1=bi,n-1 and Bi,n-1=Bi,n-1; otherwise, bi,n-1 is formed by appending a zero to bi,n-1, and Bi,n-1 is formed by appending a zero row and a zero column to Bi,n-1 and set the new diagonal entry as zi,0. Namely, the following revisions are made if xn is first sampled at stage n:

bi,n-1=bi,n-10,     Bi,n-1=   Bi,n-10T0zi,0.

fxni,n is a row vector that has a commensurate dimension with βi,n and has fjixn as the j-th entry for j=1,…,Mi and has zeros elsewhere except that a 1 at the entry whose index is the same with that of the entry in βi,n corresponding to the deviation term ζxni. For instance, suppose the last entry of βi,n corresponds to ζxni, then we have

fxni,n = f1ixn⋯fMiixn     0T1.

The updating equation (9) is similar with (4), but now instead of maintaining beliefs on the original mean vector μi, we only have to update the posterior distribution for parameter βi,n, which is typically much shorter than μi even though the size of βi,n generally grows with sampling stage n as new solutions are being sampled, as normally we have N≪K.

4.2.2 Parametric update for the variance estimate

Note that the estimate of the variance at stage n-1, λxni,n-1, is needed in equation (9) for generating the posterior distribution pnβi,n. However, it is possible that the solution to be sampled at stage n, xn, has not yet been sampled or has only been sampled a few times (e.g., less than three) before stage n, leading to a poor estimate λxni,n-1 and thus pnβi,n. In problems such as NSTPU, the sampling budget N is typically much smaller than the total number of solutions K, which makes it challenging to estimate the variance of unsampled solutions. Therefore, we design a variance estimate based on parametric model (VEPM) which captures possible similarities in the variability of two close solutions based on a straightforward set partition scheme. Specifically, we denote the domain of fjix on x∈X by Fji, for each j=1,…,Mi, and partition Fji into a set of mji disjoint subsets, fj,1i,…,fj,mjii, with l=1mjifj,li=Fji. Then there are a total of j=1Mimji different combinations of such sets over all j=1,…,Mi. Given x∈X we let qi(x) be the combination index of the partitions of fjix for j=1,…,Mi, specifically, if fjix lies in the kji-th partition of Fji, we define qix=kMii+j=1Mi-1(kji-1)h=j+1Mikhi. For example, if Mi=3, m1i=m2i=m3i=2, f1ix, f2ix and f3ix, respectively, lie in the second partition, the second partition, and the first partition of their corresponding domains, then we have qix=1+(2-1)×2×1+(2-1)×1+1=5.

For each evaluation index i=D, A, E, and for each combination index w=1,…,j=1Mimji, we start with some initial variance estimate, Λwi,0, and denote Λwi,n as the stage-n variance estimate for all the unsampled solutions with the same combination index w. We consider the following three cases for updating the variance estimates iteratively for stage n=1, 2,…,N.

Case 1: If x is sampled at stage n, i.e., x=xn, then its variance for evaluation index i is estimated by the weighted average of the initial estimate and the squared deviation of each sample from the corresponding posterior means:

λxi,n=s0iΛqxi,0+l=1sxnyi,nxl-θxi,nxl2s0i+ sxn=λxi,n-1s0i+ sxn-1+yi,n-θxi,n2s0i+ sxn,          (10a)

where s0i is the weight for the initial variance estimate of evaluation index i, sxn is the number of samples collected for solution x up to stage n, and nxl is the stage at which the l-th sample of x is collected. The second equality is due to the definition of λxi,n-1 by the first equality and l=1sxnyi,nxl-θxi,nxl2=l=1sxn-1yi,nxl-θxi,nxl2+yi,n-θxi,n2, this iterative expression is computationally more efficient than the original definition in the first equality.

Case 2: If x is unsampled up to stage n but has the same combination index as the solution that is sampled at stage n, i.e., x≠xn, sxn=0 and qix=qixn, then its variance estimate of evaluation index i is set to be the average of the variance estimate of those sampled solutions including xn that have the same combination index as x for evaluation index i:

λxi,n=Λqixi,n=x:qix=qix,sxn>0λxi,nrqixn=Λqixi,n-1+λxni,n-λxni,n-1rqixn=λxi,n-1+λxni,n-λxni,n-1rqixn,      (10b)

where rqixn is the number of solutions that have been sampled by stage n and have combination index qix, The first and the fourth equalities hold by our definition of Λqixi,n, and the third equality is true by considering the two possible cases: 1) If sxnn>1 (i.e., xn has been already sampled before stage n), then rqixn=rqixn-1, so we have Λqixi,n-Λqixi,n-1=(λxni,n-λxni,n-1)/rqixn; and 2) If sxnn=1 (i.e., xn is firstly sampled at stage n), then we have rqxn= rqxn-1+1 and thus

λxi,n=Λqixi,n-1rqixn-1+λxni,nrqxn=Λqixi,n-1rqixn-1+λxni,n-1+λxni,n-λxni,n-1rqxn

=Λqixi,n-1rqixn-1+1+λxni,n-λxni,n-1rqixn-1+1=Λqixi,n-1+λxni,n-λxni,n-1rqixn,

where the second last equality is due to λxni,n-1=Λqixi,n-1 since xn was unsampled till stage n.

Case 3: If x is sampled before stage n but not at stage n or x is unsampled up to stage n and has a different combination index with xn, then its variance estimate is unchanged, i.e., λxi,n=λxi,n-1.

In summary, the variance estimates evolve iteratively for stage n=1, 2,…,N as following:

λxi,n=λxi,n-1s0+ sxn-1+yi,n-θxi,n2s0+ sxn,                      if x=xnλxi,n-1+λxni,n-λxni,n-1rqixn,        if x≠xn, sxn=0,qix=qixnλxi,n-1,                                      otherwise       (10c)

So, after each stage n, we just need to update for each evaluation index i, the variance estimate of xn, λxni, and update Λqixni as the a common variance estimate for all the unsampled solutions which have the same combination index with xn. We will see in Section 4.3 that this variance updating approach also help make our sampling algorithm efficient.

4.2.3 The reformulated decision model for NSTPU based on parametric beliefs

Finally, using the proposed parametric belief models, problem (6) can be reformulated as

minx∈X fxD,NbD,N,fxA,NbA,N                                  s.t.  fxE,NbE,N≤ τE-Φ-1(1-ϵ)λxE,N1/2                      (11)

At the first glance, problem (11) seems nontrivial as there is a solution specific righthand side value in the constraint. However, we note that (10) ensures that the posterior variance estimate λxE,N is common for those x’s that have not been sampled by stage N and have the same combination index for evaluation index i = E. Therefore, we design the following steps in solving (11).

Step-1. We solve a problem by using only information from the posterior mean of the coefficients βji, j=1,…,Mi, for i∈{D, A, E} and applying a common posterior variance estimate ΛwE,N to all solutions t= E, as following:

minx∈X j=1MDbjD,NfjDx,j=1MAbjA,NfjAx                                     s.t.  j=1MEbjE,NfjEx≤τE-Φ-1(1-ϵ)ΛwE,N12                              fjEx∈fj,lwE,∀ w∈1,…,j=1MEmjE,j∈1,…,ME           (12)

where bji,N is the j-th entry of bi,N, i∈{D, A, E}. By our definition of the partition fj,lE, l=1,…,mjE for the domain FjE, j=1,…,ME, one natural way to solve (12) is by solving a set of j=1MEmjE subproblems (each for a distinct combination index w=qEx) and comparing their results to derive the Pareto optimal solutions of (12). Each of these subproblems can be solved by standard multi-objective solver such as NSGA-II.

Step-2. For each distinct sampled solution x in {x1,…,xN}, we evaluate its objective values fxD,NbD,N and  fxA,NbA,Nand check its feasibility fxE,NbE,N≤ τE-Φ-1(1-ϵ)λxE,N1/2.

Step-3. We compare the objective values of the sampled feasible solutions obtained by Step-2 to those of the Pareto optimal solutions of (12) (with those sampled ones deleted if any) obtained by Step-1 to form the true Pareto optimal solutions for the original problem (11).

4.3 The Knowledge Gradient (KG) sampling policy

In this subsection, we will tailor the GP-based knowledge gradient sampling policy to problem (6) considering the parametric model described in 4.2. The entire solution procedure is summarized in Algorithm 1 in the Appendix B, and will be detailed as follows.

At each sampling stage n=1,…,N, we go through the following steps (K1 and K2 are two predetermined numbers):

1) Generate a set of candidate solutions denoted by Sn, which is the union of two separately generated sets, i.e., Sn= S1n∪S2n. S1n is simply a random selection of K1 solutions in X. While S2n contains the solutions that are close to the optimizers (i.e., nondominant solutions) of the deterministic bi-objective problem (11) for stage n-1, i.e., problem (11) with subscript N replaced by n-1. We define Pn-1 as the Pareto optimal solution set of this problem, i.e.,

Pn-1=x∈X:∄ x∈X s.t. fxE,n-1bE,n-1≤ τE-Φ-11-ϵλxE,n-112,  fxibi,n-1<fxibi,n-1 ∀i∈D, A  (13)

if Pn-1 contains more than K2 solutions, we simply choose S2n= Pn-1; otherwise we generate another K2-|Pn-1| new solutions each by firstly choosing an x uniformly at random from Pn-1 and then sampling a solution from Gaussian distribution Nx,Cn and rounding this perturbed solution to the nearest integer solution in X. This approach of sampling set generation is given by Algorithm 2 in the Appendix B.

2) Compute the Knowledge Gradient (KG) factor associated with evaluation indices i∈{D, A} for each candidate solution in Sn, the KG factor for evaluation index i of solution x before sampling at stage n, denoted by vKG,i, n(x), is defined as the expected improvement in the best posterior mean of evaluation index i among all the candidate solutions if x∈Sn is sampled given the historical sampling decisions Xn-1 and observations Yn-1 (i∈{D, A}):

vKG,i,nx=minx∈Snθxi,n-1-En-1minx∈Snθxi,n|xn=x

=minx∈Snfxibi,n-1-En-1minx∈Snfxibi,n|xn=x.                   (14)

Algorithms 3 - 4 in the Appendix B detail the computation of the KG factor vKG,i, n(x), the key of which is to utilize the conditional predictive distribution pn-1θi,n|xn=x, which has mean and covariance as (these quantities can be derived from (9)):

En-1θi,n|xn=x=θi,n-1=fxi,n-1bi,n-1,                   (15a)

Varn-1θi,n|xn=x=σi,n-1(x)(σi,n-1(x))T,               (15b)

where the column vector σi,n-1(x), as a function of x, can be expressed as

σi,n-1x=Fi,n-1xBi,n-1x(fxi,n-1)T,                     (15c)

where Fi,n-1x=Fi,n-1 and Bi,n-1x=Bi,n-1 if x has been sampled before stage n; otherwise, Fi,n-1x is formed by appending a column to the right of Fi,n-1 with the entry corresponds to solution x being one and others zero, and Bi,n-1 is formed by appending a row and a column to Bi,n-1, with the new diagonal entry being zi,0 and all the other new entries zero, as shown below:

Fi,n-1  Fi,n-1   ex ,   Bi,n-1  Bi,n-1   00        zi,0 ,

where ex is a unit column vector whose entry corresponding to solution x is one.

3) Pick the sampling decision xn as any solution in Sn with nondominant KG factors for two evaluation indices i=D, A, i.e., xn∈PKG,n, where PKG,n is defined in (16). Note that this way of selecting the sample given the KG values is essentially the ‘Pareto-KG’ approach (Yahyaa 2014).

PKG,n=x∈Sn:∄ x∈Sn s.t. vKG,i,nx>vKG,i,nx ∀i∈{D, A}.        (16)

In practice, we can also use certain rule to select the solution in PKG,n to be sampled. For example, we can pick the one with the largest crowding distance (i.e., the normalized sum of its distances to its adjacent solutions in PKG,n with respect to each objective function).

4) Simulate the signal plan xn and observe the traffic flow output, then compute the evaluation indices yn=(yD,n,yA,n,yE,n). Update posterior beliefs on model parameters βi,n by (9), and update the variance estimates by (10).

5) If n=N, we solve problem (11) and obtain the Pareto optimal solution set PN, from which an optimizer is selected for final implementation. Similar to the case for selecting the sampling decision, certain rule can be applied for selecting the final implementation plan if |PN|>1, e.g., we can select the solution with the largest crowding distance in PN.

4.4 The choice of the basis functions for NSTPU

In general, the higher the order of the polynomial, the better the model can fit the underlying function, but more terms have to be included in the belief model. Note that the size of the coefficient vector βi grows exponentially in the dimension n of the solution if a full polynomial model is used (i.e., those with all cross-multiplication terms), a higher order polynomial model can result in significantly more computational cost. As noted by (Osorio and Bierlaire 2013), a quadratic polynomial can provide decent approximation for traffic network problems, so we also adopt quadratic polynomials for the belief model in NSTPU. In Section 5, we will compare various quadratic models that involves different number of terms so as to shed lights on the trade-off between computational cost and effectiveness in learning.

5 Numerical experiments

This section aims to validate the performance of GPR-KG proposed in the last section in addressing the bi-objective stochastic discrete optimization problem with a probabilistic constraint. First, the GPR-KG algorithm is tested for three synthetic constrained bi-objective stochastic discrete optimization problems, whose theoretical Pareto optimal solutions are known. Then, it is applied to address a real-field NSTPU problem, whose objectives and constraint are all evaluated by costly stochastic traffic simulation.

5.1 Test problems

By introducing stochastic noises and probabilistic constraint expressions and discretizing decision variables, the revised versions of ZDT1, ZDT2 and ZDT5 (Zitzler et al. 2000) are treated as the benchmark problems, and renamed as RZDT1, RZDT2 and RZDT5, respectively. Specifically, the white noises are added to each objective function and constraint function, respectively, and then bi-objectives and constraint are expressed by their expectation and probabilistic counterparts. The formulations of RZDT1, RZDT2 and RZDT5 can refer to Appendix C, respectively. Obviously, incorporating noises veils the true expected values of the bi-objectives as well as the probability of violating the constraint, which makes it challenging to solve these revised problems.

To solve these three revised problems, the critical parameters of the GPR-KG algorithm are set as: the simulation budget N = 100, the number of pre-samples N0=11, the weight for the initial variance estimate s0i=0.5, i=1, 2, 3, the number of random candidate samples K1=10, and the number of candidate samples generated by solving the current posterior parametric belief model K2=90. Moreover, three priori parametric belief models are built to estimate the underlying mean values of bi-objectives and the constraint function of any decision x. Specifically, for each of the functions gi, i=1, 2, 3, we choose the parametric belief model to be encoded by a linear combination of 2×d basis functions: fjix=xj,j=1, 2,…,d, fd+jix=xj2,j=1,2,…,d. For each basis function, we divide its domain into two equal-sized partitions, and so there are a total of 22d = 4d combinations of the partitions indexed by qx∈{1,2,⋯,4d}. We set the initial estimate of the variance of each partition combination qx∈{1,2,⋯,4d} as a common value Λqxi,0=0.01, i=1, 2, 3.

Numerical results in Fig. 1 illustrate that the bi-objective values of most learned solutions lie on or close to the theoretical Pareto front (c.f., blue triangles), but those for a few other learned solutions (c.f., gray triangles) lie on the left boundary of the bi-objective domain, which seem to deviate from the theoretical Pareto front. This because the posterior parametric belief models can accurately approximate g1 which is composed of the linear term related to x1, and then guide more search on the left-side region (i.e., when x1→0), where g1 for RZDT1, RZDT2 and RZDT5 all are close to its lowest value 0. This indicates the good performance of GPR-KG in addressing the bi-objective stochastic discrete optimization problem with a probabilistic constraint at the cost of limited computational resources. Specifically, in the iterative process, the bi-objective points of iterative solutions usually scatter in the regions close to the theoretical Pareto front, and some of them fortunately locate on it, which helps the parametric belief models to learn the information about theoretical Pareto front. However, there are also some bi-objective points of iterative solutions that are distant from the theoretical Pareto front or even infeasible (c.f., gray asterisks). This generally results from the following two causes. That is, the candidate solutions selected by the KG factor at each iterate might be from the set generated by sampling randomly (i.e., S1n in section 4.3), and so may contain the dominated or infeasible solutions; the early posterior parametric belief models fitted with limited observations might be less accurate, which causes a high probability of not finding the bi-objective points closed to the theoretical Pareto front and not complying with the probabilistic constraint.

(a)                                      (b)

Fig. 1 Bi-objective points of iterative solutions and finally searched Pareto optimal solutions by GPR-KG for (a) RZDT1, (b) RZDT2 and (c) RZDT5. Note: the iterative solutions represent the solutions selected from the set of candidate solutions (i.e., Sn in section 4.3) by the corresponding bi-objective KG factors. The learned solutions represent the finally optimized solutions generated with the posterior parametric belief models and the posterior variance estimations when GPR-KG terminates. The bi-objective points of all solutions in this figure are evaluated by real (or nominal) objective and constraint functions without stochastic noises.

Meanwhile, a reliable performance indicator named inverted generational distance (IGD) (Zitzler et al. 2003) is introduced to quantify the closeness of the generated non-dominated front to a set of reference points and its diversity. It is formulated as IGD=pr∈Pminpo∈Opr-po2/|P|, where P is the set of reference points in the theoretical Pareto front, O is the set of bi-objective points of optimized (or iterative) solutions generated by GPR-KG and |∙| denotes the cardinality of a set. Obviously, the smaller IGD means the better convergence and diversity of the optimized (or iterative) solutions. During the iteration process of GPR-KG, we obtain the iterative non-dominated solutions by the posterior parametric belief models and their posterior variance estimates at each iteration and then compute the corresponding IGD value. Moreover, to mitigate the impact of randomness, we carry out 10 independent runs of GPR-KG for RZDT1, RZDT2 and RZDT5, respectively, so as to calculate the corresponding average IGD values.

Numerical results in Fig. 2 show that the average IGD values decrease rapidly in the early few iterations, then the decline decelerates and finally reaches a very small level. This indicates the good convergence performance of the GPR-KG. Meanwhile, results in Table 1 indicate that the learned Pareto optimal solutions by GPR-KG at the cost of 111 function evaluations cover most or almost all theoretical Pareto optimal solutions, and their final IGD values are 0.055, 0.022 and 0.005 for RZDT1, RZDT2 and RZDT5, respectively. These results further validate the good convergence, diversity and efficiency of GPR-KG in dealing with the bi-objective stochastic discrete optimization problems with probabilistic constraints.

(a)                        (b)                        (c)

Fig. 2 Average IGD over 10 independent runs of GPR-KG for solving (a) RZDT1, (b) RZDT2 and (c) RZDT5.

Table 1 Detailed average results over 10 independent runs of GPR-KG.

| Test problem | # FEs. | # TPOS. | Avg. # LPOS. | Avg. IGD | Avg. Comp. time |
| --- | --- | --- | --- | --- | --- |
| RZDT1 | 111 | 32 | 18.5 | 0.055 | About 50 min |
| RZDT2 | 111 | 32 | 16.3 | 0.103 | About 50 min |
| RZDT5 | 111 | 12 | 11.1 | 0.005 | About 50 min |

Note: # FEs. denotes the number of function evaluations for each run of GPR-KG, and # TPOS. is the number of theoretical Pareto optimal solutions. Avg. # LPOS., Avg. # IGD and Avg. Comp. time represent the average number of learned Pareto optimal solutions, the average IGD value and the average running time over 10 independent runs of GPR-KG, respectively.

Moreover, we also use the iterative non-dominated solutions as the testing set to evaluate the estimation (prediction) errors of the current posterior parametric belief models with respect to bi-objectives and probabilistic constraint during the iteration process. These errors are measured by Root Mean Square Errors (RMSEs) based on the estimated (predicted) and real (nominal) bi-objective and constraint function values. Fig. 3 apparently shows that average RMSEs are relatively high in the early few iterations, and then decrease rapidly to reach a small and stable level. This implies the accuracy of the posterior parametric beliefs and variance estimates improve as the iteration goes on (i.e., the number of accumulative evaluated samples increases). Specifically, after 50 iterations, average RMSE of g1 reaches 0.021, 0.022 and 0.018 for RZDT1, RZDT2 and RZDT5, respectively; that of g2 reaches 0.227, 0.123 and 0.159 for RZDT1, RZDT2 and RZDT5, respectively; and that of g3 reaches 0.009, 0.006 and 0.025 for RZDT1, RZDT2 and RZDT5, respectively. So, the fitted posterior parametric belief models after 50 iterations can already well approximate the two objectives and the probabilistic constraint.

(a)                       (b)                         (c)

Fig. 3 Average estimation (prediction) errors of the current posterior parametric belief models with respect to (a) g1, (b) g2 and (c) g3, over 10 independent runs of the GPR-KG algorithm for solving RZDT1, RZDT2 and RZDT5. Note: g1 and g2 denote the expectations of two objective values (i.e., Eξ(g1) and Eξ(g2)), respectively, and g3 represents the estimation of constraint value under the probability of 1-ε (i.e., Eξc+Φ(1-ϵ)∙σ), where ϵ=0.05, σ=0.05.

In addition, we evaluate by 100 times the stochastic constraint function value (i.e., cx,ξ) of each iterative non-dominated solutions x and regard it as infeasible if cx,ξ>0 more than five times. Numerical results in Fig. 4 illustrate that average number of infeasible solutions in the iterative non-dominated solutions is relatively high in the early few iterations, and then decreases rapidly to zero. This validates the good capability of GPR-KG in dealing with the probabilistic constraint.

(e)                       (f)                     (g)

Fig. 4 Average number of infeasible solutions in the iterative non-dominated solutions over 10 independent runs of the GPR-KG algorithm for solving (a) RZDT1, (b) RZDT2 and (c) RZDT5.

Finally, we verify the effectiveness of the proposed VEPM by comparing the results with and without this variance updating scheme. Recall that our algorithm GPR-KG with VEPM will estimate the variances of those unsampled solutions based on the similarity of their features with those of the sampled ones. While, we restrict the algorithm GPR-KG without VEPM to only update variance estimates for those sampled solutions but no extrapolation to those of the unsampled solutions. Fig. 5(a) shows that for each of the three test problems, the posterior variance estimate of a randomly selected unsampled solution which lies on the theoretical Pareto front approaches the true variance when the VEPM is used, while it stays unchanged when VEPM is not used. As a result, we can see from Fig. 5(b-d) that for all the three test problems, the algorithm with VEPM identifies notably more points on the theoretical Pareto front than the algorithm without VEPM. In these cases, VEPM helps the algorithm overcome the initial over-estimates of the variances and thus improving the conservativeness in finding feasible nondominated solutions.

(a)                                         (b)

(c)                                         (d)

Fig. 5 Performance comparison of GPR-KG with VEPM and GPR-KG without VEPM in terms of (a) variance estimate for points in (b-d) during the iteration process, and the bi-objective points of learned Pareto optimal solutions for (b) RZDT1, (c) RZDT2 and (d) RZDT5. Note: all selected points for variance estimate in (b-d) lie on the theoretical Pareto front and are not in the set of iterative solutions after GPR-KG with VEPM and GPR-KG without VEPM terminate.

5.2 Network-wide signal timing problem

5.2.1 Simulation setup

Signalized intersection plays an important role in the network-wide traffic efficiency, traffic equity and vehicular emissions. Network-wide signal timing optimization considering multi-criteria is also a hot and difficult topic to study. This application employs the microscopic traffic simulator VISSIM as an evaluation tool of network-wide signal timing plan, due to its flexibility in designing any type of traffic element, and capability of capturing the complicated relationship between the signal timing plan and multiple evaluation indices. Meanwhile, the mature COM interface in VISSIM also allows the inter-process communication and data exchange between VISSIM and other compatible computational software (e.g., MATLAB). So, we first construct the SO platform by integrating VISSIM with MATLAB through the COM interface. Then, an urban road network in Changsha, China is modelled by VISSIM as the experimental scenario, which consists of 15 signalized intersections, 5 non-signalized intersections, 29 entries, 24 exits and 31 road links (c.f., Fig. 6). The detailed calibration process and results about the traffic simulation model can refer to Zheng et al. (2019a, 2019b). In the SO platform, given a specific network-wide signal timing plan, the calibrated traffic simulator will output individual vehicular information after completing the traffic simulation run, which can be used to calculate three typical traffic evaluation indices D, A and E.

For this NSTPU problem, the decision vector x consists of the signal timings of 47 signal phases in this regional urban road network (c.f., Table 2). The perturbed network inflow vector is expressed by VI1 ×29∙ξ, where VI1 ×29 is the calibrated network inflow vector of 29 entries in this network (Zheng et al., 2019a, 2019b) and can be treated as the nominal network inflow vector. τE=0.9 and ϵ=0.05 are set for the probabilistic constraint. During the numerical experiment, 100 simulation evaluations for pre-samples are used for initially fitting the parametric belief models with respect to bi-objectives and probabilistic constraint, and 300 simulations are allocated to evaluate the infilled signal control plans in the latter iterations. So, we set N=300 and N0=100. Other parameters of the GPR-KG algorithm are set as follows: weight for initial variance estimate s0=0.5, the number of random candidate samples K1=10, the number of random draws in posterior coefficient vector for generating candidate samples K2=90, and the initial variance estimate Λqxi,0=0.03 for all the distinct combinations of feature partitions qx and all three evaluation indices i=1, 2, 3.

- (b)

Fig. 6 Study site, (a) a regional urban road network in Changsha, China, and (b) its VISSIM model.

Table 2 Signal phases in the regional urban road network

| SI 1 |  |  |  |  |  |  | SI 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SI 3 |  |  |  |  |  |  |  |  | SI 4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ① |  |  | ② |  |  |  | ① |  |  |  | ② |  |  |  | ③ |  |  |  | ④ |  |  | ① |  |  |  | ② |  |  |  |  | ① |  | ② |  |  |
| SI 5 |  |  |  |  |  |  | SI 6 |  |  |  |  |  |  |  |  |  |  |  |  | SI 7 |  |  |  |  |  |  |  | SI 8 |  |  |  |  |  |  |  |
| ① | ② |  |  |  | ③ |  | ① |  |  | ② |  | ③ | ④ |  |  |  | ⑤ |  |  | ① | ② |  |  |  | ③ |  |  | ① | ② |  |  | ③ |  |  | ④ |
| SI 9 |  |  |  |  |  |  |  |  |  |  |  | SI 10 |  |  |  |  |  |  |  | SI 11 |  |  |  |  |  |  | SI 12 |  |  |  |  |  |  |  |  |
| ① |  | ② |  |  |  | ③ |  |  | ④ |  |  | ① |  | ② |  |  |  | ③ |  | ① |  |  | ② |  |  |  | ① |  |  | ② |  |  | ③ |  |  |
| SI 13 |  |  |  |  |  |  |  | SI 14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SI 15 |  |  |  |  |  |  |  |  |  |  |  |
| ① |  |  |  | ② |  |  |  | ① |  |  |  | ② |  |  |  | ③ |  |  |  | ④ |  |  |  | ① |  | ② |  |  |  | ③ |  |  |  | ④ |  |

Note: SI i denotes the ith signalized intersection in Fig.6

For estimating the mean values, we mainly choose the basis functions in the belief models to be the linear and quadratic terms of the decision variables. The features selected for variance prediction (i.e., VEPM) is simply the linear terms of the decision variables, and we divide the solution space based on bi-partition of the domain of each dimension. These two strategies are similar as those applied in section 5.1. To reduce the computational burden, we mainly consider the basis functions or features associated with intersections on the two main roads (i.e., Furong Middle Rd, and Shaoshan Rd c.f., Fig. 6(b)). More specifically, we mainly select the signal phases for five intersections numbered 2, 6, 8, 12 and 14 along the abovementioned two main roads to form the parametric models. We consider that these signal phases may play a more important role in affecting the three traffic indices (i.e., D, A and E) compared to the others. Note that all these intersections have four phases, thus the relevant decision variables (i.e., green timings) are four times the number of intersections selected. For example, if the abovementioned five intersections are all selected, then there are totally 20 critical green timings of signal phases, and thus 40 particular basis functions need to be built for estimating the mean values, while 20 features are included for bi-partition and forming 220 combinations of these partitions with the combination index qx∈{1,2,⋯,220}.

We test mainly three GPR-KG algorithms differed in term of which intersections are considered in the parametric model of the mean values: the algorithm that considers small number of intersections (one from Furong Middle Rd and the other from Shaoshan Rd), medium number of intersections (all the five on Furong Middle Rd and Shaoshan Rd), and large number of intersections (all the fifteen intersections on the network), named GPR-KG-s, GPR-KG-m, GPR-KG-l, respectively. All these three algorithms adopt quadratic polynomials with no cross-multiplication terms as the belief model. To test the impact of cross-multiplication terms, we also extend GPR-KG-m to include the full set of second order cross-multiplication terms in the quadratic polynomial as the belief model, this new algorithm is named GPR-KG-c. Details of the settings of these algorithms are given in Table 3. Note that all these four algorithms adopt the same VEPM scheme which is based on the partition of the solution domain considering the decision dimensions associated with intersections 2, 6, 8, 12 and 14. We do not include more dimensions in the partition to avoid out-of-memory, since this partition already contains 220 pieces. Our experiments are carried out on a PC with Intel Xeon 2.0 GHz, the results will be discussed in the next subsection.

Table 3 GPR-KG algorithms with different parametric models

| Algorithm | Features of the parametric belief models for the mean values |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Intersections included | # linear terms | # squared terms | #cross terms | Tot. # terms |
| GPR-KG-s | 6, 8 | 8 | 8 | 0 | 16 |
| GPR-KG-m | 2, 6, 8, 12, 14 | 20 | 20 | 0 | 40 |
| GPR-KG-l | all 15 intersections | 60 | 60 | 0 | 120 |
| GPR-KG-c | 2, 6, 8, 12, 14 | 20 | 20 | 190* | 230 |
| *The number of cross-multiplication terms is 20×19/2 = 190 |  |  |  |  |  |

5.2.2 Results discussion

We first evaluate the filed-implemented signal control plan by 100 independent simulations. The frequency histograms of the D, A and E values of this benchmark solution are shown in Fig.7, where the Gaussian distributions are also fitted. We observe that though Gaussian distributions cannot perfectly describe the underlying distributions, they are able to describe the major variability of the three indices.

(a)                                       (b)

(c)

Fig. 7 Frequency histograms of (a) D, (b) A and (c) E values of the filed-implemented signal control plan based on 100 independent simulations. Note: in (a), μD and σD are the sample mean and standard deviation of D, respectively; in (b), μA and σA are the sample mean and standard deviation of A, respectively; in (c) E1-ϵ is the estimate of 95% quantile of E approximated by the 95th largest sample.

To validate the learning process if our algorithm, we compare those iterative samples generated during the run of the algorithm with the set of learned non-dominated solutions (LNSs) finally output by the algorithm, i.e., those feasible non-dominated solutions computed based on the final posterior belief. Fig. 8 shows that the iterative solutions are quite dispersed in terms of their D, A and E values, which indicates the extensive exploration capability of the GPR-KG algorithms in the domain of the three evaluation indices. More importantly, nearly all the iterative signal control plans have their sampled bi-objective values (relative total delay D and equity measure A) dominated by the posterior mean bi-objective values of the LNSs, under either of the four algorithms. We also observe that a significant portion of the iterative solutions (mainly those of the early iterative solutions) have their sampled relative emission level E notably higher than 0.9. On the other hand, the LNSs output by all four algorithms have their posterior E values notably lower than 0.9, which verifies that our algorithm can deal with the constraint on E as expected. Hence, we have verified the significant progress made by our algorithms by exploring the solution space and updating the posteriors iteratively.

(a)                                               (b)

(c)                                               (d)

Fig.8 D, E and A values of iterative and learned non-dominated signal control plans by (a) GPR-KG-s, (b) GPR-KG-m, (c) GPR-KG-l, (d) GPR-KG-c. Note: LNSs represents the learned non-dominated solutions. The points of three evaluation indices (D, A, E) of iterative solutions in this figure are evaluated by simulation evaluations, and those for LNSs are evaluated by parametric models.

In addition, we compare the LNSs over different algorithms by re-evaluating their mean A and D values for each LNS by averaging 20 independent random simulation samples. Then those non-dominated solutions among the set of LNSs for each algorithm are identified based on their sample means of A and D values, and the corresponding Pareto front can be found. Fig.9 (a) shows that overall, the LNSs have better values by using a belief model that covers more intersections. Fig.9 (b) shows that the Pareto front of the LNSs by GPR-KG-l is the best, while that by GPR-KG-s is the worst. Interestingly, out of the six non-dominate solutions among all those LNSs found by all the four algorithms (which form the ‘final Pareto front’), four are output by GPR-KG-l, and one by GPR-KG-m and the other by GPR-KG-c.

(a)

(b)

Fig. 9 Comparison of the results when GPR-KG algorithm is applied to problem (6) with different parametric models. (a) D, E and A values of LNSs. (b) Pareto fronts for LNSs. Note: in (a) and (b), the tri-objective points of all solutions are mean values evaluated by simulation evaluations over 5 runs.

To compare the four algorithms more rigorously, Table 4 reports the following results for each algorithm: 1) some basic statistics of the three evaluation indices of the LNSs such as the average and variance of the sample means of A, D and E values as well as the average and variance of the estimated 95%-quantile E values; 2) the percentage improvement of the mean of A and D values and 95%-quantile E values of the LNSs compared to those of the filed-implemented signal timing plan; 3) the approximate IGD values of each set of LNSs by comparing their sample average A and D values with those on the final Pareto front, and 4) the computational time for making sampling decisions.

Table 4 Test results for four GPR-KG algorithms with various belief models

| Algorithm | GPR-KG-s | GPR-KG-m | GPR-KG-l | GPR-KG-c |
| --- | --- | --- | --- | --- |
| D, A, E | 0.984, 0.942, 0.860 | 0.880, 0.890, 0.828 | 0.849, 0.870, 0.794 | 0.877, 0.904, 0.818 |
| VarD, VarA, VarE | 0.002, 0.001, 0.001 | 0.001, 0.001, 0.001 | 0.001, 0.001, 0.001 | 0.002, 0.001, 0.001 |
| E1-ϵ, VarE1-ϵ | 0.973, 0.0013 | 0.899, 0.0011 | 0.864, 0.0003 | 0.894, 0.0006 |
| Dmin, Dmed, Dmax | 0.897, 0.982, 1.055 | 0.835, 0.899, 0.956 | 0.816, 0.833, 0.887 | 0.788, 0.865, 0.953 |
| Amin, Amed, Amax | 0.876, 0.939, 1.013 | 0.828, 0.878, 0.995 | 0.819, 0.868, 0.931 | 0.846, 0.899, 0.968 |
| E1-ϵmin, E1-ϵmed, E1-ϵmax | 0.893, 0.970, 1.044 | 0.837, 0.897, 0.967 | 0.821, 0.863, 0.907 | 0.842, 0.892, 0.953 |
| PD, PA | 1.5%↓, 1.2%↓ | 11.9%↓, 6.6%↓ | 15.0%↓, 8.7%↓ | 12.2%↓, 5.1%↓ |
| PE1-ϵ | 14.7%↓ | 21.2%↓ | 24.3%↓ | 21.6%↓ |
| IGD | 0.101 | 0.023 | 0.008 | 0.037 |
| Computational time | about 27 hours | about 27 hours | about 30 hours | about 65 hours |

Note: D (VarD), A(VarA) and E(VarE) denote the average (variance) of the expected D, A and E values of LNSs shown in Fig. 8(a). Dmin (Dmed) [Dmax], Amin (Amed) [Amax] and E1-ϵmin (E1-ϵmed) [E1-ϵmax] represent the minimum (medium) [maximum] expected D, A values and 95%-quantile E values among the LNSs, respectively. Here the expected D, A and E values of each LNS are estimated by sample means over 20 independent simulation evaluations, while the 95%-quantile E value of each LNS is estimated by the 19-th largest sample value. PD, PA and PE1-ϵ represent the percentage changes of D, A and E1-ϵ, respectively, of the LNSs compared to the filed-implemented signal timing plan. IGD is computed for the set of LNSs based on the final Pareto front in Fig. 8(a). The computational time is the time spent on a total of N = 300 sampling decisions (the simulation time is excluded, which is about 15 hours).

We can see from Table 4 that, in terms of the two expected objective values A and D, the improvement from GPR-KG-s to GPR-KG-m is more significant than that from GPR-KG-s to GPR-KG-l. Besides, extending GPR-KG-m to GPR-KG-c by adding cross-terms of those selected decision dimensions in the belief model does not improve the performance, with IGD even increased by 0.014. On the other hand, though the average E1-ϵ estimates of the LNSs of all four algorithms expect for GPR-KG-s are less than τE=0.9, GPR-KG-l is most reliable in generating LNSs that satisfy the probabilistic constraint as it has the lowest E1-ϵmax value. In fact, only 2 out of 22 LNSs obtained by GPR-KG-l have their E1-ϵ estimates slightly over τE=0.9, while 27 out of 30, 15 out of 35, and 14 out of 37 of the LNSs obtained by GPR-KG-s, GPR-KG-m, and GPR-KG-c, respectively, have their E1-ϵ estimates higher than 0.9. Interestingly, the absolute deviation between the quantile estimates of E by Gaussian assumption and those by the order statistics are relatively small (within 0.01 to 0.05 among the LNSs) under all four algorithms, implying that Gaussian assumption for E is rather reasonable. Hence, in this case a small improvement in mean estimate can lead to significantly more accurate feasibility prediction, which explains why the LNSs generated by GPR-KG-l have the highest feasibility rate. Meanwhile, we note in Table 4 that the computational times of GPR-KG-m and GPR-KG-s are similar, and that of GPR-KG-l is only about 11% higher. However, the computational time of GPR-KG-c is significantly higher. Therefore, in view of the trade-off between computational cost and solution quality, we conclude that a belief model considering only the linear and squared terms associated with all the intersections should be sufficient.

For each algorithm, we further generate the empirical cumulative density function (CDF) of D, A and E values of the LNSs using 100 independent samples each with a randomly selected LNS. The empirical CDF of D, A and E values of the field-implemented signal control plan are also generated by 100 independent simulation evaluations. These CDFs are plotted in Fig. 10. For each evaluation index, the CDFs of the signal plans found by either of our algorithms appear to be notably better than that of the field-implemented plan. Among all four algorithms tested, GPR-KG-l is most desirable and achieves slightly better performance compared to GPR-KG-m and notably outperforms GPR-KG-s in terms of all three evaluation indices. Compared to the solutions by GPR-KG-m, those by GPR-KG-c have very similar D and E distributions but are slightly worse in the distribution of A value. These observations are consistent with those discussed earlier. Meanwhile, Fig. 9 gives direct comparison of various quantiles of D, A and E values between different signal plans, which can lead to useful interpretations. For example, it is about at least 70% (80%) probable that any randomly selected signal plan found by GPR-KG-l achieves a lower relative total delay (relative traffic equity) than the field-implemented plan. Therefore, the effectiveness of GPR-KG algorithms and the trade-off in choosing the belief model are illustrated again.

(a)                      (b)                       (c)

Fig. 10 Empirical CDFs of (a) relative total delay, (b) relative traffic equity and (c) relative vehicular emission for nine comparison solutions.

6 Conclusion

We have proposed a SO approach for bi-objective stochastic optimization problems with probabilistic constraints and applied it to a nontrivial traffic network control problem which involves a very large number of feasible solutions. The challenges of these type of problems lie in effective learning of both the mean values of the two objectives as well as the quantile of the constraint function, particularly when the decision space is large. Therefore, we designed a flexible parametric GPR based posterior distribution over the feasible solutions and proposed an efficient approximate knowledge gradient sampling policy. The parametric belief model is flexible enough such that domain knowledge can be used to specify any feature vector associated with the solution. Moreover, we proposed a convenient feature-based iterative variance updating scheme VEMP aiming at fast learning of the variability distributions over the large solutions space under limited simulation budget. This variance updating scheme is based on grid divisions on the domain of the feature vector and deals with sampled and unsampled solutions separately such that the sampling policy has theoretical convergence guarantee.

Our experiments on a set of synthetic test problems validate the effectiveness of the proposed algorithm in finding the non-dominated feasible solutions. In all the three problems tested, the Pareto front found by our algorithm reasonably covers the theoretical one, though a higher initial variance estimate may lead to fewer solutions, mainly due to a more conservative posterior probabilistic constraint. However, we verify that the proposed VEMP scheme can facilitate variance learning and significantly remedy the inaccuracy in the initial variance estimate. The proposed algorithm is also successfully applied to a real-world traffic network signal timing problem which involves relative total delay and traffic equity as two stochastic objectives and a probabilistic constraint on the relative vehicular emission. Compared to field-implemented signal plan, the set of solutions that finally found by our algorithm can reduce 14.1~18.3% and 2.3~11.2% for the expected values of the two objectives, respectively. Unlike the field implemented signal plan, the solutions found by our algorithm all satisfy the probabilistic constraint, with the 95%-quantile emission level cut by more than 24%. The trade-off between the complexity of the parametric belief model and computational effort is also discussed. Therefore, the proposed SO approach is promising in solving probabilistically constrained bi-objective stochastic optimization problems. Although the simple quadratic model was shown effective for our problem, one meaningful future exploration might be designing adaptive feature selection method that can tailor the belief model to different problem domains.

Appendix A: Three evaluation indices of the network signal timing

Given a network-wide signal timing plan, the calibrated traffic simulation model will output the individual vehicle information, which helps to calculate the evaluation indices about traffic efficiency, traffic equity and vehicular emissions. As for the traffic efficiency, the relative total delay is utilized as the evaluation index, which is defined as

D=h=1Nvdh/C                                (A1)

where dh is the delay experienced by vehicle h during the simulation, Nv is the total number of vehicles during the simulation, C is the normalization constant regarding the total delay that is evaluated by the calibrated traffic simulator with the field implemented signal plan. Obviously, the lower D value means the higher traffic efficiency.

As for the traffic equity, the Atkinson index proposed to measure the inequity in social welfare distribution (Atkinson, 1970) is adopted and formulated as

At(tr,1,tr,2,⋯,tr,h,⋯,tr,Nv)=1-1π1Nvh=1Nvtr,h1-η11-η,for 0≤η≠1        1-1πh=1Nvtr,h1Nv,for η=1                       (A2)

where tr,h=th/tJh denotes the travel time ratio of vehicle h that can compare the benefits achieved by travelers from the same and various routes, th is the travel time of vehicle h, Jh denotes the route that vehicle h traverses, tJh=h=1|Jh|th/|Jh| is the average travel time of route Jh, |Jh| indicates the number of vehicles from route Jh. That is, tJh are measured by the average travel time of all vehicles from route Jh. π is the mean travel time ratio. η is often called the “inequality aversion parameter”. When η=0 (no aversion to inequity), At=0. When η=∞ (infinite aversion to inequity), At=1. So, At varies between 0 and 1, and the lower At for a given η value means the more equal distribution of travel time ratios.

Moreover, to describe the relative change of traffic equity under various signal plans, the relative Atkinson index is given as

A=At/At0                                (A3)

where At0 is the normalization constant regarding the Atkinson index that is evaluated by the calibrated traffic simulator with the field implemented signal plan.

As for the vehicular emissions, a linear combination of four pollutants’ emissions (Mayeres et al., 1996; Osorio and Nanduri, 2015b) is employed and formulated as

EM=wCO2nCO2fCO2+wNOxnNOxfNOx+wVOCnVOCfVOC+wPMnPMfPM             (A4)

where wCO2, wNOx, wVOC and wPM are the economic weighting parameters of CO2, NOx, VOC, and PM pollutants, respectively, and nCO2, nNOx, nVOC and nPM are normalization constants for these pollutants. The total emission (g) of pollutant p∈{CO2, NOx, VOC, PM} during the simulation is calculated by

fp=h=1Nvt=thThERhpt                            (A5)

where ERhpt=maxE0hp;C1hp+C2hpvh(t)+C3hpvh(t)2+C4hpah(t)+C5hpah(t)2+C6hpvh(t)ah(t) is the instantaneous emission rate of pollutant p (g/s) for vehicle h at time t (Panis et al., 2006), th and Th are the beginning and ending time-instants of vehicle h appearing in the simulation network, respectively. vh(t) and ah(t) denote the instantaneous speed (m/s) and the instantaneous acceleration (m/s2) of vehicle h at time t, respectively. E0hp is the minimal emission rate of pollutant p (g/s) for vehicle h, and C1hp to C6hp are emission rate coefficients for pollutant p and vehicle h.

Similarly, to describe the relative change of vehicular emissions under various signal timing plans, the relative vehicular emission is used and expressed as

E=EM/EM0                                (A6)

where EM0 is the normalization constant regarding the vehicular emissions that is evaluated by the calibrated traffic simulator with the field implemented signal plan.

Appendix B: Pseudo-codes of algorithms.

Algorithm 1: GPR-KG algorithm for solving bi-objective probabilistic constrained SO problems

Inputs: bi,0, Bi,0, zi,0, Fji, fj,li,l=1,…,mji, Λwi,0,w=1,…,j=1Mimji, i=1,2,3, j=1,…,Mi, Cn for n=1,…,N, K1, K2, xL, xU;  Output: PN (the Pareto optimal front)

1:  s0  an empty matrix % stores sampled solutions and their times sampled %if N0 random samples were used to initialize bi,0, Bi,0 and zi,0, they can also be used to initialize s0

2:  for n = 1 to N do

3:    generate Sn by Algorithm 2

4:    for k =1 to |Sn| do

5:        x  k-th element in Sn

6:      if n =1 or xn ≠ any entry of 1st column of sn-1 and qx=qxn-1

7:         λx i,n  Λq(x)i,n-1

8:      else

9:         λx i,n  λx i,n-1

10:     end if

11:   end for

10:   compute xn by Algorithm 3

12:   simulate solution xn and observe output yn=(yD,n,yA,n,yE,n)

13:   compute bi,nand Bi,n by formula (9), i=1,2,3

14:   compute λxn i,n by (10a) and Λq(xn)i,n by (10b)

15：  if xn ≠ any entry of 1st column of sn-1

sn sn-1 x    1

16:   else

snsn-1 with the last entry of the row corresponds to x added by 1

17:  end if

18: end for

19: solve problem (12) and obtain its Perato front PN defined in (18)

Algorithm 2: Parametric belief-based candidate sample set generation

Inputs: Cn, K1, K2, xL, xU;  Output: Sn

1:   Sn  Ø

2:   for k =1 to K1 do

3:    pick an integer vector x randomly from [xL,xU]

4:    if x∉Sn

5:      Sn  Sn∪{x}

6:    end if

7:   end for

8:   solve for the Perato front Pn-1 defined in (18) of problem (16) (with N replaced by n-1)

9:   if |Pn-1|≥ K2

10:    Sn  Sn∪Pn-1

11:  else

12:    for k =1 to K2-Pn-1 do

13:      select a solution x randomly from Pn-1

14:      sample x from Nx,Cn and round to integer values.

15:      x  minmaxx,xL,xU

16:      if x∉Sn

17:        Sn  Sn∪{x}

18:      end if

19:    end for

20:  end if

Algorithm 3: KG-based sampling decision

Inputs: Sn, sn-1, x, bi,n-1, Bi,n-1, zi,0, λxi,n-1,x∈Sn, i =1, 2, 3;  Output: xn

1:   VKG,n  a |Sn|-by-3 matrix

2:   PKG,n  Ø

3:   for k =1 to |Sn| do

4:    x  k-th element in Sn

5:    for i = 1 to 2 do %the two objectives

6:       compute vKG,i,nx using Algorithm 4

7:    end for

8:   k-th row of VKG,n k, vKG,1,nx,vKG,2,nx

9:   end for

10: sort the rows of VKG,n in nondecreasing order of its 2nd column

11: for k =1 to |Sn| do

12:  if k =|Sn| or  maxk'=k to |Sn| VKG,n(k',3) ≤VKG,n(k,2)

13:    x  k-th element in Sn

14:    PKG,n  PKG,n∪{x}

15:  end if

16: end for

17: return xn  an element in PKG,n %pick one arbitrarily or by certain rule

Algorithm 4: Computation of the KG factor

Inputs: Sn, sn-1, x, bi,n-1, Bi,n-1, zi,0, λxi,n-1,x∈Sn, i =1, 2, 3;  Output: vKG,i,n

1:  index each element in Sn by 1 to |Sn|

2:  x  the index of solution x

3:  Fi,n-1 a |Sn|-by-dim(bi,n-1) matrix

4:  for k = 1 to |Sn|

5:    x' k-th element in Sn

6:    k-th row of Fi,n-1  fx'i,n

7:  end for

8:  Bi,n-1  Bi,n-1

9:  if x ≠ any entry of 1st column of sn-1 % x has not been sampled

10:       Fi,n-1  Fi,n-1   ex  % add a column vector ex to the right

11:       Bi,n-1  Bi,n-100        zi,0

12: end if

13: p  Fi,n-1bi,n-1

14: q  Fi,n-1Bi,n-1(fxi,n-1)T/λxi,n-1+fxi,n-1Bi,n-1(fxi,n-1)T1/2

# 15: sort the pairs (pj,qj)j=1|Sn| so that qj are in non-decreasing order, and ties in q are broken so that pj≤pj+1 if qj=qj+1

16:  remove each pair j in (pj,qj)j=1|Sn| if (pj,qj)j=1|Sn| studyate intger p alternatives in each iteration

17:  J  the number of remaining pairs (pj,qj)

18:  a0-∞, aJ+∞

19:  for j =1 to J-1 do

20:     aj(pj-pj+1)/(qj+1-qj)

21:  end for

22: remove each triplet j in (pj,qj,aj)j=1J if aj≥aj+1

23:  J'  the number of remaining triplets (pj,qj,aj)

24: return vKG,i,nlogj=1J'-1φ(-|aj|)-|aj|ϕ(-|aj|)

Appendix C: Three test problem

The three test problems all take form (1) but differ in functions gi, i=1, 2, 3, as defined below.

1) In revised ZDT1 (RZDT1):

g1x,ξ=x1100+ξ                               g2x,ξ=1+9d-1t=2dxt100∙1-l1x,ξ1+9d-1t=2dxt100+ξg3x,ξ=-x1100-0.52+0.04+ξ

2) In revised ZDT2 (RZDT2):

g1x,ξ=x1100+ξ                                g2x,ξ=1+9d-1t=2dxt100∙1-l1x,ξ1+9d-1t=2dxt1002+ξg3x,ξ=-x1100-0.52+0.0

3) In revised ZDT5 (RZDT5):

g1x,ξ=x130+ξ                 g2x,ξ=t=2dxt5+1∙1x1+1+ξg3x,ξ=x130-0.5+ξ

The decision vector x=(x1,x2,⋯,xd)∈Zd. For RZDT1 and RZDT2, xt∈0,100, t=1,2,3,⋯,d. For RZDT5, x1∈0,30, xt∈0,5, t=2,3,⋯,d. For all benchmark problems, the white noise is ξ~N(0,0.052), and ε=0.05. Then, the theoretical Pareto optimal solutions of RZDT1 and RZDT2 are x1∈0,1,⋯,15∪{85,86,⋯,100}, and xt=0,t=2,3,⋯,d, and those of RZDT5 are x1∈{0,1,⋯,13}, and xt=0,t=2,3,⋯,d. Here, the dimension d is set as 5.