Abstract

This study proposes a simulation-based optimization (SO) method to solve stochastic constrained bi-objective urban transportation problems, whose bi-objectives and constraint can only be evaluated based on computationally expensive stochastic traffic simulators. It presents a parametric belief based Gaussian process regression to estimate the bi-objective values as well as the constraint at unsampled solutions with those at sampled solutions. We design a computationally efficient approximate knowledge gradient sampling policy to search for the feasible non-dominated solutions. A convenient variance updating scheme is also proposed, which is based on gridding of the feature space and is particularly helpful in learning the quantile of the constraint for estimating feasibility. The proposed SO method achieves satisfactory results for a set of synthetic numerical test problems. We also evaluate the performance of the algorithm for a real-word network-wide traffic signal timing problem under various uncertainty scenarios and tight computational budgets. Our method leads to well-performing nondominated signal plans, which can not only balance bi-objectives of expected normalized traffic efficiency and equity, but also guarantee a probabilistic constraint for the normalized vehicular emissions, under random perturbation of traffic volumes on the road network. Compared to the field implemented signal plan, those found by our algorithm can reduce the expected delay and fairness measures by 11% and 14% in average, respectively. Besides, these solutions all meet the 95% probabilistic constraint for the emission target, which was actually violated by the field implemented signal plan. The proposed SO method is promising for solving complex multi-criteria transportation problems with large solution spaces and can be applied to other similar bi-objective probabilistically constrained black-box problems.

Keywords: simulation-based optimization; bi-objectives; probabilistic constraint; traffic network signal timing.

1 Introduction

1.1 Background and motivation

Decision problems in urban transportation systems can have huge socio-economic impact, which need careful analysis in various levels of resolution and an efficient optimization framework. In general, these systems are complex (e.g., dynamic, stochastic, non-linear) and difficult to model analytically, which precludes the use of traditional optimization methods. A powerful alternative is simulation-enabled approach. Recently, a variety of commercial or open-source traffic simulation tools have been developed, providing us with a more flexible and comprehensive way to capture the intricate interactions between the supply and the demand components of the mobility networks. In current practices, there are widely applied traffic simulators such as microscopic ones (e.g., VISSIM, PARAMICS, SUMO) and macroscopic ones (e.g., VISUM, DTALITE, DYNUST). The former focuses on simulating traffic flow dynamics by detailed modeling of driver behavior, while the latter focuses on flow distributions on larger networks with only aggregate traffic flow model on each link. These microscopic and macroscopic traffic simulators can provide estimates of traffic network performance under various system uncertainties (e.g., random traffic volume inputs, random driving and routing behaviors, etc.). These tools are therefore often used to evaluate and compare a set of predetermined solutions (e.g., traffic facility locations or network design plans) in the context of what-if analysis or sensitivity analysis for better decision-making.

However, when the number of feasible or candidate solutions becomes large, it is impractical or prohibitive to simulate all of them. This is common in urban transportation problems which involve computationally expensive calibrated traffic simulation model and high dimensional decision variables. For example, the network signal timing problem needs to optimize the green times of all the phases at all the intersections in the network (Osorio and Bierlaire, 2013, Zheng et al. 2019, 2021), leading to a huge number of feasible solutions, which generally grows exponentially in problem dimension (Li et al., 2015). Simple enumeration is even less applicable when simulations are stochastic, as each simulation run only gives a noisy observation of the underlying quality of a solution. For such problems with large number of alternatives or measurement noises, research framework of simulation-based optimization (SO) was proposed, which design algorithms that seek good solutions quickly. SO has been an increasingly hot research topic, see relevant reviews (***).

In addition to the abovementioned challenges, urban transportation problems often involve multiple performance measures or criteria (Zheng et al., 2019...), such as traffic efficiency, safety, environmental impact and equity. Some of these performance measures are the primary objectives to be optimized, such as the total travel time on the network, whereas others have to satisfy some control targets such as the total traffic emissions on the network. Moreover, these performance measures are typically stochastic with very limited prior information about their distributions, making the problem harder. So, there is an urgent need for novel algorithms tackling stochastic constrained multi-objective problems arise in urban transportation systems. To this end, this study will focus on urban transportation problems in the following form:

minx∈X Eξ[f1x,ξ],Eξ[f2x,ξ]          s.t.   Prξ f3x,ξ-τ≤0≥1-ϵ                                    (1)

Functions f1x,ξ to f3x,ξ are three scalar-valued stochastic performance measures output from traffic simulator, which depend on the decision vector x and the random parameter vector ξ in the traffic simulator. Suppose x lies in a known feasible set X, and ξ has a known distribution p(ξ) with bounded support Ξ. Depending on what underlying system stochasticity needs to be captured for the problem, ξ can contain uncertain parameters in traffic demand, driving behavior, routing behavior, etc. Eξ and Prξ denote the expectation and the probability with respect to ξ, respectively. The constraint means the probability that f3x,ξ is over the limit τ is at most ϵ∈(0,1), where τ and E are predetermined scalars. Problem (1) is therefore to find the nondominated solutions in terms of their expected f1 and f2 values with their f3 values satisfy the probabilistic constraint.

The main purpose of this study is to propose a novel SO methodology which can tackle problem (1) effectively. We will test the proposed method using both synthetic test problems and a real-world traffic network signal timing problem which involves high-dimensional discrete decision space and uncertainty in traffic demand.

1.2 Method overview and our contribution

Generally, there are three ways to address the SO problems: direct-search methods, stochastic gradient methods and metamodel methods (Osorio and Bierlaire 2013). Note functions f1 to f3 in problem (1) are all stochastic and costly to evaluate and may require different procedures or components in simulations. Thus, an ideal SO algorithm should make full use of the information given by the simulation observations, i.e., infer useful information globally with a local observation. In this regard, direct-search and stochastic gradient methods are not the best choice, since accurate estimation of search directions or gradients is sample-intensive and only provide local information. Thus, in our algorithm design, we focus on metamodels to approximate the expectations in the objectives and the probability in the constraint of problem (1). The metamodels are iteratively fitted using the simulated observations and provide improved prediction of the underlying performance measures globally to guide the search of the feasible Pareto optimal solutions. In fact, metamodel-guided sampling policies have been proposed extensively in SO literature. For instance, kriging model is used with expected improvement (EI) sampling (Forrester and Keane 2009), RBF model with scoring criterion search (Müller et al. 2014), polynomials with trust-region search (Osorio and Bierlaire 2013), Gaussian process regression (GPR) with Bayesian optimization (BO) (e.g., Scott et al. 2011), etc. Naturally, the applicability and the performance of the algorithm rely on what combination of metamodel and sampling policy is used. We propose parametric-GPR-based approximate-KG sampling policy for tackling challenging problem (1), which is the main contribution of this study.

BO is a powerful SO framework which starts with a prior belief (estimates of the function values and the uncertainties) on the solutions and selects the new sample which maximizes the value of the measurement, the new observation is then used for updating the posterior belief. The value of measurement is called the acquisition function defined by the sampling policy. In particular, the KG policy defines acquisition function as the expected improvement in the best posterior mean (known as the KG-factor). KG policy has theoretical convergence guarantee and performs well in practice (e.g., Negoescu et al. 2011, Tan and Gao 2016, Wu and Frazier 2016). While, GPR is commonly used in BO due to neat posterior update of the Gaussian process. Many popular metamodels have similar essence as GPR, such as various kriging models that estimate spatial correlations (***), and the Gaussian Markov random fields which restrict GPR to have certain dependence structure among the solutions (Semelhago, et al. 2019). Advantages of the GPR-based KG policy include: 1) convenient KG-factor computation with explicit modeling of possible correlations among solutions (Frazier 2009); 2) flexibility in parameterizing the prior as linear combinations of any basis functions (or features) of the solution (Powell and Ryzhov 2009); and 3) available techniques for accelerating the computation of the sampling decision when solution space is large (Ryzhov and Powell 2009, Tan and Gao 2016). We design parametric GPR models for f1 to f3 with problem-specific feature vector of the solution, which makes computation affordable by maintaining posteriors only on model coefficients and the sampled solutions rather than on all the solutions. In addition, we extend the GPR based KG policy to multiobjective setting by combining the posterior distribution and the deterministic multiobjective solvers in generating candidate samples for the next iteration.

Our second contribution is designing a novel updating scheme for learning unknown and heterogeneous variances of the solutions. In urban transportation problems, the variance of the same performance measure can vary notably for different solutions, as observed, for example, in Osorio and Bierlaire (2013) and Tan and Gao (2016). This necessitates effective learning of the variances especially considering the fact that most solutions will not be sampled under limited budget. In our approach, we divide the feature space of the solutions into grids, then we estimate the variance of any unsampled solution x by the average of those of the sampled solutions whose features lies in the same grid as x. This grid-based learning approach is similar to that proposed by Salemi et al. (2019), but they defined grids directly on the solution space, which can be costly to manage when problem dimension grows. Our variance estimation approach enables cheap approximation of the KG-factor, which is one research focus of the KG policy (Qu 2015, Zhang 2016) since exact Bayesian inferences is impossible with single observation under a multivariate Gaussian prior. Also note that our approach for learning under measurement noises is quite different with those proposed for cases with relatively small number of alternatives, e.g., sample average approximation (SAA) (***), simulation allocation rule such as OCBA (Li et al., 2015), shrinking ball method (Fan and Hu, 2018), etc., where each alternative is sampled for multiple times and variances are typically known.

Finally, this study enriches the literature on BO problems with noisy constraints, a relatively new research topic. Recent studies have looked at single objective constrained BO problems for expected improvement (EI) policies (Gelbart et al. 2014, Gramacy 2016, Letham et al., 2019). They applied a GP prior directly on all the solutions and implicitly assumed known noise distributions, which are not applicable to high-dimensional problems with unknown variances. In contrast, our parametric GPR model and the variance learning scheme enable learning under noises for high-dimensional problems. Interestingly, these studies multiplied the original acquisition function (i.e., the unconstrained case) by the probability of feasibility conditional on the posterior, but this leads to very cumbersome computation. However, we do not add this complexity since unlike the EI policy, the KG policy can choose an unsampled decision for final implementation. So, our approach is tractable and can have faster fitting of the proposed parametric GPR model by allowing broader exploration, thus identify Pareto optimal solutions faster. Note that the probabilistic constraint in problem (1) is equivalent to restricting the (1-ϵ)-quantile of f3 to be at least τ. Some related study also used Gaussian process models in optimizing quantiles (Wang et al. 2021). Based on similar Gaussian assumption, we can conveniently predict feasibility using the mean and variance estimates of f3. We demonstrate the effectiveness of our approach using both test problems and a real-world urban transportation problem.

The remainder of the paper is organized as follows. We present a literature review of the related SO methods in Section 2 including the KG sampling policy. A network-wide signal timing problem under uncertainties (NSTPU) is formulated in Section 3 as an application of model (1). In Section 4, we design parametric GPR and KG sampling policy for NSTP. We apply the proposed method to a test problem and discuss the results in Section 5. Finally, some important findings are drawn in Section 6.

2 Literature review

In this section, we will first review the stochastic SO methods that can address computationally expensive simulation-based problems, whose objectives are formulated as expectations with respect to some stochastic parameters involved in the simulation. We will review those SO methods for problems that include single/multiple objectives and are with or without nontrivial constraint (i.e., simulation-based constraint or probabilistic constraint). One notable stream of the SO literature focuses on the BO approach which maintains some probabilistic beliefs of the underlying objective values over the feasible solution space, based on which different sampling policy (a.k.a. infill strategy) are designed. The BO approach has been successfully applied to problems in various fields, we will also apply it to our traffic SO problem. In particular, we will adapt the GP-based KG sampling policy to our stochastic bi-objective traffic problem with probabilistic constraint. To this end, we will also briefly review relevant studies on KG policy in this section.

2.1 Stochastic SO methods

For stochastic simulations, the expectations of simulated performance measures are often considered to be optimized. The stochastic noises in response need to be carefully considered by surrogate model and optimization algorithm. Based on random search, Hong and Nelson (2006) proposed a convergent optimization via most-promising-area stochastic search (COMPASS) algorithm to solve constrained or unconstrained discrete optimization via simulation (DOvS) problems. It employs SAR to determine the simulation repetitions for each candidate solution and AAM to estimate its performance measures. For responses with homoscedastic noises, Huang et al. (2006) proposed sequential kriging optimization (SKO) with the nugget effect GP as the surrogate model and an augmented EI as the infill strategy. For responses with heterogeneous noises, Picheny et al. (2013) put forward an extension of EI named expected quantile improvement to consider the known noise levels at both the already observed design points and the future candidates. To handle the heteroscedastic simulation noises, Ankenman et al. (2010) proposed stochastic kriging, and Deng et al. (2012) and Ng and Yin (2012) developed Bayesian kriging, which also account for the extrinsic uncertainty about the unknown response surface. The difference between them lies in the fact that the model parameter uncertainty is considered in the latter but not in the former. Recently, Chen et al. (2018) enhanced the stochastic kriging and Bayesian kriging by incorporating the parameter uncertainty into the model architecture, which results in the Bayesian stochastic kriging. This approach explicitly accounts for the parameter uncertainty in the regression and prediction, and relaxes specific assumptions on the prior probability distributions of model parameters that exist in Bayesian kriging. Additionally, Chen et al. (2017) extended the cokriging model, which was originally developed by Kennedy and O’Hagan (2000) to model deterministic multifidelity problems, to stochastic simulation metamodeling for expectations, i.e., stochastic cokriging model. Quan et al. (2013) and Pedrielli et al. (2020) proposed the two-stage sequential optimization (TSSO) and the extended TSSO (eTSSO), respectively, in which the stochastic kriging is applied as the surrogate model, the EI criterion as the infill strategy and the optimal computing budget allocation (OCBA) (Chen et al., 2000) technique to decide the number of replications at design points. Besides, Salemi et al. (2019) used a Gaussian Markov random field (GMRF) to learn and exploit spatial relationships among the decision variables and objective function values, which is particularly well suited to the discrete stochastic SO problem with a large number of feasible solutions and a high computational cost for each simulation. Later, Semelhago et al. (2020) proposed a rapid Gaussian Markov Improvement Algorithm (rGMIA), in which the underlying Gaussian process is a discrete GMRF, and this algorithm delivers both a global convergence guarantee and finite-sample optimality-gap inference for significantly larger problems. Fan and Hu (2018) presented an adaptive search algorithm called surrogate-based promising area search (SPAS) for solving stochastic SO problems with Lipschitz continuous objective functions. The SPAS algorithm employs a shrinking ball method to estimate the performance of sampled solutions, a surrogate model fitted iteratively with the performance estimates to approximate the response surface of the objective function, and a promising area adaptively constructed for searching improved solutions.

As for multiobjective stochastic SO methods, Forrester and Keane (2009) introduced regressing kriging to deal with homoscedastic noisy data and multiobjective EI as the infill strategy, which can help to tackle multiobjective stochastic SO problems. Li et al. (2015) developed multiobjective COMPASS (MO-COMPASS), as an extension of the single-objective COMPASS, to solve DOvS problems with two or more objectives by considering the Pareto optimality and the probability of correct selection. Notably, MO-COMPASS also applies SAR to determine the simulation budget for each candidate solution and AAM to estimate its performance measures. Cooper et al. (2020) proposed the retrospective partitioned epsilon-constraint with relaxed local enumeration (R-PERLE) algorithm to solve the biobjective stochastic SO problem on integer lattices, and numerical experiments showed its favorable performance relative to R-MinRLE and MO-COMPASS. Based on a biobjective derivative-free trust region (BOTR) algorithm proposed by Ryu and Kim (2014), Zheng et al. (2019) put forward a biobjective stochastic simulation-based optimization (BOSSO) method, which introduces regressing kriging models to smooth the simulation noises, an adaptive selector to determine samples in the local trust-region to be evaluated by simulation, and a decision maker’s non-interactive role to impact the search direction and trust-region. The proposed BOSSO method has been successfully applied in the network-wide traffic signal timing problem and validated to outperform other counterpart algorithms.

Previous single- or multi-objective stochastic SO algorithms involving surrogate models have mostly dealt with bound constrained (also called box-constrained) SO problems with only expensive objective functions, and so the surrogate models are used to approximate the objective function only. As for addressing (single- or multi-objective) SO problems with expensive constraints, Regis (2011) presented a new derivative-free algorithm called ConstrLMSRBF to optimize expensive black-box objective functions subject to expensive black-box inequality constraints. This algorithm uses radial basis function (RBF) surrogate models and extends the Local Metric Stochastic RBF (LMSRBF) algorithm by Regis and Shoemaker (2007) that can handle black-box inequality constraints. Müller et al. (2013) introduced a surrogate model-based algorithm named SO-MI to address expensive mixed-integer black-box global optimization problems that may have expensive constraints, in which the RBF surrogate model is used to select candidates at which the expensive objective and constraint functions are to be evaluated. Later, Müller et al. (2014) presented a variant of SO-MI, which is called surrogate optimization-integer (SO-I) algorithm, to solve purely integer optimization problems that have expensive black-box objective functions and constraints. Müller and Woodbury (2017) put forward a global optimization with surrogate approximation of constraints (GOSAC) algorithm with two phases to address problems with expensive black-box constraints, cheap objective functions and continuous, integer or mixed-integer variables. For a given budget of expensive constraint evaluations allowed, GOSAC finds better feasible solutions more efficiently than other algorithms. Feliot et al. (2016) proposed Bayesian Multi-Objective Optimization (BMOO), which handles objectives and constraints in a unified way by an extended domination rule and calculate and optimize an expected hyper-volume improvement sampling criterion using Sequential Monte Carlo techniques.

As is known, the mean of the response is a widely used performance measure in a stochastic system, while it is not able to evaluate the possible variability or describe the entire distribution adequately. In this condition, the quantile has become increasingly popular and of great interest in many fields to provide more thorough profiles of the response distribution (Longerstaey and Martin, 1996; Wipplinger, 2007; Cope et al., 2009). In risk management, quantile, also termed value at risk, is one of the primary risk measures to quantify and interpret the risk that one stochastic system may face. So, the surrogate models for quantiles have been developed extensively, e.g., quantile regression (QR) model, stochastic kriging model, and stochastic cokriging model (Koenker and Hallock, 2001; Chen, 2009; Dabo-Niang and Thiam, 2010; Chen and Kim 2016; Xie et al. 2018; Wang et al., 2021), based on which some optimization algorithms can be integrated for quantile optimization. Furthermore, the black-box constraints in aforementioned constrained SO algorithms are usually evaluated by expensive deterministic simulations, and do not consider the stochastic performance about the constraints. However, as mentioned previously, the stochasticity inherent in the simulation evaluation tool that commonly exists in various engineering fields would result in stochastic evaluation values of the constraint functions. Therefore, it is necessary and more challenging to account for the stochastic performance about the constraints. In this study, as formulated by the model (1), besides of using the expectations on two stochastic performance measures as biobjectives, we set a probabilistic constraint about another performance measure evaluated by stochastic simulation, which can be treated as the inverse solution of the corresponding quantile function. That is, the probabilistic constraint is a way to control the risk that the stochastic traffic system may face in terms of this performance measure. Some recent studies have proposed BO methods in dealing with single-objective probabilistically constrained problems (Gelbart et al. 2014, Gramacy 2016, Letham et al., 2019). These studies attached GP priors directly on the objective and constraint of each solution and proposed EI-based sampling policies, which are not applicable to problems that involve high-dimensional decision variables and multiple objectives. In contrast, our study will build a new BO approach which aims at efficient learning of the mean performances of the two objective functions as well as the quantile of the constraint function, the key challenge is that these three functions are all defined on a high-dimensional discrete decision space.

2.2 The knowledge gradient (KG) sampling policy

Note that some of the aforementioned SO methods (mainly for the single-objective problems) are good examples of the BO approach (e.g., Salemi et al. 2019; Letham et al., 2019). As noted earlier, the BO approach has two fundamental components: the belief model describing subjective probability distribution over the solutions as well as the sampling policy based on the acquisition function defined for the solutions. While the GP is the most-commonly adopted belief model in BO, the options for sampling policies are more diversified. In particular, many studies adopt the KG policy for the sampling decisions in BO, whose main advantages are mentioned in Section 1. The KG policy chooses the next solution (or alternative) to be sampled which has the largest KG-factor, i.e., the expected improvement in the best posterior mean conditional on all the previous samples. Such a methodology was shown to be effective and empirically efficient for many SO applications compared to many other approaches. The KG policy was firstly introduced in Gupta and Miescke (1996) for the independent belief case, where the unknown mean of the performance of each solution has an independent prior distribution. Frazier (2009a) proved serval key properties of the KG policy such as asymptotic optimality and derived a more general KG policy for correlated beliefs, where the prior mean vector follows a multi-variate Gaussian distributions (Frazier 2009b). Parallel-KG policy that samples alternatives in batches was also developed for the independent prior case (Wu and Frazier 2016), which was shown to outperform other batch BO algorithms.

KG policies that incorporate different belief models were further developed, which either learn the parameters of a structured functional form assumed for the underlying mean (e.g., Negoescu et al. 2011), or learn the probabilities that each of the functionals (with known parameters) is the true one (Chen et al. 2015). For problems with large number of alternatives, KG policies with parameterized beliefs enable affordable computation by maintaining beliefs only on those hyper-parameters or features of the decision vector rather than on each alternative. For example, linear-belief based KG policies were effectively applied to the drug discovery problem (Negoescu 2011) and the traffic network tolling problem (Tan and Gao, 2016). Approximation schemes of the KG policy have also been proposed to accelerate the algorithm by computing the KG-factor only for a subset of alternatives at each iteration. One intuitive and effective approach in choosing the subset is by Monte-Carlo (MC) sampling based on the posterior distributions of the mean performance (Ryzhov and Powell 2009) or of the hyper-parameters in the belief model (Tan and Gao, 2016). As most KG policies are designed for discrete alternatives, they have been extended to continuous alternatives by designing Guassian process representations and maintaining posterior beliefs at sampled solutions and those promising ones based on local derivate estimates (Scott et al. 2011).

However, those methods found in most KG literature are not readily applicable to the type of problems this study focuses on. Firstly, probabilistic constraints for simulation-based quantities have not been addressed in previous studies on KG policy, although some studies considered explicit simple deterministic (e.g., linear) constraints that defines the feasible set of the solutions space (Wang et al. 2021). Secondly, most of these studies only focused on single performance measure. Although some literature considered multi-objective KG problem, each performance measure was assumed independent across different alternatives (Yahyaa 2014). Thirdly, previous studies on the KG policy either assumed a known variance of the performance measure (e.g., Gupta and Miescke 1996, Frazier 2009a, Frazier2009b), or proposed local updating equation for the variance estimate for each sampled alternative (e.g., Tan and Gao 2016, Wang et al. 2021), or designed approximate formulae for updating the posterior variance vector of all the alternatives under the multi-variate Gaussian framework (e.g., Qu 2015, Zhang 2016). While the local updating approach neglects the possible similarities in variances of two close solutions, the approximate global posterior updating formula cannot be easily adapted to the setting with parameterized beliefs (thus is not applicable to problems with large solution space). Note that this unknown variance would make it more difficult to deal with the probability constraint as mentioned in the first point.

We will design a KG-based SO approach with parameterized GP belief and unknown variances that is applicable to problems with more than one performance measures as well as a probability constraint on another stochastic performance measure. As explained earlier, such a problem setting is commonly seen in urban transportation optimization problems and makes those problems challenging to solve, thus a reliable and efficient SO-based solution approach is highly needed. One contribution that may be of separate interest is that we proposed an updating scheme of variance estimates which not only accounts for possible similarities between the different solutions and can be easily implemented for the cases with parameterized beliefs as well. Such an updating scheme is shown effective especially when a probabilistic constraint is considered, in which case one wants to estimate the distribution of the constrained quantity as accurate as possible under limited simulation budget. Although the KG-based SO method proposed in this study is designed for discrete decision problems due to the nature of the signal timing problem, it can also be extended to continuous or mixed-integer decision problems with the techniques used in (Scott et al. 2011) mentioned earlier.