3 Network-wide Signal Timing Problem under Uncertainties (NSTPU)

Urban signalized intersections are not only the bottleneck of traffic efficiency and dense area of vehicular emissions, but also influence the equity of right-of-way in the urban road network. In this condition, traffic efficiency is set as an evaluation indicator rather than the only indicator, when designing the traffic control system. Specifically, the traffic equity has been considered in various traffic control problems, such as temporal and spatial inequities of ramp metering control (Levinson and Zhang, 2006), aggregated and non-aggregated equity indices of corridor control (Ma and Zhang, 2008), spatial equity index of dynamic ramp control (Meng and Khoo, 2010), travel time-based Atkinson index of network-wide traffic signal timing plan (Zheng et al., 2019a). Meanwhile, traffic signal control plan substantially affects the driving modes of vehicles which would result in various intensities of vehicular emissions. More specifically, vehicular emissions tend to decrease when the vehicle is accelerating, cruising, decelerating and idling (Unal et al., 2003), and increase considerably if vehicle stops are introduced (Rakha and Ding, 2003). Thus, apart from traffic efficiency, vehicular emissions on signalized traffic networks are greatly influenced by the traffic signal control plan. In this condition, traffic signal control optimization provides an alternative way to mitigate urban vehicular emissions. For example, Stevanovic et al.(2009) applied network-wide traffic signal control SO to reduce total delay, fuel consumption and CO2 emissions. Osorio and Nanduri (2015a, 2015b) applied a combined surrogate model of functional component and physical component in the regional traffic signal timing SO to reduce a weighted index of average travel time and high-resolution vehicular emissions. Zhang et al. (2013) minimized total delay and mean excess exposure simultaneously by using a simulation-based biobjective GA to solve a signal timing optimization model. Zheng et al. (2019b) used a biobjective stochastic simulation-based optimization (BOSSO) method to address a network-wide signal timing optimization problem whose objectives are a linear combination of four pollutants’ emissions and total delay.

Due to the conflicting nature between traffic efficiency and equity (Kotsialos and Papageorgiou, 2004; Zhang and Levinson, 2004; Kesten et al., 2013; Zheng et al., 2019a), we take traffic efficiency and equity as bi-objectives in this study. Meanwhile, we set vehicular emissions as another evaluation measure subject to probabilistic constraint, that is, vehicular emissions has to be no higher than a certain threshold value with a prespecified high probability. The uncertainty we consider in this study is the relative perturbation on the calibrated nominal traffic volume inputs to the network, and we denote this random perturbation level by ξ, representing the ratio of the traffic volume to its nominal value. We assume ξ follows a probability density function pξ with positive support. Let x denote an integer valued decision vector that specifies a network-wide signal timing plan. Then, as an example of model (1), the network-wide signal timing problem under uncertainties (NSTPU) is modelled as

minx∈X Eξ[Dx,ξ],Eξ[Ax,ξ]                         s.t.    Prξ⁡[ Ex,ξ-τE≤0]≥1-ϵ                              (2)

where X:= x∈Zn:xl≤x≤xu  is the feasible solution space, xl∈Zn and xu∈Zn are its lower and upper bounds, respectively, n is the dimension of x that depends on the operation needs of the intersections and the scale of the urban road network. Eξ[Dx,ξ] and Eξ[Ax,ξ] denote the expected values of the traffic efficiency measure and equity measure, respectively, with respect to the random perturbation level ξ. τE is the maximum acceptable level of vehicular emissions which cannot be violated with probability higher than ϵ (e.g., ϵ=0.05).

Since x is the set of green signal timings (typically in tens of seconds) of all signal phases (typically 2 to 6 phases for a signalized intersection) in the urban road network, the model (2) has to deal with high-dimensional discrete solution space. Let K=|X| be the total number of feasible signal timing plans, then K is a very large number. Another difficulty of solving problem (2) lies in the fact that functions Dx,ξ, Ax,ξ, and Ex,ξ are unknown and in general so complicated that can only be estimated through network traffic simulations. In the next section we propose a simulation-based optimization approach for solving problem (2) using Gaussian process regression with knowledge gradient sampling policy.

4 Gaussian process regression-based solution approach for NSTPU

4.1 Gaussian process approximation for NSTPU

Under random traffic volume perturbation, the evaluation indices are also random, whose exact distributions are in general unknown a priori due to the complicated underlying traffic network system and limited historical data. We denote the random variable Dx as the normalized total delay output for decision vector x under uncertain traffic volume perturbation, i.e., Dx is variable Dx,ξ with ξ~pξ. Similarly, we define Ax and Ex for normalized system output of traffic equity and vehicular emissions.

For each x∈X, we assume that Dx, Ax and Ex follow Gaussian distributions:

Dx ~ NμxD,λxD,                                                                      (3a)

Ax ~ NμxA,λxA,                                (3b)

Ex ~ NμxE,λxE                                (3c)

where μxi and λxi, i∈{D, A, E} are the unknown mean and variance of the corresponding evaluation index i for solution x. The key purpose of the simulation is to learn these quantities with the goal of solving optimization problem (2).

Suppose we are allowed to conduct a total of N simulations, each simulation chooses a solution x∈X for evaluation, the evaluation is noisy with an independent random realization of the uncertain parameter ξ drawn from pξ. We denote xn the solution selected for the n-th simulation, xnand yn=(yD,n,yA,n,yE,n) the corresponding evaluation indices computed based on the simulation output. yn=(yD,n,yA,n,yE,n). In other words, each simulation gives us a noisy sample of each of the three evaluation indices of the chosen solution, based on which we update our estimates for the underlying mean and variance of each evaluation index. Note that since there is noise in measuring each sample, a specific solution may be sampled for multiple times, i.e., xn=xn' for some 1≤n≠n'≤N. In the rest of the paper, we will use the notation pn, Pn, En, and Varn, n=1, 2,…,Nrespectively, to represent posterior probability distribution function, probabilities, expectations, and covariances conditional on all the sampled solutions up to stage n, Xn={x1,…, xn}, and observations up to stage n, Yn={y1,…, yn}, for n=1, 2,…,N.

For convenience, we define the mean vector μi=({μxi, ∀ x∈X}) for each evaluation index i∈{D, A, E}. We take a Bayesian viewpoint for the estimation process of the mean vector μi for each i∈{D, A, E}. We start with some prior distribution of μi, p0(μi), then we update the posterior distribution pn(μi) based on Xn and Yn for n=1, 2,…,N one after another. For computational tractability, for each i∈{D, A, E} we choose p0(μi) to be a K-dimensional normal distribution with mean θi,0 and covariance matrix Σi,0. Then based on the property of normal distribution, we know that pn(μi) is also a multivariate normal distribution for n=1, 2,…,N. Specifically, for n=1, 2,…,N, given θi,n-1,Σi,n-1, xn as well as observation  and ynn=1, 2,…,N, the posterior distribution pn(μi) is also a K-dimensional normal distribution whose mean θi,n and covariance Σi,n can be computed as

θi,n=θi,n-1+yi,n-θxni,n-1Σxnxni,n+λxni,nΣi,n-1exn,                        (4a)

Σi,n=Σi,n-1+-Σi,n-1exnexnexnTΣi,n-1Σxnxni,n+λxni,n,                          (4b)

where exn is a unit K-dimensional column vector with the entry corresponds to solution xn being one; θxni,n-1is the entry of vector θi,n-1 that corresponds to solution xn; Σxnxni,n denotes the diagonal entry of matrix Σi,n-1 that corresponds to solution xn; λxni,n denotes our best estimate of the sample variance of evaluation index i of solution xn at stage n λxni. In section 3.2, we will propose a parametric sequential estimation procedure for these sample variances.

Since the true underlying distributions of the evaluation indices Dx, Ax and Ex are not known a priori, problem (2) is itself unknown exactly. Thus we replace the true expectations E[Dx] and E[Ax] in (2) by their stage-N posterior means ENμxD=θxD,N and EN[μxA]=θxA,N, respectively, when the simulation budget is exhausted. We also replace the true probability P(Ex≤τE) by the posterior probability PN(Ex≤τE). Note that under the Guanssian assumption, we have

PNEx≤τE=PEx-θxE,NλxE,N1/2≤τE-θxE,NλxE,N1/2=ΦτE-θxE,NλxE,N1/2,

thus PNEx≤τE≥1-ϵE is satisfied by restricting

θxE,N≤ τE-Φ-11-ϵEλxE,N1/2                      (5)

where Φ is the cumulative density function of the standard normal distribution, so Φ-1 is the quantile function of the standard normal distribution. Note that similar as in (4), we use the stage-N estimate of the sample variance, λxE,N, in inequality (5).

Therefore, we can rewrite problem (2) based on Gaussian process approximation as

minx∈X θxD,N,θxA,N                                      s.t.  θxE,N≤ τE-Φ-11-ϵEλxE,N1/2                       (6)

4.2 Parametric belief on model parameters

4.2.1 Parametric belief on the mean vector

It is generally not convenient to work with a high dimensional Gaussian distribution in the learning algorithm for sampling decisions (e.g., the one that we will discuss in Section 4.3). For NSTPU, we note that the number of signal timing plan (i.e., K) can be very large. For example, consider a traffic network with only three intersections, each has only two phases whose green time ranges from 21 to 40 seconds, then we have K=(202)3=206, which is 64 million, a number that is already prohibitive for normal computation.

To this end, we seek proper parametric belief model to make the learning problem feasible. Specifically, we assume that the underlying mean of any solution x is a linear combination of a set of basis functions (or features) of x plus a deviation term, i.e., we have

μxi=ζxi+j=1Miβjifjix,  i=D, A, E,                     (7a)

where fji is the j-th deterministic basis function for evaluation index i, and Mi is the number of basis functions used for evaluation index i. The scalar βji represents the marginal contribution of the basis function value fjix to the mean of solution x’s i-th evaluation indices, μxi. ζxi is the deviation term. Thus, (7a) assumes that the underlying mean of each evaluation index for a certain solution can be written as a linear combination of the basis functions of the solution plus a specific deviation term. Define column vectors βi=(β1i,…,βMii)T and ζi=(ζxi,x∈X), and let Fi be a K-by-Mi matrix such that the  being the j-th entry of its row corresponding to solution x is fjix. We also denote fxi as the row of matrix Fi that corresponds to solution x, i.e., fxi=(f1ix,…,fMiix). . Then (7a) can be expressed in matrix form

μi=Fiβi+ζi,  i=D, A, E.                        (7b)

Now for i=D, A, E, we use a Mi-dimensional Gaussian distribution for the prior of the coefficient vector βi, p0βi, with mean bi,0 and covariance matrix Bi,0, and we use a K-dimensional Gaussian distribution for the prior of vector ζi, p0ζi, with zero mean and covariance matrix zi,0I, where zi,0>0 is a scalar and I is a K-by-K identity matrix, i.e., our prior belief is that the deviation terms of different solutions are independent and they have uniform prior uncertainty. Then the mean and the covariance matrix of the prior distribution of μi can be written as

θi,0=Fibi,0,                                     (8a)

Σi,0=FiBi,0(Fi)T+zi,0I,                          (8b)

Now we define for each i=D, A, E a stage-n coefficient vector βi,n, which contains vector βi as well as the deviation terms ζxi for all the distinct solutions x sampled up to stage n. Following this definition, we have βi,0=βi as no sample has been taken at stage 0, so the prior distribution of βi,0 is also Nbi,0,Bi,0. Then due to the property of multivariate Gaussian distribution, it can be verified that for n=1, 2,…,N, the posterior distribution (conditional on Xn, Yn and λxni,n-1), pnβi,n, is also a multivariate Gaussian distribution, whose mean bi,n and covariance Bi,n can be determined by (i=D, A, E)

bi,n=bi,n-1+yi,n-fxni,nbi,n-1fxni,nBi,n-1(fxni,n)T+λxni,n-1Bi,n-1(fxni,n)T,                (9a)

Bi,n=Bi,n-1+-Bi,n-1(fxni,n)Tfxni,nBi,n-1fxni,nBi,n-1(fxni,n)T+λxni,n-1,                           (9b)

where the column vector bi,n-1 and the matrix Bi,n-1 are defined in the following way: if xn has been sampled before stage n (i.e., the deviation term ζxni has already been included in vector bi,n-1),   then bi,n-1=bi,n-1 and Bi,n-1=Bi,n-1; otherwise, bi,n-1 is formed by appending a zero to bi,n-1, and Bi,n-1 is formed by appending a zero row and a zero column to Bi,n-1 and set the new diagonal entry as zi,0. Namely, the following revisions are made if xn is first sampled at stage n:

bi,n-1=bi,n-10,     Bi,n-1=   Bi,n-10T0zi,0.

fxni,n is a row vector that has a commensurate dimension with βi,n and has fjixn as the j-th entry for j=1,…,Mi and has zeros elsewhere except that a 1 at the entry whose index is the same with that of the entry in βi,n corresponding to the deviation term ζxni. For instance, suppose the last entry of βi,n corresponds to ζxni, then we have

fxni,n = f1ixn⋯fMiixn     0T1.

The updating equation (9) is similar with (4), but now instead of maintaining beliefs on the original mean vector μi, we only have to update the posterior distribution for parameter βi,n, which is typically much shorter than μi even though the size of βi,n generally grows with sampling stage n as new solutions are being sampled, as normally we have N≪K.

4.2.2 Parametric update for the variance estimate

Note that the estimate of the variance at stage n-1, λxni,n-1, is needed in equation (9) for generating the posterior mean and variance for pnβi,n. However, it is possible that the solution to be sampled at stage n, xn, has not yet been sampled or has only been sampled a few times (e.g., less than three) before stage n, leading to a poor estimate λxni,n-1 and thus pnβi,n. In problems such as NSTPU, the sampling budget N is typically much smaller than the total number of solutions K, which makes it challenging to estimate the variance of unsampled solutions. Therefore, we design a parametric model which captures possible similarities in the variability of two close solutions based on a straightforward set partition scheme. Specifically, we denote the domain of fjix on x∈X by Fji, for each j=1,…,Mi, and partition Fji into a set of mji disjoint subsets, fj,1i,…,fj,mjii, with l=1mjifj,li=Fji. Then there are a total of j=1Mimji different combinations of such sets over all j=1,…,Mi. Given x∈Xxl,xu we let qiq(x) be the combination index of the partitions of fjix for j=1,…,Mi, specifically, if fjix lies in the kji-th partition of Fji, we define qix=kMii+j=1Mi-1(kji-1)h=j+1Mikhi. For example, if Mi=3, m1i=m2i=m3i=2, f1ix, f2ix and f3ix, respectively, lie in the second partition, the second partition, and the first partition of their corresponding domains, then we have qix=1+(2-1)×2×2+(2-1)×2+1=7.

For each evaluation index i=D, A, E, and for each combination index w=1,…,j=1Mimji, we start with some initial variance estimatei=D, A, E, Λwi,0, and denote Λwi,n as the stage-n variance estimate for all the unsampled solutions with the same combination index w. We consider the following three cases for updating the variance estimates iteratively for stage n=1, 2,…,N.

Case 1: If x is sampled at stage n, i.e., x=xn, then its variance for evaluation index i is estimated by the weighted average of the initial estimate and the squared deviation of each sample from the corresponding posterior means:

λxi,n=s0Λqxi,0+l=1sxnyi,nxl-θxi,nxl2s0is0+ sxn=λxi,n-1s0+ sxn-1+yi,n-θxi,n2s0+ sxn,            (10a)

where s0i s0is the weight for the initial variance estimate of evaluation index i, sxn is the number of samples collected for solution x up to stage n, and nxl is the stage at which the l-th sample of x is collected. The second equality is due to the definition of λxi,n-1 by the first equality and l=1sxnyi,nxl-θxi,nxl2=l=1sxn-1yi,nxl-θxi,nxl2+yi,n-θxi,n2,, this iterative expression is computationally more efficient than the original definition in the first equality.

Case 2: If x is unsampled up to stage n but has the same combination index as the solution that is sampled at stage n, i.e., x≠xn, sxn=0 and qiqx=qiqxn, then its variance estimate of evaluation index i is set to be the average of the variance estimate of those sampled solutions including xn that have the same combination index as x for evaluation index ixn:

λxi,n=Λqiqxi,n=x:qiqx=qiqx,sxn>0λxi,nrqqixn=Λqiqxi,n-1+λxni,n-λxni,n-1λxni,n-λxni,n-1rqiqxn=λxi,n-1+λxni,n-λxni,n-1λxni,n-λxni,n-1rqiqxn,      (10b)

where rqqixn is the number of solutions that have been sampled by stage n and have combination index qiqx, The first and the fourth equalities hold by our definition of Λqixi,n, and the third equality is true by considering the two possible cases: 1) If sxnn>1 (i.e., xn has been already sampled before stage n), then rqixn=rqixn-1, so we have Λqixi,n-Λqixi,n-1=(λxni,n-λxni,n-1)/rqixn; and 2) If sxnn=1 (i.e., xn is firstly sampled at stage n), then we have rqxn= rqxn-1+1 and thus

λxi,n=Λqixi,n-1rqixn-1+λxni,nrqxn=Λqixi,n-1rqixn-1+λxni,n-1+λxni,n-λxni,n-1rqxn

=Λqixi,n-1rqixn-1+1+λxni,n-λxni,n-1rqixn-1+1=Λqixi,n-1+λxni,n-λxni,n-1rqixn,

where the second last equality is due to λxnxi,n-1=Λqiqxi,n-1 since xn was unsampled till stage n.

Case 3: If x is sampled before stage n but not at stage n or x is unsampled up to stage n and has a different combination index with xn, then its variance estimate is unchanged, i.e., λxi,n=λxi,n-1=Λqxi,n-1.

In summary, the variance estimates evolve iteratively for stage n=1, 2,…,N as following:

λxi,n=λxi,n-1s0+ sxn-1+yi,n-θxi,n2s0+ sxn,                                       if x=xnλxi,n-1+λxni,n-λxni,n-1rqiqxn,      if x≠xn, sxn=0,qiqx=qiqxnλxi,n-1,                                                                         otherwise       (10c)

So, after each stage n, we just need to update for each evaluation index i, the variance estimate of xn, λxni, and update Λqiqxni,n as the a common variance estimate for all the unsampled solutions which have the same combination index with xn. We will see in Section 4.3 that this variance updating approach also help make our sampling algorithm efficient.

4.2.3 The reformulated decision model for NSTPU based on parametric beliefs

Finally, using the proposed parametric belief models, problem (6) can be reformulated as

minx∈X fxD,NbD,N,fxA,NbA,N                                  s.t.  fxE,NbE,N≤ τE-Φ-1(1-ϵ)α1-EλxE,N1/2                      (11)

α1-E=Φ-1(1-E)At the first glance, problem (11) seems nontrivial as there is a solution specific righthand side value in the constraint. However, we note that (10) ensures that the posterior variance estimate λxE,N is common for those x’s that have not been sampled by stage N and have the same combination index for evaluation index i = E. Therefore, we design the following steps in solving (11).

Step-1. We solve a problem by using only information from the posterior mean of the coefficients βji, j=1,…,Mi, for i∈{D, A, E} and applying a common posterior variance estimate ΛwE,N to all solutions that have the same combination index w for evaluation index i = E, as following:

minx∈X j=1MDbjD,NfjDx,j=1MAbjA,NfjAx                                                                 s.t.  j=1MEbjE,NfjEx≤τE-Φ-1(1-ϵ)α1-EΛj,wwE,N12                                                                fjiEx∈fj,lwiE,∀ w∈1,…,j=1MiEmjEi,j∈1,…,MiE,i∈D, A, E,           (12)

where bji,N is the j-th entry of bi,N, i∈{D, A, E}. By our definition of the partition fj,lEi, l=1,…,mjEi for the domain FjEi, j=1,…,MEi, one natural way to solve (12) is by solving a set of j=1MEimjEi subproblems (each for a distinct combination index w=qEx) and comparing their results to derive the Pareto optimal solutions of (12). Each of these subproblems can be solved by standard multi-objective solver such as NSGA-II.

Step-2. For each distinct sampled solution x in {x1,…,xN}, we evaluate its objective values fxD,NbD,N and  fxA,NbA,Nand check its feasibility fxE,NbE,N≤ τE-Φ-1(1-ϵ)α1-EλxE,N1/2. X

Step-3. We compare the objective values of the sampled feasible solutions obtained by Step-2 to those of the Pareto optimal solutions of (12) (with those sampled ones deleted if any) obtained by Step-1 to form the true Pareto optimal solutions for the original problem (11).

4.3 The Knowledge Gradient (KG) sampling policy

In this subsection, we will tailor the GP-based knowledge gradient sampling policy to problem (6) considering the parametric model described in 4.2. The entire solution procedure is summarized in Algorithm 1 in the Appendix, and will be detailed as follows.

At each sampling stage n=1,…,N, we go through the following steps (K2 and K2 are two predetermined numbers):

1) Generate a set of candidate solutions denoted by Sn, which is the union of two separately generated sets, i.e., Sn= S1n∪S2n. S1n is simply a random selection of K1 solutions in x∈X. While S2n contains the solutions that are close to the optimizers (i.e., nondominant solutions) of the deterministic bi-objective problem (11) for stage n-1, i.e., problem (11) with subscript N replaced by n-1. We define Pn-1 as the Pareto optimal solution set of this problem, i.e.,

Pn-1=x∈X:∄ x∈XX s.t. fxE,Nn-1bE,n-1N≤ τE-Φ-11-ϵα1-EλxE,n-1N12,  fxibi,n-1n<fxibi,n-1n ∀i∈D, A,  (13)

if Pn-1 contains more than K2 solutions, we simply choose S2n= Pn-1; otherwise we generate another K2-|Pn-1| new solutions each by firstly choosing an x uniformly at random from Pn-1 and then sampling a solution from Gaussian distribution Nx,Cn and rounding this perturbed solution to the nearest integer solution in X. This approach of sampling set generation is given by Algorithm 2 in the Appendix.

2) Compute the Knowledge Gradient (KG) factor associated with evaluation indices i∈{D, A} for each candidate solution in Sn, the KG factor for evaluation index i of solution x before sampling at stage n, denoted by vKG,i, n(x), is defined as the expected improvement in the best posterior mean of evaluation index i among all the candidate solutions if x∈Sn is sampled given the historical sampling decisions Xn-1 and observations Yn-1 (i∈{D, A}):

vKG,i,nx=minx∈Snθxi,n-1-En-1minx∈Snθxi,n|xn=x

=minx∈Snfxibi,n-1-En-1minx∈Snfxibi,n|xn=x.                   (14)

Algorithm 3 - 4 in the Appendix detail the computation of the KG factor vKG,i, n(x), the key of which is to utilize the conditional predictive distribution pn-1θi,n|xn=x, which has mean and covariance as (these quantities can be derived from (9)):

En-1θi,n|xn=x=θi,n-1=fxi,n-1bi,n-1,                   (15a)

Varn-1θi,n|xn=x=σi,n-1(x)(σi,n-1(x))T,               (15b)

where the column vector σi,n-1(x), as a function of x, can be expressed as

σi,n-1x=Fi,n-1xBi,n-1x(fxi,n-1)T,                     (15c)

where Fi,n-1x=Fi,n-1 and Bi,n-1x=Bi,n-1 if x has been sampled before stage n; otherwise, Fi,n-1x is formed by appending a column to the right of Fi,n-1 with the entry corresponds to solution x being one and others zero, and Bi,n-1 is formed by appending a row and a column to Bi,n-1, with the new diagonal entry being zi,0 and all the other new entries zero, as shown below:

Fi,n-1  Fi,n-1   ex ,   Bi,n-1  Bi,n-1   00        zi,0 ,

where Xex is a unit column vector whose entry corresponding to solution x is one.

3) Pick the sampling decision xn as any solution in Sn with nondominant KG factors for two evaluation indices i=D, A, i.e., xn∈PKG,n, where PKG,n denotes the Pareto optimal solution set (vKG,D,n(x), vKG,A,n(x)) over x∈Snin terms of vKG,D,n(x) and vKG,A,n(x) values:

PKG,n=x∈Sn:∄ x∈Sn s.t. vKG,i,nx>vKG,i,nx ∀i∈{D, A}.        (16)

In practice, we can also use certain rule to select the solution in PKG,n to be sampled. For example, we can pick the one with the largest crowding distance (i.e., the normalized sum of its distances to its adjacent solutions in PKG,n with respect to each objective function).

4) Simulate the signal plan xn and observe the traffic flow output, then compute the evaluation indices yn=(yD,n,yA,n,yE,n). Update posterior beliefs on model parameters βi,n by (9), and update the variance estimates by (10).

5) If n=N, we solve problem (11) and obtain the Pareto optimal solution set PN, from which an optimizer is selected for final implementation. Similar to the case for selecting the sampling decision, certain rule can be applied for selecting the final implementation plan if |PN|>1, e.g., we can select the solution with the largest crowding distance in PN.

4.4 The choice of the basis functions for NSTPU

We adopt a polynomial basis function with at most order of three. Clearly, the higher the order of the polynomial, the more likely the model can better fit the true underlying function, but more terms including the cross-multiplication terms have to be included in the belief model. More specifically, the size of the coefficient vector βi grows exponentially in the dimension n of the solution, a higher order polynomial model can result in significantly more computational cost. As noted by (Osorio and Bierlaire 2013), a quadratic polynomial can provide decent approximation for traffic network problems. (this part can be revised if we have better results using specially designed basis functions…)

qi(x)Mifjixj=1,…,Mi